3GPP TSG RAN WG4 Meeting #106                                                                               R4-2300714

Athens, Greece, 27th Feb – 3rd Mar, 2023

Source: 	Huawei, HiSilicon

Title:	Reply LS on the ue-PowerClassPerBandPerBC-r17 (R4 16-8)

Agenda Item:	11.1.2

Document for:	Approval

# 1	Introduction

Regarding the LS [1] from RAN2 on ue-PowerClassPerBandPerBC-r17, RAN4 had some initial discussions in RAN4#105. It seems that the majority companies tend to agree that this Rel-17 UE capability is only applicable for inter-band UL CA. Additionally, some companies think that it would be useful to clarify the legacy UE behaviour when this UE capability is not indicated. In the following, we share our views on this matter and propose the text for the reply LS as shown in the end.

# 2	Discussion

### 2.1 R17 Power class per band per BC

The UE capability ue-PowerClassPerBandPerBC-r17 was introduced in Rel-17 to resolve the potential power class ambiguity for NR inter-band CA [2], when the power class for a band within a band combination may be different from that reported for the single-band operation on that band. The clause affected is 6.2A.4.1.3 Configured transmitted power for Inter-band CA. In the meantime, this capability is utilised by the higher power limit feature to calculate the sum of the maximum power for inter-band UL CA [3].

Observation 1: The UE capability ue-PowerClassPerBandPerBC-r17 was introduced in Rel-17 to resolve the potential power class ambiguity for NR inter-band CA. And the clauses affected are limited to the transmitter power requirements for inter-band UL CA in TS 38.101-1.

### 2.2 R15&R16 Power class capabilities

As seen from the LS [1], a number of power class related UE capabilities are defined in the RAN2 spec, which are listed in the table below.

| powerClassNRPart-r16 Indicates NR part power class the UE supports when operating according to this band combination. This field only applies for MR-DC BCs containing only single CC or intra-band CA in NR side in this release. | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |
| powerClass, powerClass-v1610 Indicates power class the UE supports when operating according to this band combination. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClass in BandNR), the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. This capability is not applicable to IAB-MT. | BC | No | N/A | FR1 only |
| ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700 For FR1, if the UE supports the different UE power class than the default UE power class as defined in clause 6.2 of TS 38.101-1 [2], the UE shall report the supported UE power class in this field. For FR2, UE shall report the supported UE power class as defined in clause 6 and 7 of TS 38.101-2 [3] in this field. UE indicating support for pc6 supports the enhanced intra-NR RRM and demodulation processing requirements for FR2 to support high speed up to 350 km/h as specified in TS 38.133 [5]. This capability is not applicable to IAB-MT. The power class pc7 is only applicable for RedCap UEs operation in FR2. | Band | Yes | N/A | N/A |
| ue-PowerClassPerBandPerBC-r17 Indicates the UE power class per band per band combination. | FS | No | N/A | FR1 only |

There’re some corner cases of which the applicable power classes may seem ambiguous. In the following, we discuss two of such cases.

#### 2.2.1 Power class per band for two-band UL CA

As per the description for powerClass and its extension, it’s clear that the ue-PowerClass and its extensions apply to the individual bands if the BC power class is higher. However, the opposite cases are not specified, resulting in potential ambiguity [4][5].

For example, it’s not clear which one is applicable if ue-PowerClass for a band is higher than powerClass for the BC. Since the total power of the UL CA shall not exceed the maximum power indicated by powerClass for the BC, the maximum power of the individual bands shall be confined at least by the same limit. From the perspective of the effective maximum output power, it’s reasonable to apply the lower power class (between ue-PowerClass and powerClass) to individual bands.

Table 1: Examples of applicable power classes for two-band UL CA

| Use Case | ue-PowerClass for Band A | ue-PowerClass for Band B | powerClass for CA_A-B | Applicable power class for (Band A, Band B) in UL CA | Remarks |
| --- | --- | --- | --- | --- | --- |
| BC power class is higher than individual band power class | PC3 | PC3 | PC2 | (PC3, PC3) | Apply min{ue-PowerClass, powerClass} as specified in TS 38.306 |
| BC power class is lower than individual band power class | PC1.5 | PC2 | PC3 | (PC3, PC3) | Unspecified, suggest to assume min{ue-PowerClass, powerClass} |
| BC power class is equal to individual band power class | PC2 | PC2 | PC2 | (PC2, PC2) or (PC2, PC3) or (PC3, PC2) or (PC3, PC3) | Unspecified, suggest to indicate ue-PowerClassPerBandPerBC-r17, otherwise assume min{ue-PowerClass, powerClass}. Refer to [4][5] for more details on the use cases. |

Observation 2: For two-band UL CA, the best way to avoid ambiguity is to indicate the UE capability of ue-PowerClassPerBAndPerBC-r17. Otherwise, it’s reasonable to assume min{ue-PowerClass, powerClass} to individual bands.

#### 2.2.2 Power class for single-carrier UL with DL CA

The DL CA with single-carrier UL is a special form of band combinations. In terms of maximum output power, the UE is certainly capable of supporting the power class indicated by ue-PowerClass for the UL band when the band combination is configured. However, the MSD requirement for this band combination may not be completed yet, which is the task of the corresponding HPUE basket work items.

Observation 3: Depending on the progress of the corresponding HPUE basket WI, it’s not straightforward to apply the single-band power class (ue-PowerClass) for the single-carrier UL with DL CA.

Furthermore, in Rel-16 TS 38.101-1 [6], it’s explicitly specified that PC2 is not applicable for band n41, n77, n78 and n79 when DL CA is configured. The corresponding requirements are duplicated below.

#### 6.2A.1.3	UE maximum output power for Inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in clause 6.2 apply.

For inter-band carrier aggregation with two uplink contiguous carrier assigned to one NR band, the transmitter power requirements specified in subclause 6.2A.1.1 apply.

For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.3-1.

For PC3 inter-band carrier aggregation with one uplink component carrier assigned to one NR band in NR band n41, n77, n78, and n79, the requirements for power class 2 are not applicable and the corresponding requirements for a power class 3 UE shall apply.

Observation 4: In Rel-16, power class 2 may be indicated (via ue-PowerClass or its extensions) for band n41, n77, n78 and n79, but it’s not applicable when DL CA is configured.

For the sake of specification compliance as well as avoiding ambiguity, one option is for the UE to indicate the power class for the single-carrier UL using the power class capability for the BC.

Observation 5: The power class for the single-carrier UL with DL CA may be indicated by the BC power class (i.e. powerClass IE and its extension).

# 3	Conclusion

In this paper, we have shared our understanding on the different power class related UE capabilities defined from Rel-15 to Rel-17.

Observation 1: The UE capability ue-PowerClassPerBandPerBC-r17 was introduced in Rel-17 to resolve the potential power class ambiguity for NR inter-band CA. And the clauses affected are limited to the transmitter power requirements for inter-band UL CA in TS 38.101-1.

Observation 2: For two-band UL CA, the best way to avoid ambiguity is to indicate the UE capability of ue-PowerClassPerBAndPerBC-r17. Otherwise, it’s reasonable to apply min{ue-PowerClass, powerClass} to individual bands.

Observation 3: Depending on the progress of the corresponding HPUE basket WI, it’s not straightforward to apply the single-band power class (ue-PowerClass) for the single-carrier UL with DL CA.

Observation 4: In Rel-16, power class 2 may be indicated (via ue-PowerClass or its extensions) for band n41, n77, n78 and n79, but it’s not applicable when DL CA is configured.

Observation 5: The power class for the single-carrier UL with DL CA may be indicated by the BC power class (i.e. powerClass IE and its extension).

Based on the above observations, a draft reply LS to RAN2 is provided at the end of this paper.

Proposal 1: Send the reply LS to RAN2 as shown at the end of the paper.

# References

- R2-2211023 LS the ue-PowerClassPerBandPerBC-r17(R4 16-8), RAN2#119bis-e

- R4-2210764 Draft CR to TS38101-1 Resolving power class ambiguity for NR inter-band CA, Huawei, HiSilicon, RAN4#103-e

- R4-2210767 Increasing the maximum power limit for inter-band UL CA, Qualcomm et al, RAN4#103-e

- R4-2209375 R17 Per band per BC power class capability, OPPO, RAN4#103-e

- R4-2209182 On power class ambiguity for NR inter-band CA, Huawei, HiSilicon, RAN4#103-e

- 3GPP TS 38.101-1 v16.13.0

# Text proposal for the reply LS

Title:	[Draft] Reply LS on the ue-PowerClassPerBandPerBC-r17 (R4 16-8)

Response to:	R2-2211023 LS on the ue-PowerClassPerBandPerBC-r17(R4 16-8)

Release:	Rel-17

Work Item:	NR_RF_FR1_enh

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact person:

Name: Jin Wang

E-mail Address: jinwang@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 would like to thank RAN2 for the LS to inquire the interaction between ue-PowerClassPerBandPerBC-r17 (R4 16-8) and other existing power class capabilities. More explicitly, the following two questions are asked:

- Whether R4 16-8 is applicable to only inter-band CA?

- What is the interaction between R4 16-8 and the existing power class capabilities (i.e. ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-1700, powerClassNRPart-r16 (if R4 16-8 is also applicable to the cases other than inter-band CA) and powerClass/powerClass-v1610)?

RAN4 would like to provide the following responses for RAN2’s questions:

Response to 1): Yes. It is applicable to NR inter-band UL CA only.

Response to 2): ue-PowerClassPerBandPerBC-r17 indicates the power class that a UE supports for each individual band within a given band combination, while powerClass or powerClass-v1610 indicates the power class for this band combination, in other words, the maximum total output power. If indicated, ue-PowerClassPerBandPerBC-r17 shall supersede other power class capabilities such as ue-PowerClass/powerClass and its extensions in determining the power class of the individual bands within a band combination.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to take the above RAN4 responses into consideration.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #106-bis-e	17 - 26 April 2023             E-meeting

TSG-RAN WG4 Meeting #107	22 - 26 May 2023             Incheon, Korea