3GPP TSG-RAN WG4 Meeting # 106	R4-2300821

# Athens, Greece, Feb. 27 – Mar. 3, 2023

Source:	LG Electronics

Title:	Discussion on FR2-1 UL 256QAM

Agenda item:	9.7.2

Document for:	Discussion

Introduction

In the last RAN4#105 meeting, there were agreements for operating SNR, SLS assumptions and MPR simulation assumptions through active discussion for FR2-1 UL 256QAM. In this contribution, we would like to provide our view on SLS simulation results and our view on PTRS for FR2-1 UL 256QAM.

Discussion

- 2.1 SLS

In RAN4#104-e bis and 105 meeting, system level simulation assumption and operating SNR for FR2-1 UL256QAM were discussed, and the related agreements were made as below.

System level simulation for FR2-1 UL 256QAM was conducted based on R4-2219122 and R4-2220810.

[1] R4-2219122 Table 5.2.1.1-1 System level simulation assumptions Parameters; Urban macro; Indoor. Network layout; hexagonal grid, 19 macro sites, 3 sectors per site with wrap around; 50m x 120m, 12BSs. Inter-site distance; 200m (baseline) 300m (optional); 20m. BS antenna height; 25 m; 3 m. UE location; Outdoor/indoor; Outdoor and indoor; Indoor. Indoor UE ratio; 20%. Low/high Penetration loss ratio; 50% low loss, 50% high loss. LOS/NLOS; LOS and NLOS; LOS and NLOS. UE antenna height; Same as 3D-Uma in TR 36.873; 1.5 m. UE distribution (horizontal); Uniform. Minimum BS – UE distance (2D); 35 m; 0 m. Shadowing correlation; Between cells: 1.0 Between sites: 0.5. Pathloss; Uma LOS and NLOS in table 5.2.2.1-1 of 38.803; InH – Office LOS and NLOS in table 5.2.2.1-1 of 38.803. Carrier frequency; 29GHz, 39GHz. BS antenna configuration; (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 8 dBi; (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi. UE antenna configuration; First priority: PC1/PC2/PC5: (Mg, Ng, M, N, P) = (1, 1, 4, 4, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi Second priority: PC3: (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi. System bandwidth; 200MHz. Target SNR at BS side; FFS. UE max output power; PC1: 35 dBm/PC2: 23dBm/PC3: 23 dBm/PC5: 23 dBm. [2] R4-2220810 ##### Operating SNR for 29GHz and 39GHz: - Consider operating SNR as 28 dB with MCS 20~23 for 29GHz averaged based on the submitted results from these two meetings. - Consider operating SNR as 30 dB with limited MCS 20~22 for 39GHz averaged based on the submitted results from two meetings. ##### EVM requirements of UL 256QAM for 48GHz: - UL 256QAM is unfeasible for 48GHz in Rel-18 - The capability for support of UL256QAM is defined as per band per band combinations ##### SLS: - Agree on Option 1 and Option 3. - Option 1: Add Min peak EIRP in the system level simulation assumptions.(ZTE) UE Min peak EIRP; n257 PC1:40.0 dBm/PC2:29 dBm/PC3:22.4 dBm/PC5: 30dBm n260 PC1:38.0 dBm/PC3:20.6 dBm n259 PC2:25dBm, PC5:27.7dBm. - Option 3: Add power control in the system level simulation assumptions.(Nokia) - Power control; Power control parameters set to achieve the target SNR at BS side.. - UL 256 QAM for PC1 UEs of 29 GHz is feasible - FFS for PC2/PC5

-29 GHz UL 256QAM SLS

Fig.1 CDF of SINR for PC2/PC5 in urban macro scenario at 29 GHz

Fig.2 CDF of SINR for PC2/PC5 in indoor scenario at 29 GHz

Fig.1 and Fig.2 show CDF of SINR for PC2/PC5 in urban macro scenario and indoor scenario at 29 GHz respectively.

Based on the SLS results, UL 256QAM at 29 GHz is feasible for both PC2 and PC5.

Observation 1: From the 29 GHz 256QAM SLS results, PC2 and PC5 can achieve SINR of 28dB in both urban macro scenario and indoor scenario.

Proposal 1: UL 256QAM at 29GHz is feasible for both PC2 and PC5 UEs.

-39 GHz UL 256QAM SLS

Fig.3 CDF of SINR for PC1/PC2/PC5 in urban macro scenario at 39 GHz

Fig.4 CDF of SINR for PC1/PC2/PC5 in indoor scenario at 39 GHz

Fig.3 and Fig. 4 show CDF of SINR for PC1/PC2/PC5 in urban macro scenario and indoor scenario at 39 GHz respectively.

Based on the SLS results, UL 256QAM at 39GHz is feasible for PC1, PC2, and PC5 UEs.

Observation 2: From the 39 GHz 256QAM SLS results, PC1, PC2 and PC5 can achieve SINR of 30dB in both urban macro scenario and indoor scenario.

Proposal 2: UL 256QAM at 39GHz is feasible for PC1, PC2, and PC5 UEs.

- 2.2 Consideration for PTRS configuration

Communication performance deterioration due to phase noise is larger in FR2 than in FR1, and especially in high-order modulation (ex.256QAM). In general, phase noise can be divided into CPE (Common Phase Error) and ICI (Inter Carrier Interference). The effect on CPE can be compensated by PTRS (Phase Tracking Reference Signal), and PTRS configuration can be determined by time density (LPT-RS) and frequency density (KPT-RS) of PTRS. The CPE can be better compensated in high PTRS configuration density, but data efficiency can be decreased since resources are allocated to PTRS. Therefore, communication efficiency can be improved by optimizing the PTRS configuration.

In last meeting, there were two options for PTRS configuration and these are showed below.

- Issue 4-1-1: PTRS configuration - Option 1: PTRS configuration shall be aligned with the UE’s recommended PTRS configuration. (IE PTRS-DensityRecommendationSetUL) - Option 2: Using a fixed PTRS configuration for all devices for the EVM test. - It is reasonable to stick with a Rel-15 PTRS configuration of K=2, L=1 only, If only CPE compensation method is used (with no ICI compensation) and having in mind the test implementation.

For option 1 and option 2, each option has their own pros and cons. Option 1 has high communication performance potential but complexity can be increased. Option 2 has low complexity but there is no potential for improving communication performance.

Based on the agreed simulation assumption, we performed link level simulation according to PTRS configuration(K=2 or 4, L=1) with only CPE compensation and compared the results. Fig.5 and Fig.6 show the link level simulation results for 29 GHz and 39 GHz respectively. From the results, the operating SNR improvement according to PTRS configuration seems negligible. This indicates that even if the UE’s recommended PTRS configuration is used, it may not have a large impact on EVM test. Also, the UE’s recommended PTRS configuration is optional feature, so the UE’s recommended PTRS configuration should be precluded for EVM test implementation. In addition, it is also observed that the UL 256QAM for 29 GHz and 39 GHz is feasible with no ICI compensation based on the link level simulation.

Fig.5. Throughput performance between K=2 and K=4 in 29 GHz TDL-A

Fig.6. Throughput performance between K=2 and K=4 in 39 GHzTDL-D

Observation 3 : The operating SNR improvement according to PTRS configuration seems negligible. This indicates that even if the UE’s recommended PTRS configuration is used, it may not have a large impact on EVM test. Also, the UE’s recommended PTRS configuration is optional feature, so the UE’s recommended PTRS configuration should be precluded for EVM test implementation.

Based on the observation3, option 2 on Issue 4-1-1 is supported.

Proposal 3: Use a fixed PTRS configuration without ICI compensation for all devices for the EVM test as in option 2.

Conclusion

In this discussion paper, we provided system level simulation and link level simulation results for FR2-1 UL 256 QAM and shared our observations and proposals based on simulation results.

Observation 1: From the 29 GHz 256QAM SLS results, PC2 and PC5 can achieve SINR of 28dB in both urban macro scenario and indoor scenario.

Observation 2: From the 39 GHz 256QAM SLS results, PC1, PC2 and PC5 can achieve SINR of 30dB in both urban macro scenario and indoor scenario.

Observation 3 : The operating SNR improvement according to PTRS configuration seems negligible. This indicates that even if the UE’s recommended PTRS configuration is used, it may not have a large impact on EVM test. Also, the UE’s recommended PTRS configuration is optional feature, so the UE’s recommended PTRS configuration should be precluded for EVM test implementation.

Proposal 1: UL 256QAM at 29GHz is feasible for both PC2 and PC5 UEs.

Proposal 2: UL 256QAM at 39GHz is feasible for PC1, PC2, and PC5 UEs.

Proposal 3: Use a fixed PTRS configuration without ICI compensation for all devices for the EVM test as in option 2.

- References

# [1] R4-2219122, “TP for TR 38.891 on link level simulation results and system level simulation assumption for FR2 UL 256QAM”

# [2] R4-2217729, “WF on UL 256QAM”, Xiaomi