3GPP TSG-RAN WG4 Meeting # 106	R4-2300919

Athens, Greece, Feb. 27 – Mar. 3, 2023

Agenda Item: 5.2.8.3

Source: MediaTek Inc.

Title: Discussion on R17 MR-DC enhancement

Document for: Discussion

- Introduction

In RAN4 #104-e meeting, a WF [1] was agreed on further discussions of efficient SCell activation and activation/de-activation mechanism for SCG. There are still some remaining issues. In this paper, we provide some analysis on the remaining issues.

- Discussion

2.1 Measurement requirements with deactivated SCG

In RAN4#104-e meeting, RAN4 has discussed whether to relax the measurement on inter-frequency configured by SCG when SCG is deactivated but without consensus. Although chair guides that “If there is no consensus by next meeting, suggest to discuss it in Rel-18”, we bring this issue again as R17 MR-DC Enh was not in the agenda of RAN4#104bis-e meeting and not discussed online in RAN4#105.

< Way forward >: Issue 2-6: Relax measurements on inter-frequency configured by SCG when SCG is deactivated - Proposals - Option 1 (QC, Apple, MTK, vivo, OPPO): Use the parameter measCyclePSCell to relax measurements on inter-frequency configured only by SCG when SCG is deactivated - Option 2 (Nokia, Ericsson, Huawei): No, RAN4 shall keep the new introduced parameter measCyclePSCell within agreed scope This issue was handled on GTW (2022-08-16). Chair’s note is duplicated: Chair=> Encourage experts to address the issues as soon as possible. If there is no consensus by next meeting, suggest to discuss it in Rel-18.

RAN4 has introduced a parameter measCyclePSCell for relaxation of RRM measurement on deactivated PSCell for power saving and specified relaxed measurement requirements for deactivated PSCell. In our understanding, SCG configures inter-frequency MOs for the purpose of PSCell change or SCG SCell addition. When SCG is in deactivated state, it is not urgent to change PSCell or add a new SCG SCell. Therefore, we propose to relax measurements on inter-frequency configured only by SCG when SCG is deactivated for power saving. The impact on spec is captured in the CR [2] submitted.

We admit relaxation on inter-frequency measurement may have little impact on performance. But the whole motivation of the WI is for UE power saving with a reasonable latency. It is strange that inter-frequency is measured even faster than intra-frequency. In addition, even the measurement requirements are relaxed, UE can still measure more frequently if needed (e.g., when moving fast) to balance power saving and performance.

Proposal 1: Use the parameter measCyclePSCell to relax measurements on inter-frequency configured only by SCG when SCG is deactivated. Take measurement period in FR1 as an example, the requirements for inter-frequency measurements when SCG is deactivated with gaps is as follows:

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, Ceil(8 * Kgap) Max(MGRP, measCyclePSCell)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5 * Kgap) Max(MGRP, measCyclePSCell, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(8 * Kgap) Max(measCyclePSCell, DRX cycle) CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent gaps, the MRGP above is the MRGP of the measurement gap associated with the target frequency layer to be measured if concurrent measurement gaps are configured. NOTE 4: The requirements in this table are only applicable to the inter-frequency layers configured by SCG only. |  |

2.2 SCG activation delay

In this WI, both RACH based and RACH-less based SCG activation are specified. For RACH-based SCG activation, UE would perform random access towards PSCell. For RACH-less based SCG activation, UE would transmit SR on PUCCH or PUSCH on PSCell after PSCell activation. To determine the transmission power, UE needs to maintain PL-RS to estimate downlink pathloss. For PRACH transmission on PSCell during SCG activation, the SS/PBCH block associated is used to estimate pathloss. For PUCCH or PUSCH, SSB or CSI-RS is used to estimate pathloss according to network configuration.

RAN4 had reached agreement that SCG activation delay is “Tactivation_time = TRRC_delay + Tprocessing + Tsearch + T∆ + TIU + 2 ms”. But the impact of PL-RS on SCG activation delay has not considered.

UE keeps L3 measurement on deactivated PSCell and determines the preamble and RACH occasion based on measurement results. Therefore, for PRACH transmission on PSCell during SCG activation, UE doesn’t need extra time to measure PL-RS since associated SSB is used to estimate pathloss. While for PUCCH or PUSCH transmission in RACH-less SCG activation, if the corresponding PL-RS is not under monitoring, UE needs extra time to measure PL-RS.

Since the core part requirements have been finalized for a very long time, it is not desirable to reopen the whole discussion and start to add new requirements for different possible configurations. Instead, we think it is more reasonable just add additional limitation to the configuration that the PL-RS to use is identical to SSB or source CSI-RS of the TCI state of PSCell. In this way, we can help the current requirement to address PL-RS, while avoiding adding new UE behaviors. Therefore, we propose to add a condition on the current SCG activation delay requirements: The requirements are applicable under the condition that PL-RS of PSCell to use is identical to the associated SSB or CSI-RS of the TCI state for PDCCH/PDSCH reception in PSCell. The impact on spec is captured in the CR [2] submitted.

Proposal 2: The RACH-less SCG activation delay requirements are applicable under the condition that PL-RS to use for the first transmission on PSCell is SSB or the same CSI-RS as RLM-RS/BFD-RS configured for deactivated PSCell.

- Summary

In this paper, the discussion on the remaining issues of efficient activation/de-activation mechanism for SCG is provided. We have the following proposals:

Proposal 1: Use the parameter measCyclePSCell to relax measurements on inter-frequency configured only by SCG when SCG is deactivated. Take measurement period in FR1 as an example. Define the requirements for inter-frequency measurements when SCG is deactivated with gaps as follows:

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, Ceil(8 * Kgap) Max(MGRP, measCyclePSCell)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5 * Kgap) Max(MGRP, measCyclePSCell, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(8 * Kgap) Max(measCyclePSCell, DRX cycle) CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent gaps, the MRGP above is the MRGP of the measurement gap associated with the target frequency layer to be measured if concurrent measurement gaps are configured. NOTE 4: The requirements in this table are only applicable to the inter-frequency layers configured by SCG only. |  |

Proposal 2: The RACH-less SCG activation delay requirements are applicable under the condition that PL-RS to use for the first transmission on PSCell is SSB or the same CSI-RS as RLM-RS/BFD-RS configured for deactivated PSCell.

4 Reference

R4-2214494, WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei, HiSilicon.

R4-23xxxx, CR for R17 MR-DC Enh on 38.133, Mediatek.