3GPP TSG-RAN WG4 Meeting # 106	R4-2300948

# Athens, Greece, February 27 – March 3, 2023

Source:	LG Electronics

Title:	Discussion on general RRM on Rel-18 NR Sidelink Evolution

Agenda item:	9.29.3

Document for:	Discussion

Introduction

Based on WID [1], RAN4 works on NR Sidelink Evolution start at RAN4#106. The objectives of this WI are

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#99) - Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, handling the limited capability, power control for simultaneous sidelink TX, packet duplication) - The work is limited to FR1 licensed spectrum and ITS band in FR1. - No specific enhancements of Rel-17 sidelink features with sidelink CA support. - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI) - Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel. - Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only. - Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation [RAN4] - The exact frequency bands for both licensed and ITS-dedicated spectrum in FR1 and FR2 are to be determined based on company input during the WI. - The exact frequency band combinations for both FR1 licensed and ITS-dedicated spectrum are to be determined based on company input during the WI. - Frequency bands for the unlicensed spectrum in FR1 are [n46 and n96/n102] (i.e., 5GHz and 6GHz) in accordance with corresponding national regulatory requirements. - Support of new sidelink frequency bands and band combinations should ensure coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum. - UE RRM core requirement for the new features introduced in this WI [RAN4]

In this contribution, we provide our views on general RRM issues on sidelink evolution based on the yellow highlighted objectives.

Specification structure

In Rel-18 sidelink evolution, there are several objectives, but two main objectives would be focused in RRM part; sidelink unlicensed spectrum and co-channel coexistence for LTE sidelink and NR sidelink. For co-channel coexistence, it seems to be less impact on RRM part. However, sidelink unlicensed spectrum would be impact on all RRM part, so RAN4 can define the requirements for sidelink unlicensed spectrum with suffix in clause 12 in TS38.133 like NR-U.

- Proposal 1: Define RRM requirements for sidelink unlicensed spectrum in clause 12 with suffix in TS38.133.

  - E.g., 12.3.A Initiation/Cease of SLSS Transmission

12.4.A Selection/Reselection of V2X Synchronization Reference Source

RRM impact due to new features

Based on WID [1], there are two main issues for RRM part; sidelink on unlicensed spectrum (SL-U) and co-channel coexistence between LTE sidelink and NR sidelink. Table 1 shows our views on the impact on the existing NR V2X RRM requirements.

Table 1 List of RRM impact due to new features

| NR V2X Requirements | Impact / comment |  |
| --- | --- | --- |
|  | Sidelink on unlicensed spectrum (SL-U) | Co-channel coexistence between LTE sidelink and NR sidelink |
| UE transmit timing | No | No |
| Initiation/Cease of SLSS transmission | Yes | No |
|  | Tevaluate,SLSS due to LBT failure and other impact depending on RAN1 progress |  |
| Selection/Reselection of V2X synchronization reference source | Yes | No |
|  | Tdetect,SyncRefUE_V2X and dropping rate due to LBT failure and other impact depending on RAN1 progress |  |
| L1 SL-RSRP measurement | FFS | No |
|  | depending on RAN1 progress (e.g., multi-consecutive slots transmission, multi-starting symbols for PSCCH/PSSCH, etc.) |  |
| Congestion control measurements | FFS | No |
|  | depending on RAN1 progress (e.g., multi-consecutive slots transmission, multi-starting symbols for PSCCH/PSSCH, etc.) |  |
| Interruption | Yes | FFS |
|  | additional interruption for LBT operation | depending on RAN1 progress |
| Reliability of GNSS signal | No | No |
| Scheduling availability | No | FFS |
|  |  | depending on RAN1 progress |

For SL-U, LBT operation in SL-U should be performed like NR-U operation. Therefore, the requirements for evaluation and detection time for initiation/cease of SLSS transmission and selection/reselection of sync reference source should be defined due to LBT failure, and the dropping rate associated with selection/reselection of sync reference source should also be discussed. Since RAN1 is discussing S-SSB transmission design, multi-consecutive slots transmission, multi-starting symbols in a slot, and so on, RAN4 needs to further discuss how the RAN1 outputs affect RAN4 RRM core requirements.

For co-channel coexistence between LTE sidelink and NR sidelink, RAN1 is still under discussion how to avoid impact between LTE sidelink and NR sidelink, and the in-device coexistence framework defined in Rel-16 as much as possible would be reused. Therefore, it is expected that there will be no RRM issue. However, RAN4 needs to further check whether RRM requirements should be updated depending on RAN1 discussion and conclusion.

- Observation 1: For SL-U, the general RRM requirements for sidelink due to LBT operation are impacted.

- Observation 2: For co-channel coexistence between LTE sidelink and NR sidelink, it is expected that there is no RRM issue, but further final check whether there is no impact on RRM requirements depending on RAN1 conclusion.

- Proposal 2: Consider Table 1 as starting point of Rel-18 RRM requirements.

Conclusion

In this contribution, we provide the views on general RRM issues on sidelink evolution. For specification structure, we propose

- Proposal 1: Define RRM requirements for sidelink unlicensed spectrum in clause 12 with suffix in TS38.133.

  - E.g., 12.3.A Initiation/Cease of SLSS Transmission

12.4.A Selection/Reselection of V2X Synchronization Reference Source

For RRM impact due to new features, we observe and propose

- Observation 1: For SL-U, the general RRM requirements for sidelink due to LBT operation are impacted.

- Observation 2: For co-channel coexistence between LTE sidelink and NR sidelink, it is expected that there is no RRM issue, but further final check whether there is no impact on RRM requirements depending on RAN1 conclusion.

- Proposal 2: Consider Table 1 as starting point of Rel-18 RRM requirements.

Reference

[1] RP-222806, “WID revision: NR sidelink evolution,” OPPO.