3GPP TSG-RAN WG4 Meeting #106	R4-2301001

Athens, Greece, February 27 – March 03, 2022

Agenda item:	9.9.2.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on L1 part enhancement for FR2 SCell activation

Document for:	Discussion

- Introduction

At the last RAN4 #105 meeting, RRM requirements for L1 part enhancement for FR2 SCell activation was discussion and WF was agreed [1]. There are four remaining issues as following.

- Enhancement for L1-RSRP

- TCI related enhancement for L1 part

- Aperiodic RS related enhancement for L1 part

- Other enhancement for L1 part

In this contribution, we will pick up some and propose our view on each remaining issue.

- Discussion

  - 1.	Enhancement for L1-RSRP

During last meeting, the discussion that impact on enhancement for L1-RSRP. The part of beam sweeping factor enhancement of FR2 unknown SCell activation is duplicated as follows [1]

Issue 2-1-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception) - FFS - Option 1 (Apple, Xiaomi, LGE, OPPO, Ericsson): - For unknown FR2 SCell activation enhancement, introduce the UE capability to support Rx beam sweeping factor can be less than 8 for SSB based L1-RSRP measurement. - Option 2 (Intel): - If UE has some prior information to skip L3 part and can perform L1-RSRP directly, suggest not to reduce RX beam sweeping factor in order to select more accurate beam. - Option 3 (CMCC): - for RX beam sweeping factor reduction, the agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or Rel-17 positioning enhancement can be considered as baseline - Option 4 (CTC): - If L1- RSRP measurement is not skipped, Rx beam sweeping factor reduction in L1-RSRP measurement can be considered provided that the performance loss is acceptable. Meanwhile, the usage conditions/scenarios for Rx beam sweeping factor reduction in L1-RSRP measurement should be discussed. - Option 5 (Nokia): - For FR2 unknown SCell activation, a smaller Rx beam sweeping factor is assumed for L1-RSRP measurement based on the measurement during cell detection. - Option 6 (Huawei): - Compared with completely skipping L1-RSRP, it is suggested to discuss beam sweeping factor reduction for L1-RSRP after L3 part when UE can acquire some information via L3 measurement (AGC+Cell search).

As CMCC proposed at the last meeting [2], we support the discussion based on Rel-17 FR2 HST or Rel-17 positioning enhancement. The Rx beam sweeping factor can be 2 or 6 for the FR2 HST. There are also beam sweeping factors of 1,2,4,6 as agreed in the Rel-17 positioning enhancement. We think that FR2 SCell activation delay can be shortened by allowing performance degradation.

Proposal 1: RAN4 can consider reusing the reduced number of RX beam sweep factors based on Rel-17 FR2 HST or Rel-17 positioning enhancements.

The part of how to skip L1-RSRP measurement of FR2 unknown SCell activation is duplicated as follows [1]

Issue 2-1-2: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation? - FFS - Option 1 (Apple, CMCC, Nokia (using same RS only), OPPO): - for unknown R18 FR2 SCell activation enhancement, following L1 part enhancement can be considered: - If L3 measurement is performed and if L3 and L1 measurement are using same RS or QCLed type D RSs, skip L1-RSRP measurement and use measurement result from L3 stage for L1-RSRP reporting. - Option 2 (Intel): - If UE has no prior information, L3 measurement with SSB index can be performed in cell search, AGC steps. L1-RSRP measurement can be skipped. There is no need to add other condition, i.e., L3 measurement and L1 measurement are using same RS or QCL-type D RSs. - If UE has some prior information, further discuss whether L1-RSRP can be skipped or not. - Option 3 (Qualcomm): - L1 measurement and report can be skipped when UE report L3 measurements where the RSRP is acceptable range. Acceptable range could be FFS. - Option 4 (CTC): - If UE performs the whole process of the previous L3 part without any reduction, skipping L1-RSRP measurement of FR2 unknown SCell activation can be considered. Otherwise, L1- RSRP measurement cannot be skipped. - Option 5 (Xiaomi): - L1-RSRP measurement and reporting procedures can be skipped provided that: - UE is required to report the RSRP measurement results with SSB indexes, and - the reported RSRP result is good enough, e.g. higher than a threshold - Option 6 (LGE): - Consider L1-RSRP measurement skipping based on L3 measurement results without extending the existing L3 part delay. - Option 7 (vivo): - For UE capable of basic feature of reduced FR2 SCell activation delay, the UE should be able to use the intermediate result in L3 measurement for SSB-based L1-RSRP reporting, and the time for L1-RSRP measurement TL1-RSRP_measure can be removed. - At least for the case when SCell to be activated meets Es/Iot > -2dB and L3 measurement result obtained by UE within a pre-defined period is available, for UE supports reporting MR during the unknown SCell activation procedure, the enhanced SCell activation delay requirement is specified by assuming the gNB configures TCI of SCell based the SSB index reported in the MR, and the time for L1-RSRP measurement and reporting in the SCell activation delay can be removed. - For UE supports cross-carrier QCL-C/D indication for an inter-band SCells, if the SCell to be activated meets Es/Iot > -2dB, and network configures inter-band cross-carrier QCL-C/D in the TCI of the SCell, the time for L1-RSRP measurement and reporting in the SCell activation delay can be removed. - Option 8 (ZTE): - When the following conditions are satisfied, L1-RSRP measurement can be ignored: - UE can identify Tx beam during L3 part -- L3 part includes all same RS or QCLed Type D RS used by L1 part. - UE can identify Rx beam during L3 part --UE has swept all Rx beam needed by L1 part during L3 part - Option 9 (MediaTek): - Two potential solutions to skip L1-RSRP measurements when activating unknown SCell in FR2: - Contention based random access (CBRA) approach: CBRA can be used to activate first unknown SCell in one band, which can help skipping L1 part and TCI state indication, and as a result, enhance the overall activation delay for unknown SCell. - RSRP report before L1 part: the NW can trigger the UE to report the latest RSRP measurements before L1 part (L1-RSRP measurement and report), which can help skipping L1 part for unknown SCell activation in FR2. Note: both approaches will have an impact on RAN1/RAN2 work. - Option 10 (Ericsson): - Skipping L1-RSRP report to be within NW control and NW should be able to indicate whether a particular SCell activation is with L1-RSRP or without L1-RSRP report. - Option 11 (Huawei): - When UE supports inter-band QCL-ed type D beam indication, L1-RSRP measurement and L1-RSRP report can be skipped.

During last meeting, there were so many suggestions. We think RAN4 could support option 1. L3 measurement is reported for each certain event trigger and reports long-term channel quality. If the UE has the same RS, or in the case of QCL type D, it can be considered that it has the same channel. Therefore, the UE considers that L1 measurement can be skipped or relaxed based on the measurement result of L3 measurement.

Proposal 2: If the UE uses the same RS or QCL Type D RS, the L1 measurements can be shortened or relaxed by using the L3 measurements as a basis.

- Conclusion

In this contribution, we proposed our views on L3 part enhancement for FR2 SCell activation.

Proposal 1: RAN4 can consider reusing the reduced number of RX beam sweep factors based on Rel-17 FR2 HST or Rel-17 positioning enhancements.

Proposal 2: If the UE uses the same RS or QCL Type D RS, the L1 measurements can be shortened or relaxed by using the L3 measurements as a basis.

References

[1] 3GPP, R4-2220440, Apple, “WF on Even Further RRM enhancement for NR and MR-DC – Part 1”

[2] 3GPP, R4-2218395, CMCC, “Discussion on L3 part enhancement for FR2 SCell activation”