# 3GPP TSG-RAN WG4 Meeting # 106		  R4-2301229

# Athens, Greece, Feb. 27-Mar. 03, 2023

# Source:	ZTE Corporation

# Title:	Discussion on band definition for 900 MHz NR Band

# Agenda Item:	8.32.2

# Document for:	Approval

- Introduction

During RAN#98-e meeting, a new Work Item [1] has been approved on introduction of 900 MHz NR Band in the US. In this contribution, we want to share some initial views on band definition.

- Specify a new 5G NR FDD operating band operating with

  - UL: 896 – 901 MHz and DL 935 – 940 MHz

  - To support 5 MHz, 3 MHz channel bandwidths with 15 kHz SCS.

    - NOTE: 3MHz CBW will be added once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW.

- Study and specify the co-existence requirements

  - Support FCC OOB emission mask in FCC-20-67A1 for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

  - Study and if needed, specify the protection for B5 and B26 including MPR.

  - Assume reusing band B8 duplexer

- Discussion

  - Operating band and band numbering

As indicated in the objective, a new 5G NR operating band should be defined as following.

Table 2.1-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| [n106] | 896 MHz – 901 MHz | 935 MHz – 940 MHz | FDD |

Proposal 1: To define the new NR FDD band as in Table 2.1-1.

  - Maximum transmission bandwidth configuration

For the new NR FDD operating band, as stated in the Work Item, it supports 5 MHz and 3 MHz channel bandwidths with 15 kHz SCS. The transmission bandwidth configuration NRB for 3 MHz channel bandwidth should follow TS 36.101 once 3 MHz CBW is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. For 5 MHz CBW, it should follow TS 38.101-1.

Table 2.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3MHz | 5MHz |
| --- | --- | --- |
|  | NRB | NRB |
| 15 | [15] | 25 |

Proposal 2: For 900 MHz NR band, it supports 3MHz and 5MHz channel bandwidth with 15 kHz SCS.

Proposal 3: For 900 MHz NR band, to define the transmission bandwidth configuration NRB = 25 for 5 MHz CBW, and whether to define NRB = 15 for 3 MHz CBW depends on Rel-18 NR_FR1_lessthan_5MHz_BW.

  - Minimum guardband

For the minimum guardband, it should be calculated in the following way. Thus the minimum guardband for 3 MHz channel bandwidth is 142.5 kHz for SCS = 15 kHz. The minimum guardband for the new NR FDD operating band should be defined as Table 2.3-1.

minimum guardband = (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2

Table 2.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 3MHz | 5MHz |
| --- | --- | --- |
| 15 | [142.5] | 242.5 |

Proposal 4: For 900 MHz NR band, whether to define the minimum guardband as 142.5 kHz for 3 MHz CBW should depend on Rel-18 NR_FR1_lessthan_5MHz_BW. The minimum guardband for 5 MHz CBW should follow TS 38.101-1.

  - Channel raster and sync raster

For NR-ARFCN, we propose to use the similar descending manner to define as following:

Table 2.4-1: NR-ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n106] | 100 | 179200 – <20> – 180200 | 187000 – <20> – 188000 |

Proposal 5: For NR-ARFCN, it is proposed to define as Table 2.4-1.

Table 2.4-2: Applicable SS raster entries

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n106] | 15 kHz | Case A | 2342 – <1> – 2344 |

Proposal 6: The sync raster for 3 MHz channel bandwidth should depend on the consequence of Rel-18 NR_FR1_lessthan_5MHz_BW.

Proposal 7: It is proposed that the sync raster for 5 MHz CBW should follow the current design of sync raster in TS 38.101-1 and defined as Table 2.4-2.

  - TX–RX frequency separation

For TX–RX frequency separation, we propose to define as following:

Table 2.5-1: UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [n106] | 39 MHz |

Proposal 8: For TX–RX frequency separation, it is proposed to define as Table 2.5-1.

- Conclusion

In this contribution, we give some initial discussion on band definition for 900 MHz NR band and the proposals are made as following:

Proposal 1: To define the new NR FDD band as in Table 2.1-1.

Proposal 2: For 900 MHz NR band, it supports 3MHz and 5MHz channel bandwidth with 15 kHz SCS.

Proposal 3: For 900 MHz NR band, to define the transmission bandwidth configuration NRB = 25 for 5 MHz CBW, and whether to define NRB = 15 for 3 MHz CBW depends on Rel-18 NR_FR1_lessthan_5MHz_BW.

Proposal 4: For 900 MHz NR band, whether to define the minimum guardband as 142.5 kHz for 3 MHz CBW should depend on Rel-18 NR_FR1_lessthan_5MHz_BW. The minimum guardband for 5 MHz CBW should follow TS 38.101-1.

Proposal 5: For NR-ARFCN, it is proposed to define as Table 2.4-1.

Proposal 6: The sync raster for 3 MHz channel bandwidth should depend on the consequence of Rel-18 NR_FR1_lessthan_5MHz_BW.

Proposal 7: It is proposed that the sync raster for 5 MHz CBW should follow the current design of sync raster in TS 38.101-1 and defined as Table 2.4-2.

Proposal 8: For TX–RX frequency separation, it is proposed to define as Table 2.5-1.

- Reference

RP-223529, New WID on Introduction of 900 MHz NR Band in the US, Anterix