GPP TSG-RAN WG4 Meeting # 106	R4-2301806

# Athens, Greece, February 27 – March 3, 2023

Agenda item:	10.5.6

Source: 	Huawei, HiSilicon

Title: 	Discussion RRM performance requirements for IoT NTN

Document for:	Discussion

# Introduction

The Rel-18 RAN4 WID was approved in [1] to define requirements for Rel-17 IoT NTN [2]. In Rel-17 IoT NTN, RAN1/2/3 specifications were defined to enable NB-IoT and eMTC operating over NTN. In last RAN4 meeting, most of open issues were finalized with core part completed [3]. In this paper, we provide our views on performance requirements for IoT NTN.

# Discussion

There was initial discussion on performance requirements in last meeting, some high-level agreements summarized in [3]. We will provide our views on the remaining issues in the following part of this paper.

## 2.1 PHR reporting

Regarding the PHR reporting, the status are open issues are summarized as follows:

#### Issue 4: PHR reporting - For NB-IoT operation with GSO, the legacy NB-IoT PHR reporting tables are used as baseline. - For NB-IoT operation with NGSO, FFS need to adjust the reporting values compared to the legacy reporting values. - For UE category M1 operation with GSO and NGSO, the legacy UE category M1 PHR reporting tables are used as baseline.

As analyzed in [4], since only 2 bits PHR reporting is supported for NB-IoT which means only 4 different values can be reported. Thus, it was proposed to analysis whether to adjust the value for different LEO satellites. For legacy PHR reporting tables, they are already differentiated for different coverage level which is shown below. It basically serves the same purpose as pointed out in [4] that different type of satellite may have different PL level. Thus, the appropriate table can be selected based on selected coverage level directly, and there is no need to further differentiated the case by type of satellite. Besides, for UE supporting enhanced PHR reporting, 16 values PHR reporting is supported which can report the PHR in finer granularity.

Table 9.1.23.3-1: Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR [31] when the enhanced coverage level 0 is selected during random access procedure [17] Reported value; Measured quantity value (dB). POWER_HEADROOM_0; -54 PH 5. POWER_HEADROOM_1; 5 PH 8. POWER_HEADROOM_2; 8 PH 11. POWER_HEADROOM_3; PH ≥ 11. Table 9.1.23.3-2: Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR [31] when the enhanced coverage level other than 0 is selected during random access procedure [17] Reported value; Measured quantity value (dB). POWER_HEADROOM_0; -54 PH -10. POWER_HEADROOM_1; -10 PH -2. POWER_HEADROOM_2; -2 PH 6. POWER_HEADROOM_3; PH ≥ 6.

Observation 1: Different PHR tables are used based on different coverage level depending on the estimated pathloss.

Observation 2: UE can optionally support enhanced PHR with 16 values for finer granularity.

In addition, based on the framework of PHR framework defined in TS 36.213, the PHR mainly depends on three factors: maximum transmit power, targeting receiving power and pathloss. Based on RAN1/RF agreements for Rel-17 IoT NTN, there is no enhancement for power control per RAN1/RF agreements. Regarding the pathloss, for NB-IoT, it is designed to support up to 164 dB MCL for enhanced coverage. At same time, there was no enhancement to support severer case with higher pathloss. Based on the analysis above, we didn’t see strong motivation to update/modify legacy PHR reporting table.

Proposal 1: For NB-IoT and eMTC, legacy PHR table can be reused for IoT NTN.

## 2.2 List of tests

For test case list, follow general aspects are discussed:

#### Issue 8-1: List of Tests – General - For NB-IoT and eMTC NTN, RAN4 to introduce same set of test cases (except TDD related) as defined for legacy NB-IoT and eMTC in LTE. - FFS whether to define test case involving cell detection when the target cell is in enhanced coverage for NGSO. - FFS whether to define test case for UL segmented transmission gap for UE dropping of UL resources. The new test is applicable for UEs that indicate this capability. - The transmission gap for UE dropping of UL resources does not apply when the timing advance is decreasing from one segment to the next segment. - RAN4 to reply the LS from RAN1 indicating the segmented transmission will be taken into account when developing test cases

One general issue is whether to define test cases cell detection when the target cell is in enhanced coverage for NGSO. As explained in our previous paper, for instance, the cell identification time in idle mode could be up to 500 seconds in IDLE state cell reselection. Similarly, the cell detection time for MTC in CONNECTED state could be up to 300 seconds. Apparently, the long delay is cannot suit the requirement in real IoT NTN deployment especially for NGSO. There is no need to define test cases for such case. However, when the target cell is serving cell in enhanced coverage, though the delay is much longer than that in normal coverage, it still can work (e.g. the indication time for cell in enhanced coverage in RRC Re-establishment is 14800 ms vs 1400 ms in normal coverage). Thus, it is proposed not to define test cases involving neighbor cell detection when the target cell is in enhanced coverage for NGSO.

Proposal 2: Not to define test cases involving neighbor cell detection when the target cell is in enhanced coverage for NGSO.

Another issue is whether to define test cases to verify UL segmented transmission for UE dropping UL resource. It was proposed to define the UE behavior that the transmission gap for UE dropping of UL resources does not apply when the timing advance is decreasing from one segment to the next segment. Apparently, this is within RAN1 scope which can not be decided in RAN4. After checking the currently status RAN1, in case the gap is configured, UE shall drop the transmission regardless whether the segment is overlapped or not.

Observation 3: Based on current RAN1 status, UE will drop the transmission in case the gap is configured regardless of whether the segments are overlapped or not.

Thus, there is no need to define additional gap applicability rules in RAN4.

Proposal 3: There is no need to define additional gap applicability rules in RAN4 which is in RAN1 scope.

Based on the analysis above, the dropping of UL transmission is purely gap-based, and RAN4 don’t not define test cases for gap as it is configured by NW.

Proposal 4: Not to define test case for UL segmented transmission gap for UE dropping of UL resources

One issue which may have significant impact on RAN4’s requirements is the availability of neighbor cell ephemeris based on RAN2’s latest LS reply [4]. The LS reply is as follows:

RAN2 thanks RAN4 for the LS on information for neighbor/target cell in IoT NTN. In case of handover to an NTN cell, the SIB31 of target NTN cell (which contains its satellite assistance information) is provided in RRCConnectionReconfiguration. For neighbor cell measurements, RAN2 has agreed not to introduce the satellite assistance information of neighbour cells in system information in Rel-17 IoT NTN, but this will be supported in Rel-18. 2. Actions: To RAN4: RAN2 respectfully asks RAN4 to take the above information into account in their work.

It could be observed that RAN2 will not introduce the satellite assistance information of neighbor cell in Rel-18 IoT NTN. For this RAN4 WI, which is to define RRM requirements based on Rel-17 framework, it means that the requirements shall be defined based on the assumption that no neighbor cell ephemeris is available to UE.

As discussed in our companied paper for core requirements maintenance, it is proposed to remove requirements related to neighbor cell measurement. Thus, apparently, there is no need to define test cases involving neighbor cell measurement.

Proposal 5: There is no need to define test cases involving neighbor cell measurement.

The test cases lists are updated as follows:

Table I. Test case for IoT NTN for NB-IoT

|  |  |  | # |
| --- | --- | --- | --- |
| RRC_IDLE state | Cell Re-Selection | HD – FDD Intra frequency case for UE Category NB1 in normal coverage | NB 1-1 |
|  |  | HD – FDD Inter frequency case for UE Category NB1 in normal coverage | NB 1-2 |
|  |  | HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation | NB 1-3 |
|  |  | HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX | NB 1-4 |
| RRC CONNECTED Mode Mobility | NA | NA | n.a. |
| RRC Connection Control | RRC Re-establishment | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage | NB 2-1 |
|  |  | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage | NB 2-2 |
|  | Random Access | Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage | NB 2-3 |
|  |  | Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage | NB 2-4 |
|  |  | Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage | NB 2-5 |
| Timing | UE Transmit Timing | HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage | NB 3-1 |
|  |  | HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage | NB 3-2 |
|  | UE Timing Advance | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage | NB 3-3 |
|  |  | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage | NB 3-4 |
| Signaling characteristic | RLM | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | NB 4-1 |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage | NB 4-2 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage | NB 4-3 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage | NB 4-4 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage | NB 4-5 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage | NB 4-6 |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | NB 4-7 |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | NB 4-8 |
| Measurement Performance Requirements | channel quality reporting accuracy | HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage | NB 5-1 |
|  |  | HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage | NB 5-2 |
|  |  | HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage | NB 5-3 |
|  |  | HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage | NB 5-4 |
|  |  | HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage | NB 5-5 |
|  |  | HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage | NB 5-6 |

Table II. Test case for IoT NTN for Cat-M

|  |  |  | # |
| --- | --- | --- | --- |
| RRC_IDLE state | Cell Re-Selection | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-1 |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-2 |
|  |  | E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M1 1-3 |
|  |  | E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M1 1-4 |
|  |  | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-5 |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-6 |
| RRC CONNECTED Mode Mobility | Handover and Conditional Handover | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition | M1 2-1 |
|  |  | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition | M1 2-2 |
|  |  | E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA | M1 2-3 |
|  |  | E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA | M1 2-4 |
|  |  | E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-5 |
|  |  | E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-6 |
|  |  | E-UTRAN FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-7 |
|  |  | E-UTRAN HD-FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-8 |
| RRC Connection Control | RRC Re-establishment | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-1 |
|  |  | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-2 |
|  |  | E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-3 |
|  |  | E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-4 |
|  | Random Access | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | M1 3-5 |
|  |  | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | M1 3-6 |
|  |  | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | M1 3-7 |
|  |  | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | M1 3-8 |
| Timing | UE Transmit Timing | E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | M1 4-1 |
|  |  | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | M1 4-2 |
|  |  | E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | M1 4-3 |
|  |  | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | M1 4-4 |
|  | UE Timing Advance | E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | M1 4-5 |
|  |  | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | M1 4-6 |
|  |  | E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | M1 4-7 |
|  |  | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | M1 4-8 |
| Signaling characteristic | RLM | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | M1 5-1 |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | M1 5-2 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | M1 5-3 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | M1 5-4 |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for Cat-M1 UE in CEMode A | M1 5-5 |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync in DRX for Cat-M1 UE in CEMode A | M1 5-6 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for Cat-M1 UE in CEMode A | M1 5-7 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync in DRX for Cat-M1 UE in CEMode A | M1 5-8 |
| Measurement Procedure | Event triggered reporting | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-1 |
|  |  | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-2 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-3 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-4 |
|  |  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M1 6-5 |
|  |  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M1 6-6 |
|  |  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M1 6-7 |
|  |  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M1 6-8 |
| Measurement Performance Requirements | RSRP accuracy | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-1 |
|  |  | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-2 |
|  |  | FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M1 7-3 |
|  |  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M1 7-4 |
|  | channel quality reporting accuracy | E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A | M1 7-5 |
|  |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A | M1 7-6 |
|  |  | E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B | M1 7-7 |
|  |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B | M1 7-8 |

## 2.3 Test setup

#### Issue 3: Satellite assistance information FFS for following proposals: - RAN4 to reuse the Rel-17 NR NTN test framework for IoT NTN tests which includes predefining configurations for serving cell’s satellite and neighbour cell’s satellite. - RAN4 to introduce assistance information for test cases that require neighbour cell measurements for NB-IoT and eMTC. Details of assistance information is FFS. - RAN4 to introduce assistance information for handover to target cell eMTC. Details of assistance information is FFS.

In our view, the test setup for IoT NTN has no difference compared to that for NR NTN. In NR NTN, neighbor cell measurement and HO are also supported with at most two satellites modelled. In last meeting, some agreements related to high level satellite parameters and reference model to generate satellite motion trajectory were agreed for NR NTN, and the work is ongoing to finalize remaining details such as how to generate ephemeris, epoch time and validity timer.

So far we do not identify any specifics in IoT NTN that would lead to a different test setup, so we suggest to reuse the Rel-17 NR NTN test setup and procedure for IoT NTN tests

Proposal 6: RAN4 to reuse the Rel-17 NR NTN test setup and procedure for IoT NTN tests.

# Conclusions

Observation 1: Different PHR tables are used based on different coverage level depending on the estimated pathloss.

Observation 2: UE can optionally support enhanced PHR with 16 values for finer granularity.

Proposal 1: For NB-IoT and eMTC, legacy PHR table can be reused for IoT NTN.

Proposal 2: Not to define test cases involving neighbor cell detection when the target cell is in enhanced coverage for NGSO.

Observation 3: Based on current RAN1 status, UE will drop the transmission in case the gap is configured regardless of whether the segments are overlapped or not.

Proposal 3: There is no need to define additional gap applicability rules in RAN4 which is in RAN1 scope.

Proposal 4: Not to define test case for UL segmented transmission gap for UE dropping of UL resources

Proposal 5: There is no need to define test cases involving neighbor cell measurement.

Proposal 6: RAN4 to reuse the Rel-17 NR NTN test setup and procedure for IoT NTN tests.

The updated test case list is provided in Table I and II.

Table I. Test case for IoT NTN for NB-IoT

|  |  |  | # |
| --- | --- | --- | --- |
| RRC_IDLE state | Cell Re-Selection | HD – FDD Intra frequency case for UE Category NB1 in normal coverage | NB 1-1 |
|  |  | HD – FDD Inter frequency case for UE Category NB1 in normal coverage | NB 1-2 |
|  |  | HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation | NB 1-3 |
|  |  | HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX | NB 1-4 |
| RRC CONNECTED Mode Mobility | NA | NA | n.a. |
| RRC Connection Control | RRC Re-establishment | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage | NB 2-1 |
|  |  | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage | NB 2-2 |
|  | Random Access | Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage | NB 2-3 |
|  |  | Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage | NB 2-4 |
|  |  | Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage | NB 2-5 |
| Timing | UE Transmit Timing | HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage | NB 3-1 |
|  |  | HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage | NB 3-2 |
|  | UE Timing Advance | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage | NB 3-3 |
|  |  | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage | NB 3-4 |
| Signaling characteristic | RLM | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | NB 4-1 |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage | NB 4-2 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage | NB 4-3 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage | NB 4-4 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage | NB 4-5 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage | NB 4-6 |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | NB 4-7 |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | NB 4-8 |
| Measurement Performance Requirements | channel quality reporting accuracy | HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage | NB 5-1 |
|  |  | HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage | NB 5-2 |
|  |  | HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage | NB 5-3 |
|  |  | HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage | NB 5-4 |
|  |  | HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage | NB 5-5 |
|  |  | HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage | NB 5-6 |

Table II. Test case for IoT NTN for Cat-M

|  |  |  | # |
| --- | --- | --- | --- |
| RRC_IDLE state | Cell Re-Selection | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-1 |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-2 |
|  |  | E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M1 1-3 |
|  |  | E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M1 1-4 |
|  |  | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-5 |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-6 |
| RRC CONNECTED Mode Mobility | Handover and Conditional Handover | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition | M1 2-1 |
|  |  | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition | M1 2-2 |
|  |  | E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA | M1 2-3 |
|  |  | E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA | M1 2-4 |
|  |  | E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-5 |
|  |  | E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-6 |
|  |  | E-UTRAN FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-7 |
|  |  | E-UTRAN HD-FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-8 |
| RRC Connection Control | RRC Re-establishment | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-1 |
|  |  | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-2 |
|  |  | E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-3 |
|  |  | E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-4 |
|  | Random Access | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | M1 3-5 |
|  |  | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | M1 3-6 |
|  |  | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | M1 3-7 |
|  |  | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | M1 3-8 |
| Timing | UE Transmit Timing | E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | M1 4-1 |
|  |  | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | M1 4-2 |
|  |  | E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | M1 4-3 |
|  |  | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | M1 4-4 |
|  | UE Timing Advance | E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | M1 4-5 |
|  |  | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | M1 4-6 |
|  |  | E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | M1 4-7 |
|  |  | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | M1 4-8 |
| Signaling characteristic | RLM | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | M1 5-1 |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | M1 5-2 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | M1 5-3 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | M1 5-4 |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for Cat-M1 UE in CEMode A | M1 5-5 |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync in DRX for Cat-M1 UE in CEMode A | M1 5-6 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for Cat-M1 UE in CEMode A | M1 5-7 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync in DRX for Cat-M1 UE in CEMode A | M1 5-8 |
| Measurement Procedure | Event triggered reporting | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-1 |
|  |  | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-2 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-3 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-4 |
|  |  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M1 6-5 |
|  |  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M1 6-6 |
|  |  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M1 6-7 |
|  |  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M1 6-8 |
| Measurement Performance Requirements | RSRP accuracy | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-1 |
|  |  | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-2 |
|  |  | FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M1 7-3 |
|  |  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M1 7-4 |
|  | channel quality reporting accuracy | E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A | M1 7-5 |
|  |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A | M1 7-6 |
|  |  | E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B | M1 7-7 |
|  |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B | M1 7-8 |

# References

[1] RP-221556 New WID: WID on NB-IoT/eMTC core & performance requirements for Non-Terrestrial Networks

[2] RP-211601 NB-IoT/eMTC support for Non-Terrestrial Networks

[3] R4-2220362 WF on NTN NB-IoT/eMTC RRM requirements, MediaTek inc

[4] R4-2219824 Discussions on NTN IoT RRM performance requirements, Ericsson

[5] R2-2213018 Reply LS on information for neighbor/target cell in IoT NTN