3GPP TSG-RAN WG4 Meeting #106	R4-2301854

Athens, February 27 – March 3, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial simulation results for DL NR carrier phase positioning

Agenda item:	9.21.2

Document for:	Information

Introduction

The Rel-18 WID [1] for NR positioning contains several enhancements, including:

- Support of NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning

NR carrier phase positioning (NR CPP) has been studied in the FS_NR_pos_enh2 study item by RAN1 with company contributions and outcome of the study being captured in TR 38.859 [2].

In this contribution we provide initial simulation results for DL NR CPP. It is noted that these results do not contain any RF/BB impairments. However, the great potential of this positioning technology in terms of improved positioning accuracy can be seen. First simulation assumptions are listed and then the respective simulation results are provided.

Simulation Assumptions

In this section we provide a list of simulation assumptions, considering [3] containing baseline assumptions, which are collected in Table 1.

| Parameter | Value / FR1 |
| --- | --- |
| Carrier frequency/ SCS/ duplexing mode | - 2GHz/ 15kHz / TDD - 2GHz/ 30kHz / TDD |
| Propagation conditions [TS 38.101-4] | AWGN |
| Es/Iot [dB] | -3dB, -13dB |
| Number of UE receive antennas | 1 rx antenna |
| Number of gNB antennas | 1 tx antenna |
| UE measurement bandwidth | Full carrier bandwidth |
| Data and CCH load in PRS symbols | No other cell transmission in its positioning symbols, except PRS |
| Duplex mode | TDD |
| Cyclic prefix | Normal |

Table 1: Simulation assumptions for NR CPP.

As a baseline, we considered PRS BW configurations defined in [4]; the PRS configurations as listed in Table 2 were used in the simulation study. Note the repetition factor relates to repetitions in adjacent slots.

| SCS (KHz) | PRS BW in RB | FFT-size/ sampling rate (Tc) | Num Symbols | Comb Size | Repetition Factor |
| --- | --- | --- | --- | --- | --- |
| 15 | 24 | 512 / 256 | 2 | 2 | 1,2,4 |
|  |  |  | 4 | 4 | 1,2,4 |
|  | 52 | 1024 / 128 | 2 | 2 | 1,2,4 |
|  |  |  | 4 | 4 | 1,2,4 |
|  | 104 | 2048 / 64 | 2 | 2 | 1,2,4 |
|  |  |  | 4 | 4 | 1,2,4 |
|  | 268 | 4096 / 32 | 2 | 2 | 1,2,4 |
|  |  |  | 4 | 4 | 1,2,4 |
| 30 | 24 | 512 / 128 | 2 | 2 | 1,2,4 |
|  |  |  | 4 | 4 | 1,2,4 |
|  | 48 | 1024 / 64 | 2 | 2 | 1,2,4 |
|  |  |  | 4 | 4 | 1,2,4 |
|  | 132 | 2048 / 32 | 2 | 2 | 1,2,4 |
|  |  |  | 4 | 4 | 1,2,4 |
|  | 272 | 4096 / 16 | 2 | 2 | 1,2,4 |
|  |  |  | 4 | 4 | 1,2,4 |

Table 2: PRS configurations used for NR CPP study for SCS = 15kHz and 30kHz.

No RF/BB impairments are included in the simulations.

Simulation Results

In this section we provide the simulation results for the above listed simulation assumptions. The results are reported for 90 percentile CDF of the DL carrier phase estimation error, i.e., 90% of measurements for given Es/Iot (-13dB, -3dB).

FR1, SCS = 15kHz

The DL carrier phase accuracy for AWGN channel in percentage of the carrier wavelength, derived from the 90 percentiles of the CDF for the measured carrier phase error, is listed in Table 3.

| SCS (kHz) | PRS BW in RB | FFT-size/ sampling rate (Tc) | Num Symbols/ Comb Size | Repetition Factor | Es/Iot = -13dB (accuracy in % of carrier wavelength) | Es/Iot =-3dB) (accuracy in % of carrier wavelength) |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 24 | 512 / 256 | 2 / 2 | 1 | 6.99 | 1.7 |
|  |  |  |  | 2 | 3.85 | 1.24 |
|  |  |  |  | 4 | 2.74 | 0.87 |
|  |  |  | 4/ 4 | 1 | 8.64 | 1.79 |
|  |  |  |  | 2 | 3.95 | 1.22 |
|  |  |  |  | 4 | 2.8 | 0.89 |
|  | 52 | 1024 / 128 | 2 / 2 | 1 | 3.9 | 1.24 |
|  |  |  |  | 2 | 2.7 | 0.85 |
|  |  |  |  | 4 | 1.87 | 0.6 |
|  |  |  | 4 / 4 | 1 | 3.79 | 1.22 |
|  |  |  |  | 2 | 2.67 | 0.85 |
|  |  |  |  | 4 | 1.93 | 0.62 |
|  | 104 | 2048 / 64 | 2 / 2 | 1 | 2.77 | 0.87 |
|  |  |  |  | 2 | 1.93 | 0.6 |
|  |  |  |  | 4 | 1.39 | 0.44 |
|  |  |  | 4 / 4 | 1 | 2.73 | 0.87 |
|  |  |  |  | 2 | 2.07 | 0.66 |
|  |  |  |  | 4 | 1.39 | 0.45 |
|  | 268 | 4096 / 32 | 2 / 2 | 1 | 1.89 | 0.6 |
|  |  |  |  | 2 | 1.32 | 0.42 |
|  |  |  |  | 3 | 0.91 | 0.29 |
|  |  |  | 4 / 4 | 1 | 1.88 | 0.59 |
|  |  |  |  | 2 | 1.33 | 0.43 |
|  |  |  |  | 4 | 0.94 | 0.3 |

Table 3: DL carrier phase accuracy for AWGN channel in percentage of carrier wavelength (AWGN, SCS = 15 kHz).

FR1, SCS = 30 kHz

The DL carrier phase accuracy for AWGN channel in percentage of the carrier wavelength, derived from the 90 percentiles of the CDF for the measured carrier phase error, is listed in Table 4.

| SCS (KHz) | PRS BW in RB | FFT-size/ sampling rate (Tc) | Num Symbols/ Comb Size | Repetition Factor | Es/Iot = -13dB (accuracy in % of carrier wavelength) | Es/Iot =-3dB) (accuracy in % of carrier wavelength) |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 24 | 512 / 128 | 2 / 2 | 1 | 23.18 | 1.72 |
|  |  |  |  | 2 | 4.06 | 1.27 |
|  |  |  |  | 4 | 2.8 | 0.9 |
|  |  |  | 4/ 4 | 1 | 25.12 | 1.84 |
|  |  |  |  | 2 | 4.05 | 1.27 |
|  |  |  |  | 4 | 2.89 | 0.91 |
|  | 48 | 1024 / 64 | 2 / 2 | 1 | 4.08 | 1.24 |
|  |  |  |  | 2 | 2.75 | 0.87 |
|  |  |  |  | 4 | 2.04 | 0.66 |
|  |  |  | 4 / 4 | 1 | 4.1 | 1.26 |
|  |  |  |  | 2 | 2.77 | 0.89 |
|  |  |  |  | 4 | 1.98 | 0.63 |
|  | 132 | 2048 / 32 | 2 / 2 | 1 | 2.69 | 0.85 |
|  |  |  |  | 2 | 1.94 | 0.62 |
|  |  |  |  | 4 | 1.37 | 0.44 |
|  |  |  | 4 / 4 | 1 | 2.68 | 0.85 |
|  |  |  |  | 2 | 1.91 | 0.61 |
|  |  |  |  | 4 | 1.36 | 0.44 |
|  | 272 | 4096 / 16 | 2 / 2 | 1 | 1.89 | 0.61 |
|  |  |  |  | 2 | 1.38 | 0.44 |
|  |  |  |  | 4 | 0.98 | 0.31 |
|  |  |  | 4 / 4 | 1 | 1.9 | 0.6 |
|  |  |  |  | 2 | 1.4 | 0.45 |
|  |  |  |  | 4 | 0.98 | 0.31 |

Table 4: DL carrier phase accuracy for AWGN channel in percent of carrier wavelength (AWGN, SCS = 15 kHz).

Discussion

From the simulation results reported above for the AWGN channel without RF/BB impairments, we observed:

- carrier phase accuracy dependency on the bandwidth is not so significant (except for PRS BW of 24 PRB) and

- carrier phase measurement error is less than 5% of the carrier wavelength (except for PRS BW of 24 PRB),

thereby demonstrating the great potential of this positioning technology in terms of improved positioning accuracy.

Thus, following observations are made:

The carrier phase accuracy dependency on bandwidth is not so significant (except for PRS BW of 24 PRB).

The carrier phase accuracy is less than 5% of the carrier wavelength, demonstrating the great potential of this positioning technology in terms of improved positioning accuracy.

Conclusion

In this contribution we have provided initial simulation results for DL NR carrier phase accuracy without RF/BB impairments.

Following observations are made.

- The carrier phase accuracy dependency on bandwidth is not so significant (except for PRS BW of 24 PRB).

The carrier phase accuracy is less than 5% of the carrier wavelength, demonstrating the great potential of this positioning technology in terms of improved positioning accuracy.

- References

- RP-223549, “New WID on Expanded and Improved NR Positioning”, Intel Corporation, CATT, Ericsson, RAN#98-e

- 3GPP TR 38.859 V18.0.0, “Study on Expanded and Improved NR Positioning (Release 18)”

- R4-2002287, “Updated link level simulation assumption for RSTD and RSRP”, Huawei, HiSilicon

- R4-2108313, “Summary of link level simulation result of RSTD, PRS RSRP and UE Rx-Tx time difference”, Intel