3GPP TSG-RAN WG4 Meeting #106	R4-2301877

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.13.1.3

Source:	Ericsson

Title:	TP to TR 38.876: Update of simulation assumptions

Document for:	Approval

# 1	Introduction

In [1] we present results for simulation calibration. Whilst preparing these results, a few corrections and needed additions to the simulation assumptions captured in the TR have come to light. This TP captures the needed updates.

# 2	Text Proposal

#### 6.2.1.2 TN Network Layout

A rural hexagonal grid layout is assumed for TN network clusters with the parameters of table 6.2.1.1.

Table 6.2.1.2-1: Simulation scenarios for ATG coexistence study

| Parameters |  | Values |
| --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | 7.5 km (4GHz) 7.5 km (2GHz) |
| BS antenna height |  | 25 m |
| UE location | Outdoor/indoor | Outdoor only |
|  | Indoor UE ratio | 0% |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |
|  | LOS/NLOS | LOS and NLOS, see subclause 6.2.5 |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 35 m |

---------------------   Next changed section  ------------------------------

### 6.2.2 System parameters

#### 6.2.2.1 ATG parameters

The system parameters for ATG BS for co-existence study is assumed as below.

Table 6.2.2.1-1: system parameters for ATG BS

| ATG BS height | 30m |
| --- | --- |
| Carrier frequency | 2GHz, 4GHz |
| Frequency reuse factor | 1 |
| Duplex mode | FDD@2GHz, TDD@4GHz |
| Channel bandwidth | 20MHz@2GHz, 100MHz@4GHz |
| Subcarrier spacing (SCS) | 15k@2GHz, 30k@4GHz |
| Number of cells | one cell |
| UE distribution | Single ATG UE per ATG cell Horizontal: Distributed randomly along a straight line in the boresight of the ATG BS and intersecting the ATG BS [Vertical: Distributed between 3km and 10km] |
| Indoor UE percentage | 0% |
| Number of DL active UEs per cell (NOTE 2) | one UE |
| Number of UL active UEs per cell (NOTE 2) | one UE |
| DL scheduled bandwidth per UE | Full bandwidth |
| UL scheduled bandwidth per UE | Full bandwidth |
| UL power control SINR target | 15dB |
| Traffic model | Full buffer |
| ATG BS maximum output power | 46dBm BS output power for 2GHz 53dBm BS TRP output power for 4GHz |
| ATG BS noise figure | 5dB |
| Handover margin | Not needed |
| NOTE 1: ATG BS is assumed to serve UEs in the rural environment. NOTE 2: Same as the number of BS beam(s); |  |

#### 6.2.2.2 ATG UE parameters

The system parameters for ATG UE are assumed as below.

Table 6.2.2.2-1: system parameters for ATG UE

| ATG UE altitude | Vertical: Distributed between 3km and 10km |
| --- | --- |
| Carrier frequency | 2GHz, 4GHz |
| ATG UE max TX power in dBm | Referred to Issue 2-5-2 |
| ATG UE min TX power in dBm | - Minimum TX power is not reached by the ATG UE in any scenario. |
| ATG UE noise figure | 9dB |

#### 6.2.2.3 TN BS and UE parameters

The system parameters for TN BS and TN UE are assumed as below.

Table 6.2.2.3-1: system parameters for TN BS and UE

| Parameters | Rural | Rural |
| --- | --- | --- |
| Carrier frequency | 2GHz | 4GHz |
| Channel bandwidth | 20MHz | 100MHz |
| Scheduled channel bandwidth per UE (DL) | 1 | 1 |
| Scheduled channel bandwidth per UE (UL) | 3 | 3 |
| The number of active UE (DL) (Note 1) | 1 | 1 |
| The number of active UE (UL) (Note 1) | 3 | 3 |
| Traffic model | full buffer | full buffer |
| DL power control | No | No |
| UL power control | Yes | Yes |
| UL SINR target | 15dB | 15dB |
| TN BS-UE min distance in meters | 35m | 35m |
| TN BS max TX power in dBm | 46dBm | 53dBm |
| TN UE max TX power in dBm | 23dBm | 23dBm |
| TN UE min TX power in dBm | -40dBm | -40dBm |
| TN BS Noise figure in dB | 5dB | 5dB |
| TN UE Noise figure in dB | 9dB | 9dB |
| Handover margin | 3dB | 3dB |
| Note 1 Same as the number of BS beam(s); |  |  |

### 6.2.3 Antenna and beamforming pattern modelling

#### 6.2.3.1 ATG BS antenna model

For ATG BS antenna modelling, the following two options for antenna modelling could be used for ATG coexistence study.

Option 1: non sub-array model

Table 6.2.3.1-1: AAS antenna parameters for non sub-array model

|  | ATG |
| --- | --- |
| Base Station Antenna Characteristics |  |
| Antenna pattern | TR 38.921 |
| Element gain (dBi) (Note 2) | 7.1 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements |
| Number of supported polarizations, P | 2 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V |
| Array Ohmic loss (dB) (Note 2) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 |
| Base station maximum coverage angle in the horizontal plane (degrees) | 120 |
| Base station vertical coverage range (degrees) (Note 1) | 25 |
| Mechanical uptilt (degrees) | 14 |

Option 2: sub-array model

Table 6.2.3.1-1: AAS antenna parameters for sub-array model

| Parameter | Macro urban |
| --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna sub-array configuration (Row × Column) (Note 4) | 4 × 8 elements |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Array Ohmic loss (dB) (Note 2) | 2 |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 28 |
| Base station horizontal coverage range (degrees) | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 10 |
| Mechanical up-tilt (degrees) | 6.5 |

#### 6.2.3.2 ATG UE antenna model

For 2GHz, assume omni-directional antenna, assume [40dBm] UE output power for calibration (as worst case for simulation purposes).

For 4GHz, assume that UE is equipped with directional antenna, assume a UE EIRP of [43dBm] for calibration (as worst case for simulation purposes)

  - Use following as the starting point for calibration.

Table 6.2.3.2-1:  antenna parameters for phase antenna array

|  | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V |
| --- | --- | --- |
|  | Element gain (dBi) | 5 dBi |
|  | Horizontal/vertical front-to-back ratio (dB) | 30dBc |
|  | Antenna polarization | Linear ±90º |
|  | Antenna array configuration (Row × Column x Polarization) (Note 4) | (8x2x2) or (16x1x2) |
|  | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |

# References

R4-2301880, ATG simulation calibration results, Ericsson, RAN4#106, February-March 2023