3GPP TSG-RAN WG4 Meeting #106	R4-2301889

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	6.2.6.1.2

Source:	Ericsson

Title:	Simulation results for PDCCH in FR2-2

Document for:	Information

# 1	Introduction

In this paper, we present the simulation results on the PDCCH demodulation performance in the frequency range 52.6 GHz to 71 GHz according to PDSCH partial RB allocation. Following the WF [1], we carried out our simulations using the agreed propagation environments.

Whether to reduce PDCCH CORESET BW for PDSCH Requirements with reduced PRB Allocation - Agreement: Reduce PDCCH CORESET BW if larger than PDSCH RB allocation (Apple) PDCCH Configuration for PDSCH Requirements with PDSCH Allocation=32RBs; - Proposals - Option 1: CORESET BW=30PRB, Duration = 1Sym, AL=4 (Apple); Agreement: If PDSCH Requirements with PDSCH Allocation=32RBs introduced, option 1 shall be considered as starting point for further discussion. PDCCH Configuration for PDSCH Requirements with PDSCH Allocation=20RBs; - Proposals - Option 1: CORESET BW=18PRB, Duration = 2Sym, AL=4 (Apple); Agreement: If PDSCH Requirements with PDSCH Allocation=20RBs introduced, option 1 shall be considered as starting point for further discussion. Channel Model for FR2-2 PDCCH Requirements Test #5 (480kHz SCS, AL4); Agreement: Remove test #5 for PDCCH due to testable SNR limitation.

# 2	PDCCH Simulation Results

## 2.1	Simulation setup

As per WF [1], PDSCH with partial RB allocation have been stated in the following case

| Duplex CBW/SCS | Antenna configuration | RB allocation | MCS and rank | Channel model | % of Peak Throughput | Tentative Requirement | Max Testable SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDD 100MHz/120kHz | 2x2 Low | 32 | MCS17, Rank 1 | TDLD30-200 | 70 | 11.2 | 13.2 |
| TDD 400MHz/480kHz | 2x2 Low | 20 | MCS13, Rank 1 | TDLD10-200 | 70 | 8.0 | 8.9 |

Accordingly, we carried out PDCCH demodulation simulations for 100 MHz/120 kHz with AL = 4 and 400 MHz/480 kHz with AL = 8. Furthermore, the test parameters should be updated as follow

Test Parameters for 100 MHz/120 kHz (reference from TS 38.101-4 Table 7.3.2.2-1)

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR2.120-1 |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 6 | 2 |
| Interleaver size |  | 2 | 3 |
| Shift index |  | 0 |  |

| Test case | Interleaver size | REG bundle size | CORE SET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI Format | Infor Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 2 | 6 | 24 | 1 | 4 | TDLA30-650 | 1x2 ULA Low | 1_1 | 56 |

Test parameters for 400 MHz/480 kHz

| Parameter |  | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR2.480-1 |  |
| CCE to REG mapping type |  |  | Interleaved |  |
| REG bundle size |  |  | 6 for Test 2-1 3 for Test 2-2 | 2 |
| Interleaver size |  |  | 3 | 3 |
| Shift index |  |  | [0] |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Every 4th slot |  |
|  | CSI Periodicity | Slots | [640] |  |
| CSI-RS for tracking | CSI-RS offset | Slots | [320 for CSI-RS resource 1 and 2 321 for CSI-RS resource 3 and 4] |  |
| NZP CSI-RS for beam refinement | CSI-RS periodicity | Slots | [640 for CSI-RS resource 1,2] |  |

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI Format | Infor Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 2-1 | 3 | 6 | 18 | 3 | 8 | TDLA10-200 | 1x2 ULA Low | 1_1 | 56 |
| Test 2-2 | 3 | 3 | 18 | 3 | 8 | TDLA10-200 | 1x2 ULA Low | 1_1 | 56 |

Furthermore, we used the following general parameters:

- Numerologies: 120 KHz/100 MHz and 480 KHz/400 MHz.

- Max Doppler frequency/DMRS: f_D = 200 Hz and f_D = 650 Hz

- Pre-loaded Tx EVM: 6%

- Additive Rx EVM: 0%

## 2.2	Simulation results

In this section, we present the PDCCH performance in terms of the probability of miss-detection of the downlink scheduling grant (Pm-dsg) as function of SNR (dB). The PDCCH performance metric is defined as the needed SNR (dB) for a Pm-dsg (BLER) at 1%.

SCS 120 kHz/CBW 100 MHz

| Test case | Interleaver Size | REG bundle size | CORE SET RB | CORE SET duration | Aggre-gation level | Antenna config | DCI format | Infor Bit | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 2 | 6 | 24 | 1 | 4 | 1x2 ULA Low | 1_1 | 56 | TDLA30-650 | 2.17 |

Observation 1: For SCS 120 kHz/CBW 100 MHz and referring to the max DL testable SNR at [9.8] dB and considering an additional margin of 3 dB for RF impairments, we note that the proposed SNR levels are adequate to the defined parameters.

SCS 480 kHz/CBW 400 MHz

| Test case | Interleaver Size | REG bundle size | CORE SET RB | CORE SET duration | Aggre-gation level | Antenna config | DCI format | Infor Bit | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 2-1 | 3 | 6 | 18 | 3 | 8 | 1x2 ULA Low | 1_1 | 56 | TDLA10-200 | -2.48 |
| Test 2-2 | 3 | 3 | 18 | 3 | 8 | 1x2 ULA Low | 1_1 | 56 | TDLA10-200 | -2.13 |

Observation 2: For SCS 480 kHz/CBW 400 MHz and referring to the max DL testable SNR at [2.6] dB and considering an additional margin of 3 dB for RF impairments, we note that the proposed SNR levels are adequate to the defined parameters.

# 3	Summary

In this paper, we carried out simulations for PDCCH when partial allocation for PDSCH applies. Here we propose some observations:

Observation 1: For SCS 120 KHZ/CBW 100 MHz and referring to the max DL testable SNR at [9.8] dB and considering an additional margin of 3 dB for RF impairments, we note that the proposed SNR levels are adequate to the defined parameters.

Observation 2: For SCS 480 kHZ/CBW 400 MHz and referring to the max DL testable SNR at [2.6] dB and considering an additional margin of 3 dB for RF impairments, we note that the proposed SNR levels are adequate to the defined parameters.

# References

- R4-2220187, “WF on FR2-2 UE demodulation requirements”, Qualcomm.