3GPP TSG-RAN4 Meeting #106	R4-2301955

# Athens, Greece, 27 February – 3 March, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on setup for NTN RRM test cases R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Perf |  |  |  |  |  | Date: |  |  | 2022-12-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Setup and procedure for NTN RRM tests are agreed, and need to be captured in RAN4 spec as reference for RAN5 when defining the comformance tests including the detailed satellite ephemeris. - Some general test setup are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture the setup and procedure for NTN RRM tests based on RAN4 agreements in the Annex. - Add general test setup are missing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN4 agreements regarding setup and procedure for NTN RRM tests are not captured, and cannot be used by RAN5 to develop detailed test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.36, B.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.3.36	Testing related to Satellite access

A.3.36.1	Introduction

In annex A test cases are defined for verifying various type of RRM requirements related to satellite access.

A.3.36.2	Principle of testing GSO and NGSO scenarios

In Annex A, RRM test cases related to satellite access are defined for both GSO and NGSO. The testing principle for these test cases is as follows:

-	A UE capable of GSO only is required to pass the test cases with GSO.

-	A UE capable of NGSO only is required to pass the test cases with NGSO.

-	A UE capable of both GSO and NGSO is required to pass the test cases with NGSO only.

Support of GSO and NGSO scenario is indicated via ntn-ScenarioSupport-r17.

A.3.36.2	Principle of testing different RRM requirements

In Annex A, RRM test cases related to satellite access are defined for all applicable RRM requirements. The testing principle for these test cases is as follows:

-	A UE capable of NTN only is required to pass all the test cases defined in clause A.14.

-	A UE capable of both TN and NTN is required to pass the test cases for NTN specific requirements in Table A.3.36.2-1.

Table A.3.36.2-1: Test cases for NTN specific requirements

| Clause | Test case slogan |
| --- | --- |
| A.14.1.2 | Cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements] |
| A.14.1.3 | Time-based cell reselection to FR1 intra-frequency NR cell |
| A.14.1.4 | Location-based cell reselection to FR1 intra-frequency NR cell |
| A.14.1.7 | Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements] |
| A.14.1.8 | Time-based Cell reselection to FR1 inter-frequency NR satellite access case |
| A.14.1.9 | Location-based Cell reselection to FR1 inter-frequency NR satellite access case |
| A.14.2.1.3 | Intra-frequency SAN time-based conditional Handover from FR1 to FR1 |
| A.14.2.1.4 | Inter-frequency SAN time-based conditional Handover from FR1 to FR1 |
| A.14.2.1.5 | Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 |
| A.14.2.1.6 | Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 |
| A.14.3.1.1 | NR UE Transmit Timing Test for FR1 |
| A.14.5.1.1 | SA event triggered reporting tests without gap under non-DRX |
| A.14.5.1.2 | SA event triggered reporting tests without gap under DRX |
| A.14.5.1.3 | SA event triggered reporting tests without gap under non-DRX with SSB index reading |
| A.14.5.1.4 | SA event triggered reporting tests with single measurement gap under non-DRX for satellite access |
| A.14.5.1.5 | SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access |
| A.14.5.1.6 | SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access |
| A.14.5.2.1 | Event triggered reporting test without gap under non-DRX |
| A.14.5.2.2 | Event triggered reporting tests without gap under DRX |
| A.14.6.3.1 | SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell |
| A.14.6.3.2 | SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell |
| A.14.6.4.1 | SSB based L1-RSRP measurement |
| A.14.6.4.2 | CSI-RS based L1-RSRP measurement on resource set with repetition off |

A.3.36.3	Principle of testing different ephemeris formats

In Annex A, RRM test cases related to satellite access are defined and satellite ephemeris information are sent to UE in each test case.

EphemerisInfo is configured in format of velocity state vector (PositionVelocity) for the following test cases:

      - RRC_IDLE state mobility

        - A.14.1.3, A.14.1.4, A.14.1.5, A.14.1.6, A.14.1.9 and A.14.1.10

      - Handover

        - A.14.2.1.3, A.14.2.1.4, A.14.2.1.5 and A.14.2.1.6

      - Timing

        - A.14.3.1

      - Radio link Monitoring

        - A.14.4.1.1, A.14.4.1.2, A.14.4.1.5 and A.14.4.1.6

      - Beam Failure Detection and Link recovery procedures

        - A.14.4.2.1, A.14.4.2.3 and A.14.4.2.5

      - Active BWP switch

        - A.14.4.3.1

      - UE specific CBW change

        - A.14.4.4.1

      - Pathloss reference signal switching delay

A.14.4.5.1

      - Intra-frequency Measurements

        - A.14.5.1.1, A.14.5.1.3 and A.14.5.1.5

      - Inter-frequency Measurements

        - A.14.5.2.1, A.14.5.2.3 and A.14.5.2.7

      - L1-RSRP measurement for beam reporting

        - A.14.5.3.1 and A.14.5.3.3

      - SS-RSRP

        - A.14.6.1.1

      - SS-RSRQ

        - A.14.6.2.1

      - SS-SINR

        - A.14.6.3.1

      - L1-RSRP measurement for beam reporting

        - A.14.6.4.1

EphemerisInfo is configured in format of orbital parameters (Orbital) for the following test cases:

      - RRC_IDLE state mobility

        - A.14.1.1, A.14.1.2, A.14.1.5 and A.14.1.6

      - Handover

        - A.14.2.1.1, A.14.2.1.2

      - RRC Connection Mobility Control

        - A.14.2.2.1, A.14.2.2.2 and A.14.2.2.3

      - Timing

        - A.14.3.2

      - Radio link Monitoring

        - A.14.4.1.3, A.14.4.1.4, A.14.4.1.7 and A.14.4.1.8

      - Beam Failure Detection and Link recovery procedures

        - A.14.4.2.2, A.14.4.2.4 and A.14.4.2.6

      - Active BWP switch

        - A.14.4.3.2

      - Intra-frequency Measurements

        - A.14.5.1.2, A.14.5.1.4 and A.14.5.1.6

      - Inter-frequency Measurements

        - A.14.5.2.2, A.14.5.2.4, A.14.5.2.6 and A.14.5.2.8

      - L1-RSRP measurement for beam reporting

        - A.14.5.3.2 and A.14.5.3.4

      - SS-RSRP

        - A.14.6.1.2

      - SS-RSRQ

        - A.14.6.2.2

      - SS-SINR

        - A.14.6.3.2

      - L1-RSRP measurement for beam reporting

        - A.14.6.4.2

A.3.36.4	General setup for SIB19

The general parameters for SIB19 setup is specified in Table A.3.36.4-1.

Table A.3.36.2-1: Test cases for NTN specific requirements

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5s GEO: 900s |
| cellSpecificKoffset | slot | LEO: 8 GEO: 256 |
| kmac | slot | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ntn-PolarizationDL |  | linear |
| ntn-PolarizationUL |  | linear |
| ephemerisInfo |  |  |
| ta-Report |  | Not configured |

<End of Change 1>

<Start of Change 2>

B.5	High level test procedure for SAN RRM tests

The following high level steps are conducted for test cases for SAN defined in clause A.14.

-	A set of ephemeris information are pre-defined for each satellite corresponding to different epoch times in [TS TBD].

-	For GEO an altitude of 35,786km is considered. an elevation angle relative to a UE position shall not be smaller than 30 deg during entire test time

-	For LEO an altitude of 600km on a circular orbit is considered.

-	A motion trajectory is generated for each satellite based on the ephemeris using Eckstein-Hechler model.

-	UE location is determined for the test. The ephemeris and the the UE location should be designed such that elevation angle relative to the UE position shall not be smaller than 30 deg during entire test time.

-	Test equipment adjusts the time and frequency of transmission based on the satellite motion trajectory and UE location during test time to emulate the position and velocity change of the satellite relative to the UE.

<End of Change 2>