3GPP TSG-RAN WG4 Meeting # 106	R4-2302362

Athens, Greece, February 27 – March 3, 2023

Agenda Item: 9.9.2.3

Source: MediaTek Inc.

Title: Discussion on other potential enhancement for FR2 SCell activation

Document for: Discussion

# Introduction

In R15, SCell activation delay requirement is introduced, and in R16/R17, some activation requirements are enhanced to accelerate the activation procedures. The delay requirement may be different according to the various conditions, e.g., frequency range, known/unknown cell and activating serving cell on the same band, etc. However, some of them, especially for unknown case, are still too long. Therefore, following the discussion in the last meeting, in this paper we further discuss other potential enhancement for unknown SCell activation in FR2. The WF from the last meeting is captured in [1].

# Discussion

The SCell activation delay for unknown SCell in FR2 is too long, which involves L3 (AGC adjustment and cell search) and L1 (L1-RSRP measurement & reporting) procedures as shown in Figure 1.

L1

L3

TCI activation and CSI-RS meas. Max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay)

L1-RSRP meas. & report

Cell search

MAC-CE processing & RF retuning

THARQ

MAC-CE For SCell activation

CSI reporting

AGC

Figure 1: Unknown SCell activation delay in FR2 (when P CSI-RS is used).

To reduce the SCell activation delay for the unknown SCell in FR2, enhancements are required on these procedures. For L3 part, only three samples are used (2 for AGC and 1 for cell search), which are already tighten and sample number can’t be reduced any further. For L1 part, on the other hand, has more potential for enhancements.

In this context, we discuss a different approach for the enhancement for the unknown SCell activation in FR2. The Contention Based Random Access (CBRA) is proposed as an alternative to L1-RSRP measurements for enhancing unknown SCell activation delay in FR2. According to existing TS 38.133 [2], delay is too long when the target SCell is unknown and is the first activating SCell in one band. To accelerate such SCell activation delay, one possibility is to use CBRA for the SCell. Following the similar logic as PSCell addition, the RACH resource can be pre-configured for certain SCells. In that case, UE may measure the SSBs for AGC tunning and cell search first and then transmit PRACH immediately to network based on previous L3 measurement on a suitable PRACH occasion. After four steps message transmission, the link will be established. Comparing with legacy SCell activation requirement, the procedures of TCI state indication, L1-RSRP measurement and reporting can be removed as shown in Figure 2.

PRACH procedure

TCI activation and CSI-RS meas. Max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay)

L1-RSRP meas. & report

Cell search

MAC-CE processing & RF retuning

THARQ

CSI reporting

AGC

Figure 2: Unknown SCell activation delay in FR2 (when P CSI-RS is used).

This proposal was captured in the WF [1] as shown below:

Issue 3-1-6: CBRA for FR2 unknown SCell activation - FFS - Option 1 (Apple, Ericsson): RAN4 not to consider CBRA for SCell activation. - Option 2 (MediaTek): Contention based random access (CBRA) can be used to activate first unknown SCell in one band, which can remove the need for L1-RSRP measurement and TCI state indication, and as a result, enhance the overall activation delay for unknown SCell.

In the last meeting, companies have commented and raised their concerns regarding CBRA approach in two aspects: in its interruption impact, and in its impact on RAN1/RAN2 work. For these concerns, we provide our comments as below:

- Interruption impact: we appreciate the concern raised on that CBRA could cause interruption. However, in our view, with the UE capability parallelTxPRACH-SRS-PUCCH-PUSCH defined in 38.306, UE should be able to perform CBRA on the SCell without causing interruption on the serving cell. Therefore, we believe CBRA approach in Option 1 can still bring enhancement to the unknown SCell activation delay in FR2.

- RAN1/RAN2 impact: We understand that this approach could have RAN1/RAN2 impact. However, in our view, even other potential solutions could also have similar impact to RAN1/RAN2. For example, the proposal of the framework in Issue 3-5-1 on triggering the UE to report RSRP measurements before L1 part to skip L1-RSRP will also have similar impact on RAN1/RAN2 work. In our view, this approach can be performed even faster than triggering the UE to report the RSRP measurements.

Proposal 1: CBRA can be used to activate first unknown SCell in one band, which can help skipping L1-RSRP measurement and TCI state indication.

- Note that with UE capability parallelTxPRACH-SRS-PUCCH-PUSCH defined in 38.306, UE should be able to perform CBRA on the SCell without causing interruption. In addition, any solution to skip L1/L3 parts could have similar impact on RAN1/RAN2 work.

# Summary

In this paper, the discussion on other potential enhancement for SCell activation delay reduction is provided. The following proposals were introduced:

Proposal 1: CBRA can be used to activate first unknown SCell in one band, which can help skipping L1-RSRP measurement and TCI state indication.

- Note that with UE capability parallelTxPRACH-SRS-PUCCH-PUSCH defined in 38.306, UE should be able to perform CBRA on the SCell without causing interruption. In addition, any solution to skip L1/L3 parts could have similar impact on RAN1/RAN2 work.

# Reference

- R4-2220440, WF on Even Further RRM enhancement for NR and MR-DC – Part 1, Apple.

- 3GPP TS 38.133, V17.6.0, “NR; Requirements for support of radio resource management”.