3GPP TSG-RAN WG4 Meeting #106	     											         R4-2302877

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:			9.25.6

Source:	Moderator (THALES)

Title:	WF for SAN RF requirements on above 10 GHz

Document for:	Agreement

# Introduction

This Way Forward considers remaining issues from R4-2303757, Topic summary for [106][312] NR_NTN_enh_Part1, with the following list of topics/sub-topics/issues (presented in R4-2303757):

- Topic #1: System parameters

* Include band definition

  - Sub-topic 1-1: Operating band definition and related parameters.

    - Issue 1-1-1: Above 10 GHz NTN band definition

    - Issue 1-1-2: Channel BW

    - Issue 1-1-3: SCS

    - Issue 1-1-4: Spectral Utilization

    - Issue 1-1-5: FR2-0(-NTN)/FR2-1-NTN sub-frequency range specification

    - Issue 1-1-6: Tx-Rx frequency separation

  - Sub-topic 1-2: Channel raster and synchronization raster

    - Issue 1-2-1: NR-ARFCN

    - Issue 1-2-2: GSCN

    - Issue 1-2-3: LS to RAN1

    - Issue 1-2-4: LS to RAN1 & RAN2

  - Sub-topic 1-3: Deployment, Regulatory and Protection Requirements

    - Issue 1-3-1: Protection requirements

    - Issue 1-3-2: Additional protection requirements

    - Issue 1-3-3: Deployment and Power Class

    - Issue 1-3-4: Deployment and Power Control

    - Issue 1-3-5: Deployment and beam pointing report

- Topic #2: SAN RF requirements

  - Sub-topic 2-1: General discussions

    - Issue 2-1-1: OBUE requirement for SAN above 10 GHz

    - Issue 2-1-2: SAN power flux density requirements

  - Sub-topic 2-2: SAN Conducted Requirements

    - Issue 2-2-1: Impact of insertion loss and link budget

    - Issue 2-2-2: SAN Tx Requirements

    - Issue 2-2-3: SAN Rx Requirements

  - Sub-topic 2-3: SAN OTA Requirements

    - Issue 2-3-1: SAN OTA Tx Requirements

    - Issue 2-3-2: SAN OTA Rx Requirements

- Topic #3: NR NTN enhancement workplan

  - Sub-topic 3-1 Coverage enhancement section from Revised Work plan (R4-2300973)

  - Sub-topic 3-2 NR-NTN deployment in above 10 GHz bands section from Revised Work plan (R4-2300973)

  - Sub-topic 3-3 Network verified UE location section from Revised Work plan (R4-2300973)

  - Sub-topic 3-4 NTN-TN and NTN-NTN mobility and service continuity enhancements section from Revised Work plan (R4-2300973)

  - Sub-topic 3-5 Entire Revised Rel-18 NR-NTN-enh work item Work plan (R4-2300973)

In this Way Forward we discuss only remaining issues from Topic#2 from Topic summary for [106][312] NR_NTN_enh_Part1. For band definition aspects please check agreed R4-2302879 (Ad-hoc minutes for system parameters of above 10 GHz NTN band introduction). For SAN system parameters please check R4-2302909 (WF for system parameters of above 10 GHz NTN band).

Topic#3 is to be discussed and endorsed during the RAN4#106 meeting if no concerns.

# Agreements from online BSRF session Day 1

Issue 2-1-1: OBUE requirement for SAN above 10 GHz

- Agreement: As starting point, RAN4 should consider specifying OBUE requirement for SAN above 10 GHz based on ITU SM.1541-6 / OoB masks

Issue 2-1-2: SAN power flux density requirements

- Agreement: FFS whether need to consider SAN power flux density requirements in RAN4 specification.

Issue 2-2-1: Impact of insertion loss and link budget

- Agreement:

  - Prioirtize the work on radiated requirements for Ka band, FFS whether conductive requirements needed or not

    - if the conductive requirements to be defined for Ka-band NTN SAN, the impact of insertion loss and link budget should be further clarified.

Issue 2-3-1: SAN OTA Tx Requirements

- Agreement:

| OTA Tx requirements | Applicability notes |
| --- | --- |
| Radiated transmit power (EIRP) | define the requirement based on EIRP manufacturer declaration, with the accuracy of the declared output power value reusing the [± 3.4 dB] from the BS type 2-O specification in TS 38.104 (Normal test conditions). |
| Radiated transmit power (EIRP) | no requirement for Extreme test conditions. |
| OTA Satellite Access Node output power (TRP) | define the requirement based on TRP manufacturer declaration, with the accuracy of the declared output power value reusing the [± 3 dB] from the BS type 2-O specification in TS 38.104 (Normal test conditions). |
| OTA Output power dynamics |  |
| OTA RE power control dynamic range | Align the approach with the BS type 2-O and no requirement to be defined. |
| OTA Total power dynamic range | define the requirement following the approach in TS 38.104 section 9.4.3.3 for the Minimum requirement for BS type 2-O total power dynamic range derivation in FR2-1. |
| OTA Transmit ON/OFF power | N/A for NTN SAN, since the Ka-band is FDD band. |
| OTA Transmitted signal quality |  |
| OTA Frequency error | define the OTA Frequency error requirement for SAN as 0.05 ppm observed over 1 ms. |
| OTA Modulation quality | Reuse QPSK and 16QAM requirements of 17.5% and 12.5%, respectively. FFS whether 64QAM can be considered for 2-0 |
| OTA Time alignment error | N/A for NTN SAN (no support for MIMO due to NTN specific channel characteristics, similar as Rel-17 NTN 5G NR). |
| Unwanted emissions |  |
| OTA Occupied bandwidth | define the requirement of OTA occupied bandwidth for each NR carrier shall be less than the SAN channel bandwidth. |
| OTA relative ACLR (Adjacent Channel Leakage Ratio) | FFS pending on co-existecne study conclusion |
| OTA absolute ACLR | align the requirement with SAN type 1-O no requirement to be defined. |
| OTA OBUE (Operating band unwanted emissions) | reuse the approach from SAN type 1-O and define the requirement based on related ITU recommendation SM.1541-6 as starting point. |
| OTA TX spurious emissions | General requirement: defined the requirement based on ITU-R SM.329. Protection of the SAN receiver: Align the approach with SAN type 1-O based on NTN deployment specific conditions, i.e. no requirement to be defined. |
| OTA TX intermodulation | Align the approach with SAN type 1-O based on NTN deployment specific conditions, i.e. no requirement to be defined. |

Issue 2-3-2: SAN OTA Rx Requirements

- Agreement:

| OTA Rx requirement | Applicability notes |
| --- | --- |
| OTA sensitivity | Align the approach with BS type 2-O, i.e. no OTA sensitivity requirement for FR2, the OTA sensitivity is the same as the OTA reference sensitivity (see next proposal). |
| OTA Reference sensitivity level | While the overall requirement framework can be reused from the BS type 2-O in TS 38.104, the range of EIS values applicable to the NTN SAN deployment in FR2 requires further study. |
| OTA Dynamic range | FFS pending further evaluation |
| OTA ACS | FFS pending on co-existence study conclusion |
| OTA In-band blocking | Align the approach with the SAN type 1-O, i.e. no requirement to be defined. |
| OTA Out of band blocking | FFS |
| OTA Receiver spurious emission | Align the approach with the SAN type 1-O, i.e. no requirement to be defined. |
| OTA Receiver intermodulation | Align the approach with the SAN type 1-O, i.e. no requirement to be defined. |
| OTA In-channel selectivity | FFS |
| co-location requirements | to mirror the agreement from FR1, there are no co-location requirements for SAN type 2-O either. |