3GPP TSG-RAN WG4 Meeting#106      	R4-2303655

Athens Greece, 27 Feb – 3 Mar, 2023

Title:	WF for UE RF requirements for 3 MHz

Agenda item:	9.14.3

Source:	Nokia

Document for:	Approval

# 1	Background

This contribution is a WF for UE RF requirements for 3 MHz.

# 2 	Discussion

Table 1: WF UE RF Requirements for 3 MHz

| NR UE Tx/Rx requirement | Proposal for NR 3 MHz |
| --- | --- |
| 6.2.1 UE maximum output power | No specification impact |
| 6.2.2 MPR | NR MPR is channel bandwidth agnostic --> no specification impact |
| 6.2.3 A-MPR | Has specification impact to some bands. Considering bands in this WI n8, n26 and n28 needs A-MPR study. - NS_01: 3 MHz shall be added to the list of considered channel BW for the NS. - NS_12: A-MPR shall be specified for 3 MHz channel BW, it might be based on LTE limits (table 6.2.4-6 in TS 36.101). - NS_43 and NS_43U: RB restrictions (specified for LTE for 5 and 10 MHz) was removed and replaced with A-MPR values. Further discussion might be needed when adding 3 MHz channel BW for NR. - NS_13, NS_14, NS_17 and NS_18 didn’t have any A-MPR specified for 3 MHz channel BW. - NS_100 is not dependent on the considered channel BW. |
| 6.2.4 Configured transmitted power | No specification impact |
| 6.3.1 Minimum output power | Is channel bandwidth dependent, specification change needed but re-using -40 dBm can be considered. |
| 6.3.2 Transmit OFF power | Value - 50 dBm is channel bandwidth agnostic --> 3 MHz / 15 kHz SCS shall still be added in table 6.3.2-1. |
| 6.3.3 Transmit ON/OFF time mask | No specification impact |
| 6.3.4 Power control | No specification impact |
| 6.4.1 Frequency error | No specification impact |
| 6.4.2.1 EVM | No specification impact |
| 6.4.2.2 Carrier leakage | No specification impact |
| 6.4.2.3 In-band emissions | No specification impact |
| 6.4.2.4 EVM equalizer spectrum flatness | No specification impact |
| 6.5.1 Occupied bandwidth | Specification impact, addition of 3 MHz into Table 6.5.1-1 |
| 6.5.2.2 Spectrum emission mask | Specification impact, Dedicated general SEM for 3 MHz needed. |
| 6.5.2.3 Additional Spectrum emission mask | Not needed for bands n8, n26, n28 or n100 --> no specification impact |
| 6.5.2.4.1 NR ACLR | Specification impact, addition of 3 MHz into Table 6.5.2.4.1-1 |
| 6.5.2.4.2 UTRA ACLR | Channel bandwidth agnostic for bands which is applicable --> no specification impact |
| 6.5.3.1 General spurious emissions | Boundary between E-UTRA out of band and spurious emission domain is defined generally --> no specification impact |
| 6.5.3.2 Spurious UEtoUE co-ex | Option 2: To be further studied, checking if any RB restrictions was specified for 3 MHz channel BW for the considered LTE re-farmed bands. |
| 6.5.3.3 Additional spurious emissions | n8, n26, n28 has additional spurious emissions, 3 MHz need to be listed --> specification impact |
| 6.5.4 Transmit intermodulation | Channel bandwidth agnostic --> no specification impact |
| 7.3.2 Reference sensitivity | Channel bandwidth dependent --> specification impact Option 1: LTE 3 MHz REFSENS can be reused (Nokia, Ericsson) |
| 7.4 Max input level | Specification impact, addition of 3 MHz into Table 7.4-1 Option 1: -25/-27dBm for n8, n26, n28 and n100 (Apple, Ericsson) |
| 7.5 Adjacent channel selectivity | Specification impact --> 3 MHz is added into Table 7.5-1 Option 1: 33 dBc can be reused (Apple, Nokia) Option 3: Define 3 MHz ACS and IBB blocking signals similar to LTE (Ericsson, Qualcomm) |
| 7.6.2 In-band blocking | Specification impact --> 3 MHz is added into Table 7.6.2-1 Option 2: Define 3 MHz ACS and IBB blocking signals similar to LTE (Ericsson, Qualcomm) |
| 7.6.3 Out of band blocking | Specification impact --> 3 MHz is added into Table 7.6.3-1 Option 2: Define out of band blocking requirement similar to LTE (Ericsson) |
| 7.6.4 Narrow band blocking | Specification impact --> 3 MHz is added into Table 7.6.4-1 Option 2: Define narrow band blocking requirement similar to LTE (Ericsson) |
| 7.7 Spurious response | Specification impact --> 3 MHz is added into Table 7.7-1 Option 2: Define spurious response requirement similar to LTE (Ericsson) |
| 7.8 Intermodulation characteristics | Specification impact --> 3 MHz is added into Table 7.8.2-1 Option 2: considering REFSENS +8dB for Pw, 3 MHz interferer 2 and interferer offset 1 at ±BW/2 ± 4.5 MHz, similar to LTE (Ericsson) |
| 7.9 Spurious emissions | Channel bandwidth agnostic --> no specification impact |

# 3	Conclusion

Table 1 in this contribution collects UE RF requirement agreements relating to into introduction of 3 MHz channel bandwidth.