3GPP TSG-RAN WG4 Meeting #106bis-e                                                        R4-2305200

Online, April 17th – 26th, 2023

Source:	Samsung

Title:	Discussion on non-collocated FR1 inter-band EN-DC with overlapping DL bands and FR1 intra-band NR-CA deployment

Agenda item:			5.12.3

Document for:	Discussion

# 1. Introduction

In RAN#95e meeting, a new WI is approved which focused on non-collocated FR1 inter-band EN-DC with overlapping DL bands and FR1 intra-band NR-CA deployment scenarios, while further revised in RAN#99 as [1]. As agreed in the work plan [2], the RRM part of discussion, particularly on MRTD and MTTD requirement shall be initiated from RAN4#105, i.e., this meeting.

As required in WID, the detailed objectives on RRM relevant topics are provided in WID [1] as follows:

- Phase I: - Study the feasibility to support non-co-located scenarios for both FR1 inter-band EN-DC with overlapping and FR1 intra-band NR-CA, except for 2-layer case of EN-DC with supporting the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 already specified in Rel-16 and Rel-17. - Investigate the tolerable power imbalance between carriers - Investigate the required arrival time difference between CCs - Investigate the additional impacts of contiguous case, if time units are available. - Evaluate the UE performance under the power imbalance and arrival time difference - Discuss and decide reference UE architecture considering the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 for 2-layer MIMO case for NR-CA, and 4-layer MIMO case for both EN-DC/NR-CA - NOTE 1: For UE capable of supporting 2-layer MIMO, only assuming power imbalance 25dB - NOTE 2: For UE capable of supporting 4-layer MIMO, 2-layer MIMO power imbalance assumption can be considered as a base line but another value or the same power imbalance with different throughput performance requirement is not precluded. - NOTE 3: RAN4 is recommended to start the work on 2-layer first and after that start 4-layer work based on the conclusion of 2-layer work. - Work is limited to CA/EN-DC for EN-DC/NR-CA for bands 42, n77/n78 - Investigate whether the power imbalance should be explicitly (e.g. as an RF requirement) or implicitly specified (e.g. through a demodulation performance test). Specify the power imbalance based on the outcome of the investigation. - If any change in RAN1 or RAN2 spec is needed, it will be triggered by RAN4 LS - Phase II: - Phase II work will get started after the feasibility in phase I is confirmed - Specify MRTD and MTTD requirements and other identified RRM requirement impact(s) (if any) in non-collocated deployment - Discuss and decide if the different requirements will be specified based on UE capability of interBandMRDC-WIthOverlapDL-Bands-r16.

From RAN4#104-e meeting (Aug., 2022), RF aspects on intra-band non-collocated EN-DC/NR-CA have been discussed for several meetings, with five WFs approved [3][4][5][6][9]. Furthermore, the RRM aspects have been discussed in RAN4#105 and RAN4#106, with WF approved [7][10]. Based on our analysis in last two meetings [8][11], in this contribution, we would like to further share our analysis and viewpoints on RRM aspects for intra-band non-collocated EN-DC/NR-CA.

# 2. MRTD/MTTD for Rel-18 Intra-band NR-CA Type-2 UE

For Type-2 NR-CA UE (i.e., supporting intra-band non-contiguous NR-CA with 2-layer MIMO per CC), based on the following the RAN4 agreement achieved in RAN4#105 [7], the requirement shall be specified:

Issue 1-2-1: MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE - Agreements - MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE - MRTD=33us - MTTD=34.6us

In the last meeting, we have proposed the text proposal to implement the above agreements. However, the revised dCR R4-2303307 has not been endorsed, and based our understanding it is because the following questions to be further studied:

- FFS whether/how the applicable MRTD/MTTD requirement should be related to the non-collocated deployment.

- FFS how to allow ”non-collocated deployment for intra-band non-contigous NR-CA” in the specification.

For the 1st question, we agree that the applicable MRTD/MTTD requirement shall not be specified based on certain deployment condition, e.g., co-located/non-collocated deployment, because this informatin is unknown to UE and not necessarily needs to be known by UE.

Proposal 1: For Type-2 UE (for both FR1 inter-band EN-DC with overlapping DL bands and FR1 intra-band non-contiguous NR-CA), the applicable MRTD/MTTD requirement shall not be specified based on certain deployment condition, e.g., co-located/non-collocated deployment.

Secondly, the restriction of ”collocated deployment for intra-band CA” shall be revised, since the non-collocated deployment shall be allowed for non-contiguous CA due to the Type-2 agreed to be introduced. Therefore, the following proposal is provided:

Proposal 2: The restriction of co-located deployment for intra-band CA shall be revsied to be applicable to only intra-band contiguous CA.

Accordingly, the following text proposals are provided for MRTD/MTTD requirement for FR1 intra-band non-contiguous Type 2 UE which support non-collocated deployment.

Proposal 3: The following text proposals for MTTD/MRTD are captured in Rel-18 TS38.133 for FR1 intra-band non-contiguous NR-CA Type 2 UE, which support non-collocated deployment.

### 7.5.4 Minimum Requirements for NR Carrier Aggregation The UE shall be capable of handling at least a relative transmission timing difference between slot timing of all pairs of TAGs in FR1 and/or FR2-1 as shown in Table 7.5.4-1, provided that the UE is: - configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA or NR-DC mode, or - configured with more than one sTAG for inter-band NR carrier aggregation in EN-DC or NE-DC mode. The UE shall be capable of handling at least a relative transmission timing difference between subframe timing of all pairs of TAGs between FR1 and FR2-2 as shown in Table 7.5.4-1, provided that the UE is: - configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA or NR-DC mode. For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative transmission timing difference 34.6µs between slot timing of all pairs of TAGs provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18]. Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of TAGs; Maximum uplink transmission timing difference (µs). FR1; 34.6. FR2-1; 8.5 Note1. Between FR1 and FR2-1; 26.1. Between FR1 and FR2-2; 26.1. Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA.. <unchanged content omitted> ### 7.6.4 Minimum Requirements for NR Carrier Aggregation For intra-band contiguous CA, only co-located deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE not capable of [intraBandNRCA-NonCollocated-r18] shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-1 below. For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference 33µs between slot timing of different carriers to be aggregated at the UE receiver provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18]. Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation Frequency Range; Maximum receive timing difference (µs). FR1; 31. FR2-1; 0.26. Note 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.. For inter-band NR carrier aggregation, - the UE shall be capable of handling at least a relative receive timing difference between slot timing of all pairs of carriers in FR1 and FR2-1 to be aggregated at the UE receiver as shown in Table 7.6.4-2 below. - the UE shall be capable of handling at least a relative receive timing difference between subframe timing of all pairs of carriers in FR1 and FR2-2 to be aggregated at the UE receiver as shown in Table 7.6.4-2 below. Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8 note1. Between FR1 and FR2-1; 25. Between FR1 and FR2-2; 25. Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA..

# 3. Interruption for Rel-18 Intra-band NR-CA Type-2 UE

For the interruption requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE, in the last RAN4 meeting, it is agreed to reuse the legacy FR1 inter-band CA interruption requirements for SCell addition/release/ activation/deactivation, as follows [10].

Issue 1-2-1: Interruption requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE - Agreements - Reuse legacy FR1 inter-band CA interruptions requirements for SCell addition/release/ activation/deactivation for FR1 intra-band non-contiguous CA for Type 2 UE. - Note: the same interruption requirements are applicable to both colocated and non-colocated deployments

One point needs the additional explanation is that the above inter-band CA interruption requirement shall only be when applicable for Type-2 UE when the active serving cell and the SCell being added/released/activated/deactivated are belongs to different sub-blocks in FR1 intra-band non-contiguous NR CA.

Proposal 4: Reuse legacy FR1 inter-band CA interruptions requirements for SCell addition/release/ activation/deactivation for FR1 intra-band non-contiguous CA for Type 2 UE, only when:

- The active serving cell and the SCell being added/released/activated/deactivated are belongs to different sub-blocks in FR1 intra-band non-contiguous NR CA.

Based on the above agreements and proposal 3, it is proposed that the following applicability rule for the relevant clauses:

Proposal 5: The following applicability rule is provided for interruption requirements for SCell addition/release/activation/deactivation for FR1 intra-band non-contiguous NR-CA Type 2 UE:

-  For the active serving cell and the SCell being added or released are belongs to different sub-blocks in FR1 intra-band non-contiguous NR CA, the inter-band CA interruption requirement shall be applicable provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18].

- For the active serving cell and the SCell being activated or deactivated are belongs to different sub-blocks in FR1 intra-band non-contiguous NR CA, the inter-band CA interruption requirement shall be applicable provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18].

Furthermore, for the interruption requirement during measurements on deactivated SCC, longer interruption length is expected for the case where the PCell or activated SCell(s) is in the same band as the deactivated SCell. We propose that the same principle should be followed for Type-2 UE, i.e., reuse legacy FR1 inter-band CA interruptions requirements for the interruption requirement during measurements on deactivated SCC.

Proposal 6: Reuse legacy FR1 inter-band CA interruptions requirements for the measurements on deactivated SCC, for FR1 intra-band non-contiguous CA for Type 2 UE:

- when the active serving cell and the deactivated SCell being measured are belongs to different sub-blocks in FR1 intra-band non-contiguous NR CA

- Note: the same interruption requirements are applicable to both colocated and non-colocated deployments.

# 4. Scheduling restriction for Rel-18 Intra-band NR-CA Type-2 UE

During last meeting, the following agreement on the scheduling restriction requirement is provided:

Issue 1-4-1: Impacts on scheduling availability requirements - Agreements - For Type 2 UE with FR1 intra-band CA, there are no scheduling restrictions on carriers in the same band.

Based on the above agreement, the following applicable rule is proposed for all relevant scheduling restriction clauses.

Proposal 7: The following applicability rule for scheduling is provided for FR1 intra-band non-contiguous NR-CA Type 2 UE:

### 3.6.xx Applicability of requirements for FR1 intra-band non-contiguous NR CA When FR1 intra-band non-contiguous carrier aggregation is configured, the scheduling restrictions in clause 8.1.7.2, 8.5.7.2, 8.5.8.2, 9.2.5.3.1, 9.2.5.3.2, 9.3.9.3.1, 9.3.9.3.2, 9.3.10.3.1, 9.3.10.3.2, 9.5.6.2, 9.7.4.1, 9.8.6.2, 9.10.2.6, 9.13.6.2 are not applicable on FR1 serving cell(s) if the following condition is met: - FR1 serving cell(s) are belongs to different sub-blocks in FR1 intra-band non-contiguous NR CA from the cell in which the concerned operation causing the scheduling restriction is performed, and - UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18].

# 5. Rel-18 Non-collocated EN-DC/NR-CA for New Type UE for 4-Layer MIMO case

In RAN4#105 RF session, it is agreed to discuss both RRM requirement (MRTD and TAE) and RF requirement (mainly power imbalance) in joint session in RAN4#106 and treat them in one email thread for e-meeting [6], due to the mutual impact among them. And below table is agreed for the architecture reference.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | NRCA/ ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | NRCA ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |
| [3a] | 1 | 4 shared |  | 4 | 4 | 4Rx | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding 2LO/BB/Rx and RF split after 2 LNAs out of 4 => common AGC on LNA => 25dB partial range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |
| [3b] | 1 | 4 shared |  | 4 | 4 | 4Rx | NRCA ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding 2LO/BB/Rx and RF split after all 4 LNAs => common AGC on LNA => 25dB partial range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |
| [4a] | 1 | 4 | 6 total | 4 | 4 | 4Rx | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |
| [4b] | 1 | 4 | 8 total | 4 | 4 | 4Rx | NRCA ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |

Furthermore, as we illustrated in our paper for RF analysis, for Type-3b architecture which could be applicable for both EN-DC and NR-CA deployment, in terms of 2-CC scenario, CC1 and CC2 have to share the RF LNA (multiple gain mode) which may call for more RF requirement relaxation, especially when one CC is close to maximum input level, the common RF LNA has to be set to full attenuation mode which may lead to the other CC cannot be well amplified and the signal is out of ADC saturation range, correspondingly more Rx requirement relaxation can be discussed in case power imbalance is defined as 25dB. Similarly for Type-3a UE.

Figure 1: Illustration for Type-3b UE architecture

From RRM perspective, MRTD shall include the TAE at BS TX and the required propagation delay difference at UE receiver. We acknowledge that MRTD＜CP could be a strong condition which make the common LNA gain switching be accommodated in the CP duration while no signal distortion on the duration of useful OFDM symbol. However, MRTD＜CP is too restrictive for non-collocated deployment. By assuming 3us TAE, it is already larger than CP when SCS is 30kHz or 60kHz, as below Table 2 shown. As explained in our RF paper, we think the Rx requirement relaxation can compensate the signal distortion if it could be assessed, and the assessment need to take many factors into account such as how many gain modes (AGC steps) are assumed, how much dB above REFSENS the first common RF AGC attenuation step is activated in current operation, the SNR loss, etc.

Table 2: SCS and corresponding CP length

| Parameter/Numerology | 0 | 1 | 2 |
| --- | --- | --- | --- |
| SCS (kHz) | 15 | 30 | 60 |
| CP length (us) | 4.69 | 2.34 | 1.17 |

Observation 1: TAE is assumed as 3us in TS38.133, which is already larger than CP for SCS=30 or 60 kHz

Observation 2: MRTD＞CP is the more practical assumption for Type-3 UE.

It should be noted that RF session is discussing how/whether Type 3 shall be introduced in Rel-18, and it is obvious that RRM session shall follow the conclusion from RF session, where the reference RF architecture is discussed and accordingly RRM requirement can be defined.

Proposal 8: Follow the conclusion from RF session on whether/how Type-3 UE is introduced in Rel-18.

# 6. Conclusion

In this contribution, we provided our viewpoints on the RRM aspects for this work item, accordingly the following observations and proposals are obtained:

For Rel-18 Intra-band Non-collocated NR-CA Type-2 UE:

Proposal 1: For Type-2 UE (for both FR1 inter-band EN-DC with overlapping DL bands and FR1 intra-band non-contiguous NR-CA), the applicable MRTD/MTTD requirement shall not be specified based on certain deployment condition, e.g., co-located/non-collocated deployment.

Proposal 2: The restriction of co-located deployment for intra-band CA shall be revsied to be applicable to only intra-band contiguous CA.

Proposal 3: The following text proposals for MTTD/MRTD are captured in Rel-18 TS38.133 for FR1 intra-band non-contiguous NR-CA Type 2 UE, which support non-collocated deployment.

Proposal 4: Reuse legacy FR1 inter-band CA interruptions requirements for SCell addition/release/ activation/deactivation for FR1 intra-band non-contiguous CA for Type 2 UE, only when:

- The active serving cell and the SCell being added/released/activated/deactivated are belongs to different sub-blocks in FR1 intra-band non-contiguous NR CA.

Proposal 5: The following applicability rule is provided for interruption requirements for SCell addition/release/activation/deactivation for FR1 intra-band non-contiguous NR-CA Type 2 UE:

-  For the active serving cell and the SCell being added or released are belongs to different sub-blocks in FR1 intra-band non-contiguous NR CA, the inter-band CA interruption requirement shall be applicable provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18].

- For the active serving cell and the SCell being activated or deactivated are belongs to different sub-blocks in FR1 intra-band non-contiguous NR CA, the inter-band CA interruption requirement shall be applicable provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18].

Proposal 6: Reuse legacy FR1 inter-band CA interruptions requirements for the measurements on deactivated SCC, for FR1 intra-band non-contiguous CA for Type 2 UE:

- when the active serving cell and the deactivated SCell being measured are belongs to different sub-blocks in FR1 intra-band non-contiguous NR CA

- Note: the same interruption requirements are applicable to both colocated and non-colocated deployments.

Proposal 7: The following applicability rule for scheduling is provided for FR1 intra-band non-contiguous NR-CA Type 2 UE:

### 3.6.xx Applicability of requirements for FR1 intra-band non-contiguous NR CA When FR1 intra-band non-contiguous carrier aggregation is configured, the scheduling restrictions in clause 8.1.7.2, 8.5.7.2, 8.5.8.2, 9.2.5.3.1, 9.2.5.3.2, 9.3.9.3.1, 9.3.9.3.2, 9.3.10.3.1, 9.3.10.3.2, 9.5.6.2, 9.7.4.1, 9.8.6.2, 9.10.2.6, 9.13.6.2 are not applicable on FR1 serving cell(s) if the following condition is met: - FR1 serving cell(s) are belongs to different sub-blocks in FR1 intra-band non-contiguous NR CA from the cell in which the concerned operation causing the scheduling restriction is performed, and - UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18].

For Rel-18 Non-collocated EN-DC/NR-CA for New Type UE for 4-Layer MIMO case:

Observation 1: TAE is assumed as 3us in TS38.133, which is already larger than CP for SCS=30 or 60 kHz

Observation 2: MRTD＞CP is the more practical assumption for Type-3 UE.

Proposal 8: Follow the conclusion from RF session on whether/how Type-3 UE is introduced in Rel-18.

# 7. Reference

[1] RP-230742 (revision of RP-221809 and RP-222309), “New WID: Support of intra-band non-collocated EN-DC/NR-CA deployment”, KDDI, RAN#97-e.

[2] R4-2211795, “RAN4 RF/RRM work plan for NonCol_intraB_ENDC_NR_CA WI”, KDDI, RAN4#104-e.

[3] R4-2214458, “WF on NonCol_intraB_ENDC_NR_CA”, KDDI, RAN4#104-e.

[4] R4-2217733, “WF on NonCol_intraB_ENDC_NR_CA for NR-CA Type-2 UE”, KDDI, RAN4#104-bis-e.

[5] R4-2217734, “WF on NonCol_intraB_ENDC_NR_CA for NR-CA and EN-DC New Type UE”, KDDI, RAN4#104-bis-e.

[6] R4-2220537, “WF on the requirement for intra-band non-collocated CA/EN-DC”, KDDI, RAN4#105.

[7] R4-2220398, “WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA”, Huawei, HiSilicon, Samsung, RAN4#105.

[8] R4-2219286, “Discussion on RRM requirements for intra-band non-collocated EN-DC/NR-CA deployment”, Samsung, RAN4#105.

[9] R4-2303696, “WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment”, KDDI, RAN4#106.

[10] R4-2303225, “WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA”, Huawei, HiSilicon, RAN4#106.

[11] R4-2300448, “Discussion on non-collocated FR1 inter-band EN-DC with overlapping DL bands and FR1 intra-band NR-CA deployment”, Samsung, RAN4#106.