3GPP TSG-RAN WG4 Meeting #107	R4-2307031

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Supporting simulations for 4Tx Demodulation

Agenda item:	8.5.3.2

Document for:	Discussion

Introduction

In RAN #96, a new RAN4 WI “NR_ENDC_RF_FR1_enh2” was approved [1] with the following objectives defined regarding the performance aspects of Rel-18 FR1:

The objectives of performance part for Rel-18 RF FR1 requirement focus evolution include: - Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices - Specify the BS demodulation performance requirements to support UL 4-layer MIMO UE operation - Enable 8Rx for CPE/FWA/vehicle/industrial devices - Specify RLM test cases to support 8Rx - Investigate if the existing 4Rx RLM test can be reused or the new test will be specified - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers - Specify the SDR requirements with 8 MIMO layers - Specify release independence requirements in TS 38.307 if needed.

Hence, given this plenary agreement, it is needed that the performance requirements of the BS demodulator are agreed, in order to define the minimum performance requirements of a 5G NR BS, for the demodulation of PUSCH when utilizing 4Tx in FR1.

The objectives of core part for Rel-18 RF FR1 requirement focus evolution include: Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices [RAN4] - Investigate framework and architecture Example bands: - TDD bands: n41, n77/n78 - FDD bands: n1 - Note 1: the total number of example bands should be limited to 3. n77/n78 are considered as one band during the study. - Note 2: other bands to be introduced in the release independent way later on from Rel-18 - Note 3: specifying requirements for TDD bands has first priority - Specify the UE RF requirements to support 4Tx - First priority: 4x4 UL MIMO - Second priority: investigate and if necessary specify TxD requirement to support the same power class in UL MIMO and single antenna port - PA configuration assumption: - First priority: 4x23dBm - Second priority: 2x23dBm + 2x26dBm, 4x26dBm - UE power class - First priority: PC 1.5 - Second priority: PC2/PC3, and/or new power class if needed - Note 1: PC1.5 is only applicable for TDD bands - Note: detailed combinations for 2nd priority PA/UE power class assumptions are to be revisited in RAN#97

In this contribution we provide simulation results to support our companion discussion Tdoc [3], based upon the parameters defined in

Discussion

This paper presents the results of simulations which have been conducted by Nokia based on the parameters defined for 4Tx Simulations at RAN4 #106-bis-e and distributed in the WF for 4Tx [2]. These parameters can be seen summarised in Table 1.

Table 1 : Extended Simulation Parameters for 4Tx following RAN4# 106

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |  |
|  | DM-RS port | {0, 1, 2, 3} |  |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |  |
| Time domain | PUSCH mapping type | A and B |  |
| resource | Start symbol | 0 |  |
| assignment | Allocation length | 14 |  |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |  |
| assignment | Frequency hopping | Disabled |  |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| Antenna Configuration |  | 4T4R and 4T8R |  |
| Bandwidth and SCS |  | BW | SCS |
|  |  | 5 MHz | 15 kHz |
|  |  | 20 MHz (FFS) |  |
|  |  | 50 MHz |  |
|  |  | 10 MHz | 30 kHz |
|  |  | 40 MHz (FFS) |  |
|  |  | 100 MHz |  |
| Propagation Conditions and MCS |  | Propagation | MCS (Table 1) |
|  |  | TDLA30-10 | 17 |
|  |  | TDLB 100-400 | 2 |
|  |  | TDLC 300-100 | 12 |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |  |

All the values which will be presented in the following tables represent the SNR value in dB at which 70% of the maximum throughput could be reached.

Results will be presented broken down by antenna configuration.

Results for 4T4R Antenna Configuration

Table 2 presents the results for 4T4R, the results are presented for both PUSCH mapping types, all propagation conditions and all results are using TPMI 0 and ‘low’ correlation.

Table 2 :4Tx Results for 4T4R (Rank 4)

| Parameter |  |  |  | SNR Value (@ 70% TPUT) ideal |  |
| --- | --- | --- | --- | --- | --- |
| SCS | BW | MCS | Propagation | PUSCH Mapping Type A | PUSCH Mapping Type B |
|  |  | 2 | TDLB100-400 | -1.23 | -1.16 |
|  | 5MHz | 12 | TDLC300-100 | 11.88 | 11.98 |
|  |  | 17 | TDLA30-10 | 16.23 | 16.22 |
|  |  | 2 | TDLB100-400 | -0.94 | -0.88 |
| 15kHz | 20MHz | 12 | TDLC300-100 | 12.28 | 12.26 |
|  |  | 17 | TDLA30-10 | 17.29 | 17.26 |
|  |  | 2 | TDLB100-400 | -0.87 | -0.73 |
|  | 50MHz | 12 | TDLC300-100 | 12.36 | 12.34 |
|  |  | 17 | TDLA30-10 | 18.48 | 18.50 |
|  |  | 2 | TDLB100-400 | -1.42 | -1.39 |
|  | 10MHz | 12 | TDLC300-100 | 12.34 | 12.48 |
|  |  | 17 | TDLA30-10 | 16.44 | 16.43 |
|  |  | 2 | TDLB100-400 | -1.13 | -1.17 |
| 30kHz | 40MHz | 12 | TDLC300-100 | 12.82 | 12.87 |
|  |  | 17 | TDLA30-10 | 17.11 | 17.10 |
|  |  | 2 | TDLB100-400 | -0.99 | -0.99 |
|  | 100MHz | 12 | TDLC300-100 | 13.36 | 13.42 |
|  |  | 17 | TDLA30-10 | 18.11 | 18.19 |

Results for 4T8R Antenna Configuration

Table 3 presents the results for 4T8R, the results are presented for both PUSCH mapping types, all propagation conditions and all results are using TPMI 0 and ‘low’ correlation.

Table 3 : 4Tx Results for 4T8R (Rank 4)

| Parameter |  |  |  | SNR Value (@ 70% TPUT) ideal |  |
| --- | --- | --- | --- | --- | --- |
| SCS | BW | MCS | Propagation | PUSCH Mapping Type A | PUSCH Mapping Type B |
|  |  | 2 | TDBLB100-400 | -4.55 | -4.47 |
|  | 5MHz | 12 | TDLC300-100 | 5.12 | 5.18 |
|  |  | 17 | TDLA30-10 | 9.01 | 9.03 |
|  |  | 2 | TDBLB100-400 | -4.20 | -4.14 |
| 15kHz | 20MHz | 12 | TDLC300-100 | 5.42 | 5.43 |
|  |  | 17 | TDLA30-10 | 9.42 | 9.43 |
|  |  | 2 | TDBLB100-400 | -4.16 | -4.05 |
|  | 50MHz | 12 | TDLC300-100 | 5.39 | 5.40 |
|  |  | 17 | TDLA30-10 | 9.83 | 9.90 |
|  |  | 2 | TDBLB100-400 | -4.63 | -4.55 |
|  | 10MHz | 12 | TDLC300-100 | 5.62 | 5.74 |
|  |  | 17 | TDLA30-10 | 9.18 | 9.23 |
|  |  | 2 | TDBLB100-400 | -4.22 | -4.20 |
| 30kHz | 40MHz | 12 | TDLC300-100 | 5.95 | 6.06 |
|  |  | 17 | TDLA30-10 | 9.30 | 9.33 |
|  |  | 2 | TDBLB100-400 | -4.10 | -4.04 |
|  | 100MHz | 12 | TDLC300-100 | 6.28 | 6.38 |
|  |  | 17 | TDLA30-10 | 9.83 | 9.91 |

Conclusion

In this contribution, simulation results were provided on the 4Tx parameters as agreed in R4-2305890 [2].

These results are discussed, and a series of observations and proposals made in [3].

- References

- RP-221496 Revised WID Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)

- R4-2305890 WF on FR1 4Tx demodulation requirement, Ericsson, RAN4 #106bis-e. Electronic Meeting, April 17 – 26, 2023

- R4-2307030, Discussion on 4Tx BS Demodulation, RAN4 #107, Incheon, Korea, May 22 – 26, 2023.