3GPP TSG-RAN WG4 Meeting #107 	R4-2307033

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on UE Demodulation for non-collocated FR1 intra-band EN-DC/NR-CA

Agenda item:	8.12.4

Document for:	Discussion

Introduction

At RAN#95 meeting, the WI was approved to study the feasibility to support non-co-located scenario for FR1 intra-band non-contiguous EN-DC/NR-CA [1] with the following objectives of the performance part.

- Phase II: - Phase II work will get started after the feasibility in phase I is confirmed - Specify PDSCH demodulation requirements for non-collocated scenarios for intra-band non-contiguous EN-DC and NR-CA - Define PDSCH demodulation performance requirement based on the applicable MRTD and power imbalance values. - Power imbalance between the carriers is limited

In this paper, we present Nokia’s view on the impact of non-collocated FR1 intra-band EN-DC/NR-CA demodulation requirements.

Discussion

At RAN4#106-bis-e several open issues were presented in the WF [2], as follows:

Issue 1-2-1: UE type assumption Agreement: - Discuss the UE demodulation requirements for Type 2 UE first. - Test scope covers both EN-DC and NR-CA - Not define the UE demodulation requirements for Type 3 UE. - For UE demodulation requirements for Type 4 UE, depending on the progress of RF discussion. Issue 1-2-2: Test scope Whether to define the corresponding UE demodulation requirements for Type 2 UE Way forward: - Option 1: Define new UE demodulation requirements for Type 2 UE for intra-band non-contiguous EN-DC/NR-CA deployment scenario. - Interested companies assume the maximum received time difference value (33us) and maximum power imbalance assumption (25dB). - Option 2: Not define new UE demodulation requirements for Type 2 UE for intra-band non-contiguous EN-DC/NR-CA deployment scenario.

UE Type Assumption

Type 2 UE in comparison

It is our understanding that from a baseband perspective, type-2 and type-3 differ in the number of BB/Rx chains available for each cell. In particular, type-2 has a BB/Rx chain per CC, while type-3 shares one [3].

| UE Type | CC# | antenna / LNA | Mixer | Analog BB | #Rx | NRCA / ENDC | power imbalance | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 (NR) | 4 shared | 4 shared | 4 shared | 4Rx | NR-CA EN-DC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 (LTE/NR) |  |  |  | 4Rx |  |  |  |
| 2 | 1 (NR) | 2 | 2 | 2 | 2Rx | NR-CA EN-DC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 (LTE/NR) | 2 | 2 | 2 | 2Rx |  |  |  |
| 3a | 1 (NR) | 4 shared | 4 | 4 | 4Rx | EN-DC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding 2LO/BB/Rx and RF split after 2 LNAs out of 4 => common AGC on LNA => 25dB partial range |
|  | 2 (LTE) |  | 2 | 2 | 2Rx |  |  |  |
| 3b | 1 (NR) | 4 shared | 4 | 4 | 4Rx | NR-CA EN-DC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding 2LO/BB/Rx and RF split after all 4 LNAs => common AGC on LNA => 25dB partial range |
|  | 2 (LTE/NR) |  | 4 | 4 | 4Rx |  |  |  |
| 4a | 1 (NR) | 4 | 4 | 4 | 4Rx | EN-DC | 25dB full range | Requires 6 antennas and LNA => FWA only |
|  | 2 (LTE) | 2 | 2 | 2 | 2Rx |  |  |  |
| 4b | 1 (NR) | 4 | 4 | 4 | 4Rx | NR-DC EN-DC | 25dB full range | Requires 8 antennas and LNA => FWA only |
|  | 2 (LTE/NR) | 4 | 4 | 4 | 4Rx |  |  |  |

Figure 1: UE architecture comparison from [R4-2305181]

Figure 2: UE architecture comparison from [R4-2215736]

However, as per the agreement at RAN4#106-bis-e [3], RF support for type-3 UEs have been postponed for future releases. In our understanding this is in part this is due to their inability to compensate for receive time differences (RTDs) and power imbalances, this is discussed further in our contribution for RAN4#106-bis-e [4].
Additionally also RF support for type-4 UEs has been postpones.

Hence type 2 UEs will be the focus of Rel-18, which are constrained to 2Rx per band and have 2 mixers/LOs.

Type 3 and 4 UEs are postponed in RF. Type 2 UEs, which use 2 Rx per band/LNA and have 2 mixers/LOs, are focus of Rel-18.

Type-4 UEs are very similar to type-2, just with the full RX path (including e.g., LNA). Due to the similar architecture that RTD, etc., configurations from type-2 can be re-used in type-4 in the future. And even the power imbalance agreements can be re-used, as the imbalance stems more from geometry and less from LNA/AGC considerations.

Type 2 UEs in Demod

The main difference between type 1 and type 2 is, that in type 1 (legacy) all CCs are downmixed and sampled together, while in type 2 the CCs are downmixed sampled separated. As such only type 2 allows to frequency shift/compensate the waveform per CC and time shift/fft_window shift the samples per CC. The capability to time/frequency shift each CC separately is of high importance in NonCol scenarios, where the frequency shift and time delays differ significantly between the two CCs. 
Demodulation performance depends strongly on correct FOC (frequency offset compensation) and TOC (time offset compensation) of the waveform and samples. BLER for each CC increases quickly once residual FOE exceeds DM-RS density related BB FOC range, and once residual TOE exceeds CP. Type 1 UEs have larger residual FOE/TOE as only a mean value between the non-collocated CCs can be compensated.

However, the demodulator implementation has a large impact on the FO/TO residual in the first place (e.g., via FFT window placement choice) and the demod performance impact the FO/TO residual has (e.g., via DM-RS based FOC).

BLER for each CC increases quickly once residual FOE exceeds DM-RS density related BB FOC range, and once residual TOE exceeds CP. BLER is different for demodulator implementations and FO/TO difference scenarios between CCs.

Type 1 UEs are expected to not have feasible performance in many non-collocated environments, as the RRM session has already defined time differences of up to 33us (i.e., on a symbol order of magnitude). Type 2 UEs will, thus, most commonly be served in non-collocated environments.

Mostly type 2 (and type 4) UEs are expected to be served in non-collocated environments.

Power imbalance may also have impact on demodulation performance, as the sensitivity and, thus, SNR per CC differs due to LNA and gain control. However, the demodulation requirements are usually defined with a large AWGN_offset to isolate from such RF effects.
It should be studies, if power imbalance is an effect that we want to have minimum demod performance requirements for, and if yes, how to model it in the demod test setup, e.g., via interference profiles.

All the above leads us to the following proposal:

We propose for RAN4 to define UE demod requirements based on type-2 UEs.

Test Scope

As previously discussed, type-2 (and type-4) UEs can compensate for the expected and already agreed (in RRM) receive time differences (RTDs) and power imbalance, thanks to independent FFT windows and LNAs. (Type-3 UEs would have had more troubles dealing with the power imbalance due to a shared LNA.)

As such we propose to set performance requirements for type 2 UEs for intra-band non-contiguous EN-DC/NR-CA deployment scenarios, with future requirements to be defined for type 4 if desired.

In accordance with the WF and RRM, it is agreed that the maximum received time difference value (33us) and maximum power imbalance assumption (25dB).

Type-2 (and type-4) UEs are able to compensate for RTDs and power imbalance, with maximum values agreed at RAN4#106-bis-e.

RAN4 shall define PDSCH UE demodulation requirements for type-2 based upon maximum received time difference value (33us) and maximum power imbalance assumption (25dB).

Note that for both CC all reference signals are present as usual (SSB, CSI-RS, etc.)
As such the “only” challenges are to define a setup for demod testing both CCs at the same time with the RRM agreed RTD, and/or to agree on a residual TO/FO in such a deployment case.

Conclusion

Within this contribution we discuss the demodulation requirements for non-collocated FR1 intra-band EN-DC/NR-CA.

In the paper, the following Observations and Proposals were made:

Observation 1: Type 3 and 4 UEs are postponed in RF. Type 2 UEs, which use 2 Rx per band/LNA and have 2 mixers/LOs, are focus of Rel-18.

Observation 2: BLER for each CC increases quickly once residual FOE exceeds DM-RS density related BB FOC range, and once residual TOE exceeds CP. BLER is different for demodulator implementations and FO/TO difference scenarios between CCs.

Observation 3: Mostly type 2 (and type 4) UEs are expected to be served in non-collocated environments.

Proposal 1: We propose for RAN4 to define UE demod requirements based on type-2 UEs.

Observation 4: Type-2 (and type-4) UEs are able to compensate for RTDs and power imbalance, with maximum values agreed at RAN4#106-bis-e.

Proposal 2: RAN4 shall define PDSCH UE demodulation requirements for type-2 based upon maximum received time difference value (33us) and maximum power imbalance assumption (25dB).

- References

- RP-221496 “Revised WID Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”

- R4-2305891 “WF UE demod NonCol intraB”, Ericsson, RAN4#106-bis-e, Electronic Meeting, 17 April ‒ 26 April 2023

- R4-2306605 “WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment”, KDDI, RAN4#106-bis-e, Electronic Meeting, 17 April ‒ 26 April 2023

- R4-2215736 “Views on UE RF aspect for non-collocated EN-DC, NR-CA deployment”, Samsung, RAN4#104-bis-e, Electronic Meeting, 10th-19th, Oct, 2022

- R4-2304102, “Demodulation requirements for non-collocated FR1 intra-band EN-DC/NR-CA”, Nokia, Nokia Shanghai Bell, RAN4#106-bis-e, Electronic Meeting, 17 April ‒ 26 April 2023