3GPP TSG-RAN4 Meeting #107 	R4-2307042

# Incheon, South Korea, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1488 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR1 OOB requirements correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, NR_FR1_35MHz_45MHz_BW-Core |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is currently a difference in the way the requirements of FR1 OOB are defined in R15/R16 and R17/R18 38.101-1 specifcations. The transmission power is defined as “REFSENS + specific value in Table 7.6.3-1” in R15/R16, whereas it is defined using a formula in R17/R18: REFSENS + (9 + 10log10(BWChannel /20)) dB (with NOTE 2: 10log10(x) is rounded to the next higher 0.5dB value.), the values listed for R15/R16 seem based on “10log10(x) is rounded to the closest next higher 0.5dB value”. Taking as example the 90MHz and 100MHz channel bandwidths, R15/16 give requirements of 15.5dB for 90MHz and 16dB for 100MHz, whereas R17/18 give requirements of 16dB for both 90MHz and 100MHz (The next higher 0.5dB value for 15.53dB and 15.99dB being for both 16dB). |  |  |  |  |  |  |  |  |
| Summary of change: |  | A identical table style is used for R15/16 and R17/18 with consistent “transmission power” values for identical channel bandwidths. The use listed values also R17/18 makes it clearer what requirement applies, and what is the value that applies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of consistency with different requirements applying for identical channel bandwidths between R15/16 and R17/18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

### 7.6.3	Out-of-band blocking

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band. The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3-1 and Table 7.6.3-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.3-1: Out-of-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Channel bandwidth | Power in transmission bandwidth configuration [dBm] |
| --- | --- |
| 5 MHz | REFSENS + 6.0 dB |
| 10 MHz | REFSENS + 6.0 dB |
| 15 MHz | REFSENS + 7.0 dB |
| 20 MHz | REFSENS + 9.0 dB |
| 25 MHz | REFSENS + 10.0 dB |
| 30 MHz | REFSENS + 11.0 dB |
| 35 MHz | REFSENS + 11.5 dB |
| 40 MHz | REFSENS + 12.0 dB |
| 45 MHz | REFSENS + 12.5 dB |
| 50 MHz | REFSENS + 13.0 dB |
| 60 MHz | REFSENS + 14.0 dB |
| 70 MHz | REFSENS + 14.5 dB |
| 80 MHz | REFSENS + 15.0 dB |
| 90 MHz | REFSENS + 15.5 dB |
| 100 MHz | REFSENS + 16.0 dB |
| NOTE: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |

Table 7.6.3-2: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, | Pinterferer | dBm | -44 | -30 | -15 |
| n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n34, n38, n39, n40, n41, n485, n50, n51, n536, n54, n65, n66, n67, n70, n71, n74, n75, n76, n85, n91, n92, n93, n94, n100, n101, n105 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. For band 50, the FDL_low of band 51 is applied as FDL_low for band 50. NOTE 3: For band 76 the FDL_high of band 75 is applied as FDL_high for band 76. For band 75, the FDL_low of band 76 is applied as FDL_low for band 75. NOTE 4: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. NOTE 5: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz. NOTE 6: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2580 MHz and FInterferer < 2775 MHz. NOTE 7 For UE supporting both bands 25 and 70, the FDL_high of band 70 is applied as FDL_high for band 25, and the FDL_low of band 25 is applied as FDL_low for band 70. NOTE 8: For bands 91 and 93 the FDL_high of bands 92 and 94 are applied as FDL_high for bands 91 and 93. For bands 92 and 94, the FDL_low of bands 91 and 93 are applied as FDL_low for bands 92 and 94 NOTE 9: For SDL bands, requirements shall be applied only for CA band combination cases. NOTE 10 For a UE supporting CA_20A-28A and higher order band combinations in which CA_20A-28A is a subset, the requirements for Band n20 and Band n28 apply with FDL_low given by the lower limit of the restricted operating frequency range in Band n28 and FDL_high by Band n20. |  |  |  |  |  |

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6.3-2, a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz withthe number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

For NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range up to 3*BWChannel below or from 3*BWChannel above the UE receive band, where BWChannel is the channel bandwidth. The throughput of the wanted signal shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3-3 and Table 7.6.3-4. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6.3-3: Out-of-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB |
| NOTE: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |

Table 7.6.3-4: Out of-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low ≤ -3*BWChannel or 3*BWChannel ≤ f – FDL_high < 60 | -200 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MAX(200,3*BWChannel) or FDL_high + MAX(200,3*BWChannel) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel) or FDL_high + MAX(150,3*BWChannel) ≤ f ≤ 12750 |
| n104 (NOTE 5) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel) or FDL_high + MAX(150,3*BWChannel) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: BWChannel denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For BWChannel > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3*BWChannel from the band edge. For BWChannel larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For BWChannel ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. NOTE 5: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 5175 MHz. For BWChannel > 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. The power level of the interferer (PInterferer) for Range 2 shall be modified to -33 dBm for the range 5925 – MAX(60,3*CBW) ≤ f < FDL_low - MAX(60,3*CBW). |  |  |  |  |  |

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6.3-4, a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz withthe number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

<<End of change>>