3GPP TSG RAN WG4 #107														     R4-2307054

Incheon, May 22nd – 26th, 2023

Agenda Item:		8.30.2.1

Source:				InterDigital Communications

Title:						Configured Power and MPR derivation for STxMP mDCI case

Document for:		Approval

# Background

RAN plenary #94e approved the WID in [1] for Rel-18 MIMO enhancements. As described in WID, one of the goals in Objective 7 is to study and specify the operation of simultaneous UL transmission across multiple UE panels (STxMP). In this context, for the case of simultaneous UL transmissions, the operation is limited to the description of Objective 6 in WID.  RAN4 discussions led to an LS reply to RAN1 in [2] regarding UE power limitation for STxMP in FR2. In the last RAN4 e-meeting #107, a WF [4] was agreed outlining the way forward for the STxMP RF requirements assumptions.

We are listing below the content of the WF for convenience:

<Way forward> ## Configured power per panel (per TCI state): - Relaxation factor can be added based on the study outcome of the configured power and requirements for STxMP - RAN4 will further study how to improve the proposed per-TCI state configured power as proposed in RAN4#107, and if necessary while considering the following issues. Other solutions are not precluded > Whether/how to improve the per panel configured power to make it clearer for the two-panel transmission > Solution to differentiate the per-beam power for different TCI-state - It is expected that RAN4 waits for RAN1 updates regarding per-TCI power control before confirming the concept of the configured power for STxMP UE RF requirements for STxMP: - Max EIRP and Max TRP should be based on the legacy requirements - Clarification of EIRP for STxMP can be discussed if it is necessary to consider the sum of the EIRP of all respective beams in a certain direction based on the contribution to the next meeting Per-panel related: - RAN4 focuses on the new configured power for STxMP power control while considering the relevant requirements, e.g., Min peak EIRP (PPowerclass) and MPR (MPRf,c,k), and its testability issues raised in RAN4#106bis-e > Legacy requirements can be starting point > Further discussions are required for how to address the testability issue, e.g., relaxation factor and TE enhancements - In addition to the requirement needed for the output power configuration, other requirements, e.g., spherical coverage and beam correspondence, can be discussed when the requirements of peak EIRP and MPR per panel are clear enough in RAN4 MPE considerations - RAN4 needs further study of the MPE scenario for the CPE/FWA/vehicle/industrial devices (if applicable) with its use case of STxMP

# Discussion

In this contribution, we share our analysis on Maximum Configured Power, the TCI approach for Pcmax definition, and how to handle MPR, A-MPR for mDCI STxMP case.

# Maximum Configured power for STxMP

In our understanding, the proposed form-factors may have specific antenna and RF frontend capabilities that may need to be known by the network. However, in RAN4 we agreed to make the requirements as transparent as possible and have wide options for UE implementation. Thus, a good approach is to have Maximum Configured Power defined in relation with the TCIs indicated for UL transmissions for STxMP as this approach also connects the reference pathloss measurements with the correct RS measurements as indicated in the active TCIs.

For the sake of discussion, we are reproducing the agreed panel definition assumptions from WF in [4]:

‘Panel’ is defined as a group of antenna element that controls beam independently and has the following attributes:

  - Within a panel, one beam can be selected and used for DL reception or UL transmission.

  - Across different panels, multiple beams (each selected per panel) may be used for DL reception or UL transmission

  - ‘Beam’ is assumed to mean spatial filter associated with transmission or reception

In the above WF agreement, the term “beam” is associated with the “spatial filter” used for reception. This is an important assumption that makes a direct connection with the active TCI state describing the RS QCL properties, the RS linkage used for reception, and RRM measurements.

In 38.101-2 specification, subclause 6.2.4, the Pcmax definition has a reference point for pathloss estimation that is related to the one indicated in the PUSCH power allocation equation (subclause 12, TS38.213):

where the PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, of carrier f of serving cell c.

On the other hand, the TCI as in the RRC structure, contains the pathlossReferenceRS-Id, the ul-powerControl-r17 and the referenceSignal with its QCL characteristics. The BWP-UplinkDedicated information element describes the PUCCH, PUSCH configurations where the UL-TCI list is provided along with RS identifiers and UL power control identifier. Thus, the TCI state relation to the reference point of the measured pathloss for the UL beam power control is an important feature to note. We agree with the fact that defining the maximum configured power requirements for STxMP case requires the TCI state as beam indicator as it is part and fully aligned with the power allocation equation in 38.213 specification where the measured pathloss is involved.

Observation 1: RAN4 agreed to not use the panel notion in the RF requirements definition. Per TCI definition of the Maximum Configured Power is important for pathloss reference determination used in the power allocation equations from TS38.213.

Proposal 1: RAN4 to confirm the definition of the Maximum Configured Power per TCI.

# The mDCI STxMP related MPR derivation for Pcmax

In our opinion, the maximum Configured Power Pcmax for STxMP case requires a specific MPR approach. This is an important step that must be accomplished for Pcmax equations definition. The MPR per beam can be reused and with some specific derivation applied to the STxMP case when mDCI scheduling is assumed, as the UL grants may have different RB allocations and MCSs. Based on a specific UL transmissions AoD, the UE may have to apply different MPR values. We propose to explore these cases in the following paragraphs.

Assuming per beam Pcmax, i.e., TCI-based Pcmax, in defining the requirement for STxMP, to respect the EIRPmax limits, the EIRP allocated power may or may not need to be summed across the panels. Clearly, this would be rather related to the UE implementation and beamforming capabilities. Figure 1 illustrates two exemplary scenarios:

Scenario (a)											Scenario (b)

Figure 1

Scenario (a):  There are two UL beams served by two panels, where the angle of departure (AoD) is sufficiently wide to consider two separated beams with their own EIRPmax limit (per panel). In this case, each UL beam may have its own EIRPmax limit with the corresponding per beam legacy MPR reductions.

Scenario (b): There are two beams served by two panels, but the angle of departure (AoD) is small enough to assume that the two separated beams may have their sum of EIRP powers smaller than EIRPmax. In this case the MPR that could be applied on both beams should be derived as a function of the legacy MPRs related to the UL grants. This MPR is applicable to both overlapping beams.

The TRPmax compliance per UE must be respected all cases, however the MPR needs to be derived differently because the mDCI scheduling case may lead to UL grants that have different RB allocations (overlapping or non-overlapping) and different MCSs.

Observation 2: For overlapping beams case, the STxMP related MPR needs to be derived differently because the mDCI case may lead to UL grants that have different RB allocations (overlapping or non-overlapping) and different MCSs.

In our opinion, in either case of Scenario (a) or (b), the corresponding MPR values should be determined on per beam basis. However, what would be different is the actual determined value for each case, as we will discuss it in the following paragraphs.

Observation 3: The MPR per beam (legacy MPR) is valid and its TCI relation is important under STxMP scenario.

Observation 4: For overlapping beams, the legacy MPRk per TCIk state for a Beamk and the MPRp per TCIp state for a Beamp can be used to derive the generic STxMP MPRg applicable to each beam, as a function of the two individual beams based MPRs:

MPRg = f(MPRk, MPRp)

For determination of the MPR for mDCI STxMP, there could be several cases in terms of UL grants where each grant may have a different RB allocation and MCS. As mentioned earlier, this becomes especially critical when the beams are overlapping, i.e., AoD is less than a certain value. In the following, we discuss MPR consideration with more details for each case of Scenario (a) and (b).

Scenario (a): When the beams are not-overlapping.

In this case, in our opinion, each UL beam may use its legacy defined MPR as required by its UL grant and then use an implementation-based Δ STxMP to accommodate different implementations for emission mask compliance.

Observation 5: When the beams are not overlapping, MPRk for TCIk state for a Beamk and MPRp for TCIp state for a Beamk are valid and can be applied individually while the TRPmax is the per UE limit to comply with.

Scenario (b): When beams are overlapping.

Case (b1): RB allocations are fully overlapping and the MCS may be different. For this case we believe that MPRg can be determined as,

MPRg = max {MPRk, MPRp} + ΔMPRg , where ΔMPRg = 3dB.

Beamk and RBk allocation  MPR1

Beamp and RBp allocation  MPR2

Case (b2): RB allocations RBk and RBp are partially overlapping and the MCS may be different. From our perspective, for STxMP, we may have to consider the union of the RB allocation. As a result, the following rule may be used:

MPRg = max {MPRk (RBk ∪ RBp),  MPRp (RBk ∪ RBp)} + ΔMPRg , where ΔMPRg = 3dB

where ∪ is the union symbol used to define the total RB allocations for STxMP.

Beamk and RBk allocation  MPR1

Beamp and RBp allocation  MPR2

Union of RB allocations between the two UL grants: RBk ∪ RBp  MPRg

Observation 6: For overlapping beams and when RB allocations RBk and RBp are overlapping or partially overlapping the following rule may be used:

MPRg = max {MPRk (RBk ∪ RBp),  MPRp (RBk ∪ RBp)} + ΔMPRg , where ΔMPRg = 3dB

where ∪ is the union symbol used to define the total RB allocations for STxMP.

Case (b3): RB allocations are not overlapping, but the MCS values may be different. In this case, as the UE is served by different panels, the MPRg may not be impacted. With the assumption of two different panels transmitting, in our opinion, these two non-overlapping RB allocations don’t impact have a mutual impact non per beam MPR.

Beamk and RBk allocation  MPR1

|  |  |  |  | Gap |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Beamp and RBp allocation  MPR2

|  |  |  |  | Gap |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Based on our discussion, we can see that when the STxMP UL transmissions are executed over 2 different panels (different PAs) the MPRp and MPRk per beam can be maintained and the EIRP directional limit must be respected and the TRPmax limit requirement stands.

Observation 7: When the STxMP UL transmissions are executed over 2 different panels (different PAs) while the beams are overlapping, but the RB allocation don’t overlap, the MPRp and MPRk per beam can be maintained and the EIRP directional limit must be respected and the TRP max limit requirement stands.

Proposal 2: For overlapping beams and overlapping RB allocations, consider the following generic STxMP MPR derivation rules:

MPRg = max {MPRk (RBk ∪ RBp),  MPRp (RBk ∪ RBp)} + ΔMPRg ,  where ΔMPR,g = 3dB

where

∪ is the union symbol used to define the total RB allocations for STxMP;

MPRg is applicable for each beam.

# The mDCI STxMP related MPR rules signaling

Based on UE beamforming capability there are cases where the MPR for STxMP derivation rules change, e.g., when the UL beams are overlapping. When the beams overlap, even with different hardware resources per beam, the UE needs to manage its emissions using the new MPR rules. The difference between legacy MPR per beam and the MPR for STxMP that is derived differently is essential for the gNB scheduler(s), as the overlapping of beams is not known by gNB. Additionally, for testing purposes, the MPR derivation rules status must be known eventually, so the appropriate power limits with the correct side conditions are tested.

In our opinion, it is important that the gNB(s) to know about the UE MPR derivation status. Since MPR derivation has an impact on Pcmax which itself is used for power headroom calculation, it would be important for gNB to know under what condition or rule the MPR value is determined. As such, the system can properly perform and assess the UE power capabilities, leaving plenty of implementation freedom.

Observation 8: Signaling the MPR derivation rules change for STxMP mDCI case for a combination of TCI states is required for gNB(s) scheduler(s) operation.

Moreover, if a new UL TCI is activated and an old one is de-activated, the MPR derivation rules may change, and this new situation may need to be signaled as well to the network for scheduling purposes.

Observation 9: Signaling the MPR rules change status for STxMP mDCI case when the combination of serving(active) UL TCI states changes is required for gNB(s) scheduling(s) and testing purposes.

Proposal 3: Send an LS to RAN2 and inform about the MPR rules derivation status for active TCI combination signaling in the PHR for STxMP capable UEs. A draft LS is presented in the Annex of this document.

# The Pcmax requirement for STxMP  mDCI

The Pcmax for each UL beam is evaluated at each slot when UL grants is/are scheduled. Therefore, the Pcmax requirement for STxMP may have to consider the MPR derivations rules according to the active serving TCI combination. Also, the same rules can be applied to A-MPR derivation.

For the MPR and A-MPR related STxMP rules, we propose to add explanatory text in the general sections of 6.2.2.0 and 6.2.3.0 subclauses respectively.

Proposal 4: For the MPR and A-MPR related STxMP rules when RB allocations are overlapping, add explanatory text in the Maximum Configured Power for STxMP case.

### 6.2D.4.1 Configured transmitted power for STxMP The UE can configure its maximum output power for each UL TCI state. The configured UE maximum output power PCMAX,f,c,k for TCI state k of carrier f and serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11]. The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k for each active TCI,k state indicated for STxMP is within the following bounds PPowerclass + DPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k) + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k,)), T(P-MPRf,c,k)} - ∆TSTxMP ≤ PUMAX,f,c,k ≤ EIRPmax where the corresponding measured peak EIRP for carrier f of a serving cell c, over all active TCI states configured for [STxMP], PUMAX,f,c satisfies PUMAX,f,c ≤ EIRPmax ∆TSTxMP is an additional implementation allowance for STxMP capable UEs. When the UE signals STxMP MPR and A-MPR new rules for beam overlapping and the RB1 and RB2 allocations are overlapping or partially overlapping, the union of the RB1 and RB2 allocations indicated by the mDCI UL grants corresponding to TCI states k and p respectively are considered for the MPRf,c,k and A- MPRf,c,k derivation as follows MPRf,c,k = max {MPR,k (RB,1 U RB,2), MPR,p (RB,1 U RB,2)} + 3dB , A-MPRf,c,k = max {A-MPR,k (RB,1 U RB,2), A-MPR,p (RB,1 U RB,2)} + 3dB The corresponding measured total radiated power PTMAX,f,c is always bounded by PTMAX,f,c ≤ TRPmax

Proposal 5: We propose the following text for the Pcmax definition changes that are specific to STxMP capability:

# The MPE handling in the context of STxMP

In the WF from meeting #106, for the STxMP case in FR2, companies agreed to consider the following power classes from TS38.101-2:

Consider PC1/PC2/PC4/PC5/[PC6] only.

The above power classes are considered for the RF requirements study. It is important to mention that the proposed power classes have specific form-factors and thus they may have different antenna capabilities as shown below in Table 6.2.1.0-1 (38.101-2).

Table 6.2.1.0-1: Assumption of UE Types

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |
| 5 | Fixed wireless access (FWA) UE |
| 6 | High Speed Train Roof-Mounted UE |
| 7 | RedCap UE |
| Note: RedCap variants of non-RedCap UEs are not precluded |  |

Based on the agreement, a handheld UE is not part of the study (PC3).

However, since the MPE related P-MPR is reported per beam in the current specifications, we believe that its possible impact is covered within the PHR P-MPR per beam reporting framework.

Observation 10: The agreed list of power classes excludes the handheld device (PC3). Current PHR P-MPR per beam reporting cover the possible impact for Rel-18.

# Conclusions

This contribution discussed RF requirements for STxMP mDCI. Based on the discussion, following observations and proposals are made,

Observation 1: RAN4 agreed to not use the panel notion in the RF requirements definition. Per TCI definition of the Maximum Configured Power is important for pathloss reference determination used in the power allocation equations from TS38.213.

Observation 2: For overlapping beams case, the STxMP related MPR needs to be derived differently because the mDCI case may lead to UL grants that have different RB allocations (overlapping or non-overlapping) and different MCSs.

Observation 3: The MPR per beam (legacy MPR) is valid and its TCI relation is important under STxMP scenario.

Observation 4: For overlapping beams, the legacy MPRk per TCIk state for a Beamk and the MPRp per TCIp state for a Beamp can be used to derive the generic STxMP MPRg applicable to each beam, as a function of the two individual beams based MPRs:

MPRg = f(MPRk, MPRp)

Observation 5: When the beams are not overlapping, MPRk for TCIk state for a Beamk and MPRp for TCIp state for a Beamk are valid and can be applied individually while the TRPmax is the per UE limit to comply with.

Observation 6: For overlapping beams and when RB allocations RBk and RBp are overlapping or partially overlapping the following rule may be used:

MPRg = max {MPRk (RBk ∪ RBp),  MPRp (RBk ∪ RBp)} + ΔMPRg , where ΔMPRg = 3dB

where ∪ is the union symbol used to define the total RB allocations for STxMP.

Observation 7: When the STxMP UL transmissions are executed over 2 different panels (different PAs) while the beams are overlapping, but the RB allocation don’t overlap, the MPRp and MPRk per beam can be maintained and the EIRP directional limit must be respected and the TRP max limit requirement stands.

Observation 8: Signaling the MPR derivation rules change for STxMP mDCI case for a combination of TCI states is required for gNB(s) scheduler(s) operation.

Observation 9: Signaling the MPR rules change status for STxMP mDCI case when the combination of serving(active) UL TCI states changes is required for gNB(s) scheduling(s) and testing purposes.

Observation 10: The agreed list of power classes excludes the handheld device (PC3). Current PHR P-MPR per beam reporting cover the possible impact for Rel-18.

Proposal 1: RAN4 to confirm the definition of the Maximum Configured Power per TCI.

Proposal 2: For overlapping beams and overlapping RB allocations, consider the following generic STxMP MPR derivation rules:

MPRg = max {MPRk (RBk ∪ RBp),  MPRp (RBk ∪ RBp)} + ΔMPRg ,  where ΔMPR,g = 3dB

where

∪ is the union symbol used to define the total RB allocations for STxMP;

MPRg is applicable for each beam.

Proposal 3: Send an LS to RAN2 and inform about the MPR rules derivation status for active TCI combination signaling in the PHR for STxMP capable UEs. A draft LS is presented in the Annex of this document.

Proposal 4: For the MPR and A-MPR related STxMP rules when RB allocations are overlapping, add explanatory text in the Maximum Configured Power for STxMP case.

Proposal 5: We propose the following text for the Pcmax definition changes that are specific to STxMP capability:

### 6.2D.4.1 Configured transmitted power for STxMP The UE can configure its maximum output power for each UL TCI state. The configured UE maximum output power PCMAX,f,c,k for TCI state k of carrier f and serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11]. The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k for each active TCI,k state indicated for STxMP is within the following bounds PPowerclass + DPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k) + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k,)), T(P-MPRf,c,k)} - ∆TSTxMP ≤ PUMAX,f,c,k ≤ EIRPmax where the corresponding measured peak EIRP for carrier f of a serving cell c, over all active TCI states configured for [STxMP], PUMAX,f,c satisfies PUMAX,f,c ≤ EIRPmax When the UE signals STxMP MPR and A-MPR new rules for beam overlapping and the RB1 and RB2 allocations are overlapping or partially overlapping, the union of the RB1 and RB2 allocations indicated by the mDCI UL grants corresponding to TCI states k and p respectively are considered for the MPRf,c,k and A- MPRf,c,k derivation as follows MPRf,c,p = MPRf,c,k = max {MPR,k (RB,1 U RB,2), MPR,p (RB,1 U RB,2)} – 3dB , A-MPRf,c,p = A-MPRf,c,k = max {A-MPR,k (RB,1 U RB,2), A-MPR,p (RB,1 U RB,2)} – 3dB where MPR,k and MPR,p are defined in the subclause 6.2.2 and A-MPR,k, A-MPR,p are defined in the subclause 6.2.3. The corresponding measured total radiated power PTMAX,f,c is always bounded by PTMAX,f,c ≤ TRPmax

# References

- RP-213598, “New WID: MIMO evolution for Downlink and Uplink”, Samsung, 3GPP RAN Meeting #94e, December 6-17, 2021

- R4-2303494, LS reply for UE power limitation for STxMP in FR2, Samsung, RAN4 #106, Athens, Greece

- 38.101-2, NR User Equipment (UE) radio transmission and reception; Part 2; Rel-17

- R4-2306629, WF on UE RF requirements for STxMP, Samsung, RAN4 106bis-e, April 2023, e-Meeting

# Annex

Title:	LS on multi-DCI multi-TRP STxMP PHR enhancement

Response to: 	-

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	3GPP TSG-RAN WG4

To:	3GPP TSG-RAN WG2

CC: 		3GPP TSG-RAN WG1

Contact Person:

Name:			Virgil Comsa

Email Address:	virgil.comsa@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

RAN4 studied the Maximum Configured Power requirements and as a part of discussion the MPR derivation for STxMP transmissions when the beams have a certain overlap leading to a different MPR derivation than for single beam transmissions.

RAN4 agreed to a new MPR derivation rule which happens when overlapping UL beams occur while serving a pair of TCI active states in STxMP mode. Due to this situation, there are two sets of possible MPR derived values for the Pcmax per beam requirement (for overlapping and non-overlapping beams respectively). Non-overlapping beams use the legacy MPR values per individual beam, while overlapping beams use the new MPR derivation rule applicable to each individual beam.

RAN4 also noted that this situation, leading to an UL beam overlap can happen for a certain STxMP TCI active states combination. When at least one TCI active state within the STxMP active TCI pair combination change, the MPR derivation rules may change as well.

Thus, RAN4 respectfully suggest RAN2 enhancing the PHR with an indication for the STxMP MPR derivation rules change.

2. Actions:

To RAN2 group:

ACTION: 	RAN4 respectfully asks RAN2 to take the agreement into consideration in the future work.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG4 Meeting #108 	August 2023      Toulouse

TSG-RAN WG2 Meeting #108bis	October 2023     P.R. China