3GPP TSG RAN WG4 #107		R4-2307055

Incheon, Korea, May 22nd – May 26th , 2023

# Title:	Simulation results for NB-IoT NTN demodulation

# Source:	Lockheed Martin

# Document for: 	Information

# Agenda Item:	[9.7.7] 	LTE_NBIOT_eMTC_NTN_req-Perf

# Introduction

From RAN4#105, there were agreements on IoT-NTN UE demodulation simulation assumptions [1]. Updates to the assumptions were agreed upon in RAN4#106 [2] and the simulation configurations held in RAN4#106-bis-e [3].

Agreements on IoT-NTN SAN demodulation simulation assumptions were made in RAN4#106bis-e [4].

In this contribution we provide our simulation results for both the NB-IoT UE and SAN demodulation NTN testcases.

# Simulation Results

## NB-IoT: NPDSCH for UE demodulation

In this section we provide our simulation configuration used and results for NB-IoT NTN UE demodulation of the downlink shared channel.

### Simulation Parameters

Shared parameters across tests are shown in Table 1. Two unique test cases were examined under the conditions and settings provided in Table 2.

Table 1. Shared simulation parameters for NPDSCH demodulation requirements.

| Operation mode | CBW | Antenna configuration | K_offset | CFO |
| --- | --- | --- | --- | --- |
| Stand-alone | FDD 200 kHz | 1 Tx, 1 Rx | 8 ms | 200 Hz |

Table 2. Test configurations for NPDSCH NTN UE demodulation requirements.

| Parameter | Test Case |  |
| --- | --- | --- |
|  | 1 | 2 |
| MCS | QPSK 1/2 (R.NB.6 FDD) | QPSK 1/3 (R.NB.6-1 FDD) |
| NPDSCH repetition | 32 | 128 |
| Propagation condition | NTN-TDLC5-200 | NTN-TDLA100-10 |
| Carrier type | Anchor | Non-anchor |
| ITBS/ISF | 9/3 | 6/3 |
| TBS (bits) | 616 | 392 |

### Simulation Results

The SNR (dB) required to achieve 70% of the maximum throughput for each test case is presented in Table 3.

Table 3. Simulation results for NPDSCH NTN UE demodulation.

| Test Case | Required SNR (dB) for 70% maximum throughput to be achieved |
| --- | --- |
| 1 | -6.9 |
| 2 | -12.6 |

## NB-IoT: NPUSCH Format 1 for SAN demodulation

In this section we provide our simulation configuration used and results for NB-IoT NTN SAN demodulation of the uplink shared channel.

### Simulation Parameters

Unique test cases were examined under the conditions and settings provided in Table 4.

Table 4. Test configurations for NPUSCH Format 1 NTN SAN demodulation requirements.

| Parameter | Test Case |  |
| --- | --- | --- |
|  | 1 | 2 |
| Sub-carrier spacing | 3.75 kHz | 15 kHz |
| No. of allocated sub-carriers | 1 | 12 |
| MCS | BPSK 1/3 (TS 36.104 A16-1) | QPSK 2/3 (TS 36.104 A16-5) |
| NPUSCH repetition | 4 | 16 |
| Propagation condition | Option 1: NTN-TDLA100-1 Option 2: NTN-TDLC5-1 | Option 1: NTN-TDLC5-1 Option 2: NTN-TDLA100-1 |
| Carrier frequency offset | 200 Hz | 200 Hz |
| Number of Tx antennas | 1 | 1 |
| Number of Rx antennas | 1 and 2 | 1 and 2 |
| ITBS/IRU | 0/1 | 9/0 |
| Payload size (bits) | 32 | 136 |

### Simulation Results

The SNR (dB) required to achieve 70% of the maximum throughput for each test case using 1 receive antenna is presented in Table 5. Similarly, the simulation results for using 2 receive antennas is shown in Table 6.

Table 5. NTN SAN demodulation simulation results for NPUSCH Format 1 with 1 Rx antenna.

| Test Case | Propagation condition | Required SNR (dB) for 70% maximum throughput to be achieved |
| --- | --- | --- |
| 1 | NTN-TDLA100-1 | 1.3 |
|  | NTN-TDLC5-1 | -0.6 |
| 2 | NTN-TDLC5-1 | -7.2 |
|  | NTN-TDLA100-1 | -5.6 |

Table 6. NTN SAN demodulation simulation results for NPUSCH Format 1 with 2 Rx antennas.

| Test Case | Propagation condition | Required SNR (dB) for 70% maximum throughput to be achieved |
| --- | --- | --- |
| 1 | NTN-TDLA100-1 | -2.1 |
|  | NTN-TDLC5-1 | -3.2 |
| 2 | NTN-TDLC5-1 | -10.3 |
|  | NTN-TDLA100-1 | -9.1 |

## NB-IoT: NPUSCH Format 2 for SAN demodulation

In this section we provide our simulation configuration used and results for NB-IoT NTN SAN demodulation of the uplink control channel.

### Simulation Parameters

Unique test cases were examined under the conditions and settings provided in Table 7.

Table 7. Test configurations for NPUSCH Format 2 NTN SAN demodulation requirements.

| Parameter | Test Case |  |
| --- | --- | --- |
|  | 1 | 2 |
| Sub-carrier spacing | 3.75 kHz | 15 kHz |
| No. of allocated sub-carriers | 1 | 1 |
| NPUSCH repetition | 16 | 16 |
| Propagation condition | Option 1: NTN-TDLA100-1 Option 2: NTN-TDLC5-1 | Option 1: NTN-TDLA100-1 Option 2: NTN-TDLC5-1 |
| Carrier frequency offset | 200 Hz | 200 Hz |
| Number of Tx antennas | 1 | 1 |
| Number of Rx antennas | 1 and 2 | 1 and 2 |

### Simulation Results

The SNR (dB) required to achieve an ACK missed detection probability less than or equal to 1% for each test case using 1 receive antenna is presented in Table 8. Similarly, the simulation results for using 2 receive antennas is shown in Table 9.

Table 8. NTN SAN demodulation simulation results for NPUSCH Format 2 with 1 Rx antenna.

| Test Case | Propagation condition | Required SNR (dB) for ACK missed detection probability ≤ 1% |
| --- | --- | --- |
| 1 | NTN-TDLA100-1 | 3.8 |
|  | NTN-TDLC5-1 | -0.5 |
| 2 | NTN-TDLA100-1 | 0.3 |
|  | NTN-TDLC5-1 | -6.2 |

Table 9. NTN SAN demodulation simulation results for NPUSCH Format 2 with 2 Rx antennas.

| Test Case | Propagation condition | Required SNR (dB) for ACK missed detection probability ≤ 1% |
| --- | --- | --- |
| 1 | NTN-TDLA100-1 | -2 |
|  | NTN-TDLC5-1 | -5.2 |
| 2 | NTN-TDLA100-1 | -7.5 |
|  | NTN-TDLC5-1 | -10.7 |

# References

- R4-2220278, “WF on IoT-NTN UE demodulation and CQI reporting requirements”, MediaTek.

- R4-2302932, “WF on IoT-NTN UE demodulation and CQI reporting requirements”, MediaTek.

- R4-2305960, “WF on IoT-NTN UE demodulation requirements”, MediaTek.

- R4-2305965, “WF on IoT NTN SAN demodulation requirements”, Ericsson.