3GPP TSG-RAN WG4 Meeting # 107			R4-2307130

# Incheon, KR, May 22 – May 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On MultiRx RLM and BFD/CBD requirements

Agenda item:	8.8.3.3

Document for:	Discussion

Introduction

In this contribution, we discuss RLM and BFD/CBD requirements for multi Rx for the single-DCI and multi-DCI scenarios.

RLM requirements

In the RAN4 requirements, RLM is treated considering the procedure defined in RAN1 with the definition of measurement times and scheduling/measurement restrictions when performing RLM. One general overview of the RLM procedure is shown in Figure 1. In this figure, the UE is configured with a set of reference signals (either SSB or CSI-RS) for performing RLM.  In case those are not configured, there are existing RAN1 rules where the reference signals are implicitly defined based on the configured TCI states for PDCCH reception. Two criteria, Qin and Qout, are used, which are equivalent to X % PDCCH BLER for a given PDCCH configuration defined in RAN4 requirements (Table 8.1.2.1-1 and 8.1.2.1-2 in 38.133). If the counter for out-of-sync indication exceeds N310, the UE declares radio link failure, and starts monitoring the in-sync criteria. If the UE does not meet the Qin criteria within the timer T310, the UE declares radio link failure.

Figure 1 Single TRP diagram of RLM procedure

One important consideration about RLM is that it monitors the UE capability of receiving PDCCH. In that case, the different scenarios that are considered for multi Rx UEs have to be considered. In order to analyse this scenario in this paper we evaluate the requirements for the single DCI scenario with non-SFN PDCCH and multi-DCI scenario.

Qin/Qout thresholds for RLM are based on hypothetical PDCCH BLER.

Single-DCI

When considering the single-DCI non-SFN PDCCH, there is only one TRP with one indicated TCI state for PDCCH, while the UE may have two simultaneous TCI states indicated for receiving PDSCH form two TRPs. In that case, it makes sense that the UE performs RLM only on that anchor TRP carrying PDCCH, since the other TRP is not monitored by the UE for PDCCH. Therefore, the baseline of the RLM requirements should apply for multi TRP as well.

For single-DCI non-SFN scenario, UE performs RLM only on one anchor TRP, which is why baseline of single TRP RLM requirements should also apply for multi TRP.

For single DCI scenario, RLM is monitored only on the link with PDCCH, and legacy RLM requirement baseline applies.

Multi-DCI

One case that is not clearly covered in RAN4 specification is the case of multi-DCI. The configuration of RLM failureDetectionResources is not preventing the network to configure reference signals from two different TRPs. Additionally, from RAN1 specification it is implicit that in the case of multi-DCI, if RLM resources are not configured, the RLM-RS are implicitly configured with the reference signals of the indicated TCI states for PDCCH.

The configuration of failureDetectionResources for RLM can include reference signals from different QCL-D sources.

If failureDetectionResources for RLM is not configured, the reference signals of the indicated TCI states for PDCCH are implicitly configured as sources for RLM-RS.

For out-of-sync and in-sync evaluation in multi-DCI scenario the above observations mean the following:

Qout: Out-of-sync is indicated to higher layers when the radio link quality is worse than the threshold Qout for all resources in the set of resources for radio link monitoring. When the RLM-RS originate from two TRPs and out-of-sync is indicated, this means that both links are out-of-sync, and if radio link failure follows, this applies to both links i.e. the whole cell. It is not possible to detect out-of-sync for only one of the links, since the evaluation is done based on RLM-RS originating from two TRS together. Therefore, radio link failure based on RLM always applies for the whole cell.

In multi-DCI scenario, out-of-sync indication means that the radio link quality for all RLM-RS originating from the two different TRPs is worse than Qout during the out-of-sync evaluation period. Out-of-sync indication therefore means that both links are out-of-sync.

Qin: In-sync is indicated to higher layers when the radio link quality is better than threshold Qin for any resource in the set of resources for radio link monitoring. When the RLM-RS originate from two TRPs, it is not clear which of the links the in-sync indication applies to. In-sync indication purely means that on at least one of the links Qin was better than threshold during the evaluation period.

In multi-DCI mode, in-sync indication means that the radio link quality for any RLM-RS originating from any of the two TRPs is better than Qin during the in-sync evaluation period. It cannot be distinguished for which of the links the in-sync indication applies to.

Based on observations 3-6, we think RAN4 should clarify the existing RLM requirements to cover the multi-DCI scenario, where the RLM-RS originate from two different TRPs.

RAN4 to clarify in the RLM requirements that the RLM-RS resources in multi-TRP mode may originate from two different TRPs and out-of-sync and in-sync indication applies for the whole cell.

Regarding the beam sweeping factor N for the RLM requirements for single-DCI and multi-DCI scenarios, RAN4 can follow the discussion under L1 measurement requirements AI.

Regarding the beam sweeping factor N for the RLM requirements for single-DCI and multi-DCI scenarios, follow the discussion under L1 measurement requirements AI.

Link recovery procedures

Multi-DCI

In multi-DCI scenario, PDCCH is transmitted from both TRPs, and link quality is monitored on both links. The framework for link recovery was enhanced in Rel-17 for multi TRP operation by the possibility to configure different sets of reference signals for each TRP as shown in Figure 3. When beam failure is detected on both links, beam failure recovery is triggered by the UE by initiating a random access procedure. When beam failure is detected one of the links, beam failure recovery is triggered by initiating a transmission of a BFR MAC CE for the BFD-RS set for the link that fails.

Figure 3 Beam failure detection and recovery framework for multi TRP operation

TRP-specific link recovery procedure is also reflected in RAN4 requirements, where the two sets of reference signals are considered in clause 8.18 of 38.133.

For multi-DCI scenario, the baseline of the legacy TRP-specific link recovery requirements in section 8.18 of TS 38.133 can in our view apply directly. However, one aspect that the Rel-17 requirements include is the PTRP factor, which accounts for overlapping between SSB/CSI-RS resources from both TRPs. In the case of multi-Rx, the PTRP factor may be evaluated again, since there is the possibility for the UE to monitor reference signals from the sets from one TRP simultaneously with the ones from the other TRP.

TRP specific link recovery procedure in Rel-17 include PTRP=2 for CBD and BFD if sets of reference signals related to both TRPs are overlapped.

For multi-DCI scenario, RAN4 to adopt Rel-17 TRP specific link recovery requirements as a baseline for multi-Rx operation.

RAN4 to consider PTRP=1 for the BFD and CBD requirements of multi-Rx operation.

Single-DCI

In single-DCI mode, PDCCH is transmitted only from one TRP. According to the existing RAN1 specifications, link quality monitoring is defined for PDCCH, which means in our understanding that in single-DCI scenario, beam failure detection and candidate beam selection applies only to this anchor link.

In single-DCI scenario, link quality monitoring for beam failure detection applies only to the link with PDCCH.

Since only single link is considered, this would mean that the existing cell-specific requirements in section 8.5 of TS 38.133 may apply.

In single-DCI scenario, link quality monitoring is done only on the link with PDCCH, so legacy link recovery requirements in section 8.5 apply.

Conclusion

In this contribution we have made the following observations and proposals:

Observation 1: Qin/Qout thresholds for RLM are based on hypothetical PDCCH BLER.

Observation 2: For single-DCI non-SFN scenario, UE performs RLM only on one anchor TRP, which is why baseline of single TRP RLM requirements should also apply for multi TRP.

Proposal 1: For single DCI scenario, RLM is monitored only on the link with PDCCH, and legacy RLM requirement baseline applies.

Observation 3: The configuration of failureDetectionResources for RLM can include reference signals from different QCL-D sources.

Observation 4: If failureDetectionResources for RLM is not configured, the reference signals of the indicated TCI states for PDCCH are implicitly configured as sources for RLM-RS.

Observation 5: In multi-DCI scenario, out-of-sync indication means that the radio link quality for all RLM-RS originating from the two different TRPs is worse than Qout during the out-of-sync evaluation period. Out-of-sync indication therefore means that both links are out-of-sync.

Observation 6: In multi-DCI mode, in-sync indication means that the radio link quality for any RLM-RS originating from any of the two TRPs is better than Qin during the in-sync evaluation period. It cannot be distinguished for which of the links the in-sync indication applies to.

Proposal 2: RAN4 to clarify in the RLM requirements that the RLM-RS resources in multi-TRP mode may originate from two different TRPs and out-of-sync and in-sync indication applies for the whole cell.

Proposal 3: Regarding the beam sweeping factor N for the RLM requirements for single-DCI and multi-DCI scenarios, follow the discussion under L1 measurement requirements AI.

Observation 7: TRP specific link recovery procedure in Rel-17 include PTRP=2 for CBD and BFD if sets of reference signals related to both TRPs are overlapped.

Proposal 4: For multi-DCI scenario, RAN4 to adopt Rel-17 TRP specific link recovery requirements as a baseline for multi-Rx operation.

Proposal 5: RAN4 to consider PTRP=1 for the BFD and CBD requirements of multi-Rx operation.

Observation 8: In single-DCI scenario, link quality monitoring for beam failure detection applies only to the link with PDCCH.

Proposal 6: In single-DCI scenario, link quality monitoring is done only on the link with PDCCH, so legacy link recovery requirements in section 8.5 apply.

- References

- R4-2306318, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo

- R4-2306392, Topic summary for [106bis-e][202] FR2_multiRx_part1, vivo