3GPP TSG-RAN WG4 Meeting #107 	R4-2307187

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Rel-18 extension of Unified TCI framework for mTRP operation

Agenda item:	8.30.3.3

Document for:	Discussion

Introduction

In RAN4#106bis, the following agreements were made regarding the scope of the MIMO_evo_DL_UL work in RAN4 R4-2306362 [1]:

Topic #3 Unified TCI Framework extended to M-TRP Issue 3-1-1: In general, do you agree RRM requirements are impacted by extension of unified TCI framework to M-TRP? - Proposals - Option 1: Yes. (Nokia, Intel, Samsung, Ericsson, Huawei) - Option 2: Not so clear, wait for more RAN1 progress. (Apple, vivo, MediaTek) Issue 3-1-2: For extension of Rel-17 unified TCI framework, whether to support sDCI and mDCI? Agreement: - Both sDCI and mDCI based MTRP are considered for extension of Rel-17 unified TCI framework for multi-TRP. Issue 3-1-3: For extension of Rel-17 unified TCI framework, whether to support intra-cell mTRP and inter-cell mTRP scenarios? GTW agreements: - Agreements - Consider both intra-cell and inter-cell mTRP scenarios - FFS if inter-cell mTRP scenario would apply for simultaneous reception based mTRP scheme Issue 3-1-4: For extension of Rel-17 unified TCI framework, whether to support simultaneous reception in mTRP? - Proposals - Option 1: Not consider simultaneous reception in mTRP in Rel-18 (Intel, MediaTek, Apple, vivo, Samsung) - Option 2: Consider simultaneous reception in mTRP in Rel-18. FFS on how to do the extension (Nokia, Xiaomi, Ericsson) - Option 3: FFS if any aspects of unified TCI extension to mTRP impacts RRM requirements with multi-RX reception in FR2. (Apple, Huawei) Issue 3-1-5: How to separate the TCI state switching requirements? - Proposals - Option 1: no RRM impacts - Option 2: wait for further RAN1 progress - Option 3: - Separate for sDCI and mDCI. - For sDCI based mTRP: - FFS on whether/how to impact TCI state switching requirements. - For mDCI based mTRP: - FFS on whether/how to impact TCI state switching requirements. Issue 3-1-8: Whether to enhance TRP-specific BFR requirements? Agreement: - Postpone the discuss until there is more RAN1 conclusion. Issue 3-1-9: Unified TCI extension if UE can support sTxMP? - Proposals - Option 1: (Apple) - R17 MAC CE TCI switch requirements for UL TCI can be applicable to R18 extension for mTRP and STxMP. - Option 2: (Intel, vivo, MTK) - For single-panel based scheme, Rel-17 UL TCI state list update delay can apply for MAC CE based TCI states activation in both sDCI and mDCI scenario. - Suggest to discuss multi-TX panel related requirement in future release. - Option 3: (Samsung, Huawei, Nokia, Ericsson) - FFS Issue 3-1-10: For extension of Rel-17 unified TCI framework, whether to consider repetition and SFN for RRM impacts? - Proposals - Option 1: (Intel, MTK) - suggest not to consider PDCCH repetition and SFN. - Option 2: (Huawei, Apple, Ericsson) - FFS

This paper discusses the extension of the Unified TCI framework to mTRP. In this context, sDCI and mDCI operations are addressed as well as support for intra- and inter-cell scenarios. Furthermore, switching requirements are discussed.

There were two agreements in RAN4#106 bis (namely on support for sDCI and mDCI and on postponing TRP-specific BFR) which are not rediscussed in this paper.

Discussion

Simultaneous reception in mTRP

In the last RAN4 meeting, the simultaneous reception using mTRP was discussed in the following issues [1]:

Issue 3-1-3: For extension of Rel-17 unified TCI framework, whether to support intra-cell mTRP and inter-cell mTRP scenarios? - Agreements - Consider both intra-cell and inter-cell mTRP scenarios - FFS if inter-cell mTRP scenario would apply for simultaneous reception based mTRP scheme Issue 3-1-4: For extension of Rel-17 unified TCI framework, whether to support simultaneous reception in mTRP? - Proposals - Option 1: Not consider simultaneous reception in mTRP in Rel-18 (Intel, MediaTek, Apple, vivo, Samsung) - Option 2: Consider simultaneous reception in mTRP in Rel-18. FFS on how to do the extension (Nokia, Xiaomi, Ericsson) - Option 3: FFS if any aspects of unified TCI extension to mTRP impacts RRM requirements with multi-RX reception in FR2. (Apple, Huawei)

The Rel-17 FeMIMO Objectives include inter-cell scenario (RP-193133), see objective 2 below:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception

Ending Rel-17, we have crystallized the Rel-17 features of FeMIMO in RP-220802 [2], where inter-cell multi-TRP operation is listed:

In Rel-17, multi-TRP PDSCH transmission is further extended to inter-cell operation. A UE can be configured with SSB associated with PCI which are different from serving cell PCI, known as additional PCI. At most 7 different additional PCI can be configured to the UE, and only one is activated for inter-cell multi-TRP operation. The additional PCI can be associated with one or more TCI states, and gNB can schedule PDSCH dynamically from either TRP by indicating TCI in DCI. In Rel-17, beam reporting and BFR are enhanced for mTRP scenario specifically. For beam reporting enhancement, framework of Rel-15 group based beam reporting is extended to facilitate simultaneous transmission of multiple TRPs. To achieve that, one CSI resource setting including two resource sets corresponding to two TRPs can be configured. In a CSI-report, UE can report N (Nmax = {1, 2, 3, 4}) groups of simultaneously received beams, wherein each reported beam in one group corresponds to one TRP. Differential L1-RSRP reporting across all beam groups is supported, and 1-bit is added to indicate CMR set associated with the largest RSRP value in the first group

Furthermore, in Rel-17, RAN1 agreed:

Agreement (RAN1#109e) : On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17

From TS38.331, the SSB-Resourceset IE includes the servingAdditionalPCIList-r17, which gives the PCI information:

Furthermore, from TS 38.331, if groupBasedBeamReporting-v1710 is configured in the CSI-ReportConfig IE, then it indicates that this CSI-ResourceConfig is used as resourceForChannelMeasurement. The network configures 2 resource sets, which may be:

- two NZP CSI-RS resource sets,

- two CSI SSB resource sets, or

- one NZP CSI-RS resource set and one CSI-SSB resource set

(see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2) and copied below:

Moreover, aperiodic CSI resources may be associated to resourcesForChannel and resourcesforChannel2-r17 as shown below:

To summarize on the inter-cell support of group based beam reporting:

Current specification support CSI reports, CMR, the additional PCI list and even the ICBM framework. As such, it seems that in practice, both intra-cell and inter-cell operations as feasible for group based beam reporting.

Both intra-cell and inter-cell operations as feasible for Rel-17 group based beam reporting.

Since group based beam reporting is feasible for inter-cell mTRP, support in RAN4 that the inter-cell mTRP scenario applies to simultaneous reception based mTRP scheme

RAN4 to define requirements for simultaneous reception in Rel-18 including intra-cell and inter-cell mTRP scenario.

TCI state switching requirements

In the last RAN4 meeting the TCI switching requirements were discussed in the following issues [1]:

Issue 3-1-5: How to separate the TCI state switching requirements? - Proposals - Option 1: no RRM impacts - Option 2: wait for further RAN1 progress - Option 3: - Separate for sDCI and mDCI. - For sDCI based mTRP: - FFS on whether/how to impact TCI state switching requirements. - For mDCI based mTRP: - FFS on whether/how to impact TCI state switching requirements.

In single-DCI scenario, a single-DCI is used to indicate both TCI states. In case of multi-DCI based TCI state switching, the UE is expected to receive DCI commands for each PDSCH independently, and therefore will have two TCI states indicated for PDCCH and two TCI states indicated for PDSCH. In Rel-16, RAN1 has already specified that TCI switching is related to the timeDurationForQCL,

On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the UE can assume that one beam application time (BAT) for a given SCS is configured for all the CCs configured with the common TCI state ID update.

In our view, this discussion has to be analysed case by case, since some of the requirements only make sense either for sDCI or for mDCI. The mechanisms for indicating 2 TCI states are not common for the sDCI and mDCI. As an example, indication of 2 TCI states using DCI is not supported by mDCI operation currently, since in multi DCI scenario the indication of 2 TCI states is done independently for each coresetPoolIndex. Note that the configuration of s-DCI or m-DCI is performed via RRC Configuration.

TCI switching commands for mTRP are different for sDCI and mDCI.

Support option 3: RAN4 to define different requirements for sDCI and mDCI scenarios.

TCI switching requirements for sDCI and for mDCI to include support of both configurations of joint or separate frameworks.

BFR requirements enhancements

The beam failure recovery requirements were discussed in the following issues [1]:

Issue 3-1-8: Whether to enhance TRP-specific BFR requirements? Agreement: - Postpone the discuss until there is more RAN1 conclusion.

In RAN4#106bis, it was agreed to postpone the discussion such that RAN1 can further on this topic. In the meantime, RAN1#110bis-e made the following agreements to study enhancements for TRP-specific BFR as follows:

RAN1#110bis Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request

As the S-DCI mTRP does not use CORESETpoolIndex, the determination of BFD-RS in BFD-RS sets would need to be based on the indicated TCI states (first and/or second). The S-DCI framework allows one or two indicated TCI states for PDCCH (e.g. for repetition) based on the configuration associated with one or more CORESETs.

If the CORESET(s) of a BWP are associated with a configuration for first indicated TCI state only UE determines that it monitors the beam failure on using one BFD-RS set. If UE receives indication for second TCI State it is applied for at least PDSCH reception but not for PDCCH monitoring and thus it does not have effect on implicit BFD-RS configuration

If CORESET(s) of a BWP are associated with configuration of first indicated TCI state, UE monitors beam failure on one set of q0, and the set includes RS included in the TCI state.

If a CORESET (or set) is associated with a configuration for first indicated TCI state and another CORESET (or sets) is associated with a configuration for second indicated TCI state, the UE determines to monitor beam failure on two BFD-RS sets. The RS indicated by the first TCI state is included in the first BFD-RS set and the RS indicated by the second TCI state in the second BFD-RS set. (set q0,0 includes RS included in the first TCI state, set q0,1 includes RS included in the second TCI state).

As such, there was an update on RAN1 agreements related to BFR, but it is still too early to conclude on how RAN4 requirements should be updated.

Postpone discussion on TRP-specific BFR requirements.

Unified TCI extension for STxMP

The unified TCI extension for STxMP was discussed in the following issues [1]:

Issue 3-1-9: Unified TCI extension if UE can support sTxMP? - Proposals - Option 1: (Apple) - R17 MAC CE TCI switch requirements for UL TCI can be applicable to R18 extension for mTRP and STxMP. - Option 2: (Intel, vivo, MTK) - For single-panel based scheme, Rel-17 UL TCI state list update delay can apply for MAC CE based TCI states activation in both sDCI and mDCI scenario. - Suggest to discuss multi-TX panel related requirement in future release. - Option 3: (Samsung, Huawei, Nokia, Ericsson) - FFS

Current MAC-CE TCI Switch requirements for UL TCI are defined for the unified TCI framework considering a single UL target TCI state. One big difference in Rel 18 is that simultaous transmission is being defined, where 2 target TCI states may be indicated/activated. Therefore, in order to support STxMP, RAN4 needs to define requirements for TCI switch and activation delay considering the simultaneous transmission.

RAN4 to discuss requirements for STxMP MAC CE TCI switching requirements when target TCI state includes 2 TCIs.

RAN4 to discuss requirements for STxMP DCI TCI switching requirements when target TCI state includes 2 TCIs.

RAN4 to consider TCI switch delay that include timing reference signal per TCI state.

PDCCH Repetition and SFN for RRM impact

The PDCCH repetition and SFN was discussed in the following issues [1]:

Issue 3-1-10: For extension of Rel-17 unified TCI framework, whether to consider repetition and SFN for RRM impacts? - Proposals - Option 1: (Intel, MTK) - suggest not to consider PDCCH repetition and SFN. - Option 2: (Huawei, Apple, Ericsson) - FFS

From Rel-17, RAN1 has agreed the following in FeMIMO for PDCCH repetitions in RP-220802 [2]:

In Rel-17, PDCCH repetition is defined by explicit linkage between two search space sets. The two linked search space sets can be associated with corresponding CORESETs with different TCI states, hence, achieving beam-diversity for PDCCH transmission. In Rel-17, only intra-slot PDCCH repetition is supported, and also, PDCCH repetition is only supported for USS or Type3 CSS. In addition, the linkage is specified at the PDCCH candidate level by restricting configurations of two linked search space sets resulting in one-to-one mapping between monitoring occasions and between PDCCH candidates of the two linked search space sets. Two linked PDCCH candidates have the same aggregation level, same coded bits, and the same DCI payload. To avoid ambiguity at the UE, a reference PDCCH candidate is defined for various procedures such as timelines, PUCCH resource determination, PDSCH reception with mapping Type B or mapping Type A, determination of QCL assumption for PDSCH when TCI field is not present in DCI, etc. UE can report whether two blind decodes or three blind decodes are needed for two linked PDCCH candidates. In the case of three blind decodes, overbooking for PDCCH is enhanced accordingly. Furthermore, determination of two QCL-TypeD is specified for FR2 to support time-overlapping PDCCH repetitions. PDCCH repetition is supported also for cross-carrier scheduling through linking two search space sets in both scheduling cell and scheduled cell.

PDCCH repetitions are part of the Rel-17 Unified TCI work and should be enabled with requirements in RAN4. Multi Rx is not working on unified TCI framework and is basing the work on Rel 16 framework. Considering that, it is natural to assume that the requirements related to features related to Rel 17 unified framework should be defined as part of the MIMO evolution work.

PDCCH repetition and SFN use cases should be considered for extension of Rel-17 unified TCI framework.

Conclusion

In this paper we addressed the issues regarding Unified TCI framework extension to mTRP for MIMO EVO. The following observations and conclusions were made.

Observation 1: Both intra-cell and inter-cell operations as feasible for Rel-17 group based beam reporting.

Observation 2: Since group based beam reporting is feasible for inter-cell mTRP, support in RAN4 that the inter-cell mTRP scenario applies to simultaneous reception based mTRP scheme

Proposal 1: RAN4 to define requirements for simultaneous reception in Rel-18 including intra-cell and inter-cell mTRP scenario.

Observation 3: TCI switching commands for mTRP are different for sDCI and mDCI.

Proposal 2: Support option 3: RAN4 to define different requirements for sDCI and mDCI scenarios.

Proposal 3: TCI switching requirements for sDCI and for mDCI to include support of both configurations of joint or separate frameworks.

Proposal 4: Postpone discussion on TRP-specific BFR requirements.

Proposal 5: RAN4 to discuss requirements for STxMP MAC CE TCI switching requirements when target TCI state includes 2 TCIs.

Proposal 6: RAN4 to discuss requirements for STxMP DCI TCI switching requirements when target TCI state includes 2 TCIs.

Proposal 7: RAN4 to consider TCI switch delay that include timing reference signal per TCI state.

Proposal 8: PDCCH repetition and SFN use cases should be considered for extension of Rel-17 unified TCI framework.

- References

- R4-2306362, WF on R18 NR MIMO RRM requirement, Samsung, 17 April –26 April, 2023

- RP-220802, WI summary for WI Core part: Further enhancements on MIMO for NR, Samsung, March 17-23, 2022