3GPP TSG-RAN WG4 Meeting #107 	R4-2307190

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on receive timing difference between different directions

Agenda item:	8.8.3.6

Document for:	Discussion

Introduction

In this paper we discuss MRTD aspects related to the multi-RX chain reception in FR2 for Rel-18. Based on analysis and observations and proposals are derived concerning inter-TRP distance requirements considering MRTD to be within or equal to Cyclic Prefix (CP) length.

Discussion

In RAN4#106-e-bis meeting, the following WF were made regarding Maximum Receive Time Difference [2][1].

Issue 3-1-2: Others Way forward: Please bring further analysis on the below proposal. - Proposal 1: RAN4 to study impact of receive timing differences from mTRPs on beam pair selection for group-based beam reporting

MRTD requirements have direct influence on the mTRP network deployment. MRTD requirements are designed with direct consideration of the network deployment, including synchronization of two cells and maximum propagation delay. Most of the MRTD requirements are currently specified for carrier aggregation and dual connectivity scenarios, and they relate the timing differences that the UE must support to be able to receive from two component carriers, or two cells.

During RAN4 #106 it was decided that RAN4 would define requirements for MRTD<CP, and that the case of MRTD>CP could be discussed after RAN4#107 [3]:

## MRTD Handling - Agreements in online session: - Define RRM requirements for MRTD < CP - FFS whether to support MRTD > CP and the discussion can take place after RAN4 #107 - Note: the MRTD above is the maximum receive time difference between the signals coming from two directions on the same carrier

The range of MRTD agreed in RAN4#106 as a baseline is much smaller than the existing MRTD requirements used for carrier aggregation and dual connectivity. The existing networks are planned with a maximum inter site distance that is calculated based on the existing CA and DC MRTD requirements. As a result, if multiRx feature is deployed in existing sites, the experienced receive time difference of a UE would potentially exceed the MRTD<CP requirements for most of the coverage area.

Existing network deployments are designed with maximum inter-site distance that is limited by CA/DC MRTD requirements.

If the MRTD is smaller than the maximum propagation delay difference the UE may experience in a cell, there is no way for the network to determine if MRTD is exceeded or not.

Group-based beam reporting for beam pairs/groups from multiple TRPs is introduced in Rel-17. In L1 measurement delay agenda item, it was discussed that timing differences between beam pair RSs could have impact on the beams that are selected for reporting in group-based beam reporting.

A multi-Rx UE that is configured to simultaneously receive from multiple TRPs needs to be able to select the best set of downlink beam pairs. It was discussed in the L1 measurement delay agenda item that conditions for selection of best beam pairs in Group Based Beam Reporting need to be defined.

There is a maximum receive time difference between beam pair RSs that a multi-Rx UE can support to achieve optimum DL reception in multi-TRP scenarios.

Considering the challenges above related to MRTD, it would be desirable that the UE only reports beam pairs in GBBR for which MRTD condition is met. Consider the example where the UE is configured to report 1 beam pair related to 3 TRPs, TRP0, TRP1, and TRP2, where all RSs on Set 1 are related to TRP0, and Set 2 contains RSs from TRP1 and TRP2. Consider also that the RTD between TRP0 and TRP1 exceeds the MRTD limit, whereas the RTD from TRP0 and TRP2 doesn’t. If in this situation the RSRP for TRP1 is larger than the RSRP for TRP2, the UE could potentially report that the best beam pair is composed of reference signals from TRP0+TRP1. In that situation the network would proceed with activating and indicating TCI state pairs related to TRP0+TRP1, and the UE would experience a very high block error rate. In that error situation the network would attempt again to configure the UE to report GBBR in order to find a new suitable beam. That example shows how slow the setup of multi TRP operation would be if the UE is reporting beam pairs for which it cannot really be able to receive data due to exceeding the MRTD limit. Therefore, it is reasonable to assume that GBBR should include conditions for beam reporting taking into consideration the receive time difference between the reported reference signals.

For mTRP GBBR, UE should select Beam pair RSs that have relative receive time difference not exceeding the UE supported maximum receive time difference.

Conclusion

In this paper, analysis of impact of Receive Time difference between RSs in a beam pair in multi-TRP scenarios was done, considering MRTD to be up to CP length:

Observation 1: Existing network deployments are designed with maximum inter-site distance that is limited by CA/DC MRTD requirements.

Observation 2: If the MRTD is smaller than the maximum propagation delay difference the UE may experience in a cell, there is no way for the network to determine if MRTD is exceeded or not.

Observation 3: There is a maximum receive time difference between beam pair RSs that a multi-Rx UE can support to achieve optimum DL reception in multi-TRP scenarios.

Proposal 1: For mTRP GBBR, UE should select Beam pair RSs that have relative receive time difference not exceeding the UE supported maximum receive time difference.

- References

- R4-2306234    Topic summary for [106-bis-e][203] FR2_multiRx_part2.

- R4-2306319    WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 2)

- R4-2303317, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 2)