3GPP TSG-RAN WG4 Meeting #107 	R4-2307192

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on interRAT measurements without gaps

Agenda item:	8.10.3.2

Document for:	Discussion

Introduction

In this paper it is discussed issues related to gap-less interRAT measurements. As part of this discussion, we analyse scheduling restriction, and interruption related requirements, and provide Nokia’s views on the open issues related to that topic.

Discussion

In this paper we discuss scenarios for inter-RAT measurement without gap as described below [1]:

# Topic #2: inter-RAT measurement without gap(AI 5.10.3.2) [Moderator notes: Up to this meeting, all agreed using scenarios for inter-RAT NR/LTE measurements without gap can summarized as: - the inter-RAT NR measurements without gap in Rel18 includes the two scenarios below. - Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements - Case a-2: NR reference signal to be measured are fully contained within UE’s LTE channel bandwidth - the inter-RAT LTE measurements without gap in Rel18 includes the two scenarios below. - Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements - Case b-2: LTE CRS are fully contained within UE’s active BWP ]

Scenarios for case b-2

The following was discussed regarding scenarios for case b-2 [1]:

- Issue 2-1-1: Differentiate scenarios for Case b-2 < Way forward/Agreement >: - FFS on whether to differentiate the following scenarios in case b-2. - Scenario 1: Inter-RAT LTE measurement for LTE CRS rate-matching feature - Scenario 2: Inter-RAT LTE measurement for CRS-IM receiver

Regarding the scenarios for case b-2 it was proposed to differentiate the requirements depending on whether the UE supports CRS-IM or LTE  CRS rate-matching. CRS rate-matching and CRS-IM are used for mitigating the interference of LTE reference signals over the NR data transmission. In the case of CRS rate-matching the network punctures the REs used for CRS in order to avoid the interference. In the case of CRS-IM the UE is estimating interference caused by LTE CRS signals over NR. In order to do so a UE would typically estimate CRS interference over gaps in a first step, to after cancel out that interference over NR PDSCH transmissions. In our view it there might be UEs capable of performing gapless measurements while using CRS-IM. Therefore, we think that we should either consider that the same requirements apply for CRS-IM and rate matching, otherwise there would be much larger standardization effort in order to specify that feature. One alternative is that we consider only CRS rate matching for case b-2.

RAN4 to discuss among the following options for case b-2:

  - Option 1: Both CRS rate matching and CRS-IM have the same requirements

  - Option 2: Gapless measurements for Case b-2 are only allowed with CRS rate matching

UE capabilities

The following was discussed regarding UE capabilities [1]:

- Issue 2-2-2: UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2) < Way forward >: - A new per-UE capability to support Case b-2 should be defined. - FFS on indication of “no gap with interruption” because of AGC,T/F offset issue

From RAN4 #106-bis [1]:

Issue 2-6-1: Interruption requirements for inter-RAT measurement without gap < Way forward/Agreement >: - Option 1: - interruption requirements can be defined - for case a-1 and case b-1, - Option 1a: - interruption requirements can be defined - for case a-1 and case b-1, - FFS for case b-2 - Option 1b: - Interruption requirements can be defined - for inter-RAT NR measurements without gap(case a-1)

For the case b-2 the CRS is fully contained within the UE active BWP in NR. For that reason, there is no retuning needed when performing interRat measurements for case b-2. Typically interruption requirements are justified by the UE requiring to retune to a new frequency in order to perform the measurements. If the CRS is in contained within the active BWP for NR this step is not necessary, since the UE is already tuned to the center frequency of the NR BWP. Therefore, there is no technical reason that justify the need for interruption for case b-2.

In case b-2 the LTE CRS are fully contained within UE’s active BWP.

No interruption is expected for measurements within the UE’s active BWP since retuning is not needed.

Do not consider interruption for case b-2

Measurement reporting period requirements

From RAN4 #106-bis [1]:

- Issue 2-4-2: Measurement period requirements for case a-1 with “no gap but interruption allowed” < Way forward/Agreement >: - FFS on measurement period requirements for case a-1 with “no gap but interruption allowed” - Issue 2-4-3: Measurement period requirements for case a-1 with “no gap no interruption” < Way forward/Agreement >: - FFS on measurement period requirements for case a-1 with “no gap no interruption”

The measurement period requirements for case a-1 has similarities to the interrat measurements without gaps. As part of the needForGaps discussion in this WID RAN4 is discussing the same type of measurement delays.

Measurement period requirements for case a-1 have relationship to measurement period discussed for NR measurements without gaps.

Wait for further progress on measurement requirements for NR measurements without gaps before discussing measurement period requirements for case a-1.

Scheduling restriction

From RAN4 #106-bis [1]:

- Issue 2-5-2: Scheduling restriction for inter-RAT LTE measurements < Way forward/Agreement >: - Scheduling restriction due to inter-RAT LTE measurement are applicable when - UE does not support simultaneous Tx and Rx on the serving cell and target band - FFS on Serving cell and target MO have mixed SCS and they are in the same band

During previous meetings scheduling restrictions were discussed for the inter RAT cases, where scheduling restrictions due to LTE measurements are still open. In the current state of the specification, requirements without gaps include scheduling restrictions for NR measurements around symbols to be measured for reasons such as: different numerology used for SSB and data; the UE doesn’t support simultaneousRxTxInterBandCA and therefore UL transmission is precluded around the SSB symbols to be measured.

Even though similar UE capabilities could be considered for the definition of scheduling restrictions for LTE measurements, there is a big difference on the measurement procedure  of LTE signals in comparison to NR. In NR the measurements are based on SSB or CSI-RS, which have a very well defined location, e.g. SMTC can be configured for every 40 ms in a window of 5 ms. On the other hand, LTE measurements are performed based on CRS, which are transmitted in every subframe of a ratio frame. In the example of Figure 1 CRS is transmitted in 4 symbols out of 14 symbols. If the same approach as in NR is used where 1 symbol after and before are subject to scheduling restrictions, only 2 out of the 14 symbols in a 1ms subframe would be available to be scheduled.

Considering the above, it is very inefficient to perform LTE measurements without gaps if scheduling restrictions have to be applied around CRS. In the example below, the scheduling restrictions would basically consume most of the throughput available for the UE, and performing measurements with gaps would be much more efficient.

As a conclusion, we understand that it only makes sense to perform LTE interRAT measurements without gaps for UEs that are capable of performing those without scheduling restrictions. Therefore it can be assumed that those UEs are also capable of measuring CRS and receiving data with different numerologies, and the UE should be also able to support simultaneousRxTxInterBandCA.

Figure 1 Cell specific reference signal

LTE measurements are based on CRS, which are transmitted in every DL slot.

Scheduling restrictions around CRS would be applicable entire radio frame, and would results in only 2 out of the 14 symbols of a subframe to be available for scheduling.

For cases b1 and b2 it is expected that the UE performs LTE measurements with a spare RF chain.

For cases b1 and b2 the UEs are already expected to be using either rate matching or CRS-IM in order to mitigate LTE CRS interference.

RAN4 to consider that UEs reporting no-gap for cases b1 and b2 support simultaneousRxTxInterBandCA and simultaneousRxDataSSB-DiffNumerology/ simultaneousRxDataSSB-DiffNumerology-inter.

RAN4 to define requirements for cases b1 and b2 support with no scheduling restrictions.

Conclusion

In this paper we have discussed Nokia’s views on interRAT measurements without gaps. As part of this discussion, the following Observations and proposals are derived:

Proposal 1: RAN4 to discuss among the following options for case b-2:

a.	Option 1: Both CRS rate matching and CRS-IM have the same requirements

b.	Option 2: Gapless measurements for Case b-2 are only allowed with CRS rate matching

Observation 1: In case b-2 the LTE CRS are fully contained within UE’s active BWP.

Observation 2: No interruption is expected for measurements within the UE’s active BWP since retuning is not needed.

Proposal 2: Do not consider interruption for case b-2

Observation 3: Measurement period requirements for case a-1 have relationship to measurement period discussed for NR measurements without gaps.

Proposal 3: Wait for further progress on measurement requirements for NR measurements without gaps before discussing measurement period requirements for case a-1.

Observation 5: Scheduling restrictions around CRS would be applicable entire radio frame, and would results in only 2 out of the 14 symbols of a subframe to be available for scheduling.

Observation 7: For cases b1 and b2 the UEs are already expected to be using either rate matching or CRS-IM in order to mitigate LTE CRS interference.

Proposal 4: RAN4 to consider that UEs reporting no-gap for cases b1 and b2 support simultaneousRxTxInterBandCA and simultaneousRxDataSSB-DiffNumerology/ simultaneousRxDataSSB-DiffNumerology-inter.

Proposal 5: RAN4 to define requirements for cases b1 and b2 support with no scheduling restrictions.

- References

- R4-2306331, WF on NR_MG_enh2 Part 2, Intel Corporation

- R4-2303198, WF on Rel-18 NR measurements without gaps, Intel Corporation