3GPP TSG-RAN WG4 Meeting #105	R4-2307193

Toulouse, FR, November 14 – November 18, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on performance requirements for SDT

Agenda item:	6.8.3

Document for:	Discussion

Introduction

This paper presents Nokia’s view on RRM performance requirements for SDT, specifically on the testing of SDT and updates Nokia’s view from RAN4 #106.

Discussion

This paper discusses the open issues related to the test cases for CG-SDT.

Discussion of issues

Issue 6-2-1: Test procedures - Proposals - Proposal: - Send a new test mode command with new timer and data just before the RRC release, similar to test point TC - RAN4 to consider that CG-SDT test starts in RRC_Connected state, i.e. before test point TA - RAN4 to define test times for CG-SDT test case with that TB is W1 before TC and TA 1 second before TB - Design the test case to have W2 at 320ms for FR1 and 480ms for FR2-1 - RAN4 to configure CG-SDT test cases using DRX cycle of 320 ms - Suggest to R5 to change the T_delay_modeB to be in the granularity of ms. If so, the TE can calculate forwards from TC instead of calculating backwards from TG all the way back to TC - RAN4 to define requirements such the T_delay_modeB timeout happens just after a CG-SDT resource - RAN4 to define test procedure with that SFN offset can be configured freely by the test equipment to determine by the time of sending RRCRelease w/ CG-SDT configuration - Define T3 as a range of 1 time unit of T_delay_modeB: 2.36s<=T3<=3.36s - RAN4 to discuss common test description for CG-SDT in FR1 and FR2 - Design the test case to have W2 at 320ms for FR1 and 480ms for FR2-1 - Recommended WF - TBA Issue 6-2-2: Power levels - Proposals - Proposal: - Update the definition of P0 as P0 = P2 - cg-SDT-ChangeThreshold – Pmargin, and adopt Figure 2 in the spec - Take 4dB and 6.5dB for Pmargin in FR1 and FR2 TCs - Use the following power values in the TCs - P0 = P3 = -100dBm - P1 = -92dBm and -87dBm for FR1 and FR2 - P2 = -88dBm and -85.5dBm for FR1 and FR2 - Recommended WF - TBA Issue 6-2-3: Time points - Proposals - Proposal: - Leave the UL data trigger for the second subtest to RAN5, and RAN4 only to specify that UE is triggered to transmit UL data at TJ - TH = TG + 500ms - TJ is the first possible data trigger point from (W1+W2) after TH - Recommended WF - TBA

Looking at R17 TS 38.133 in current version (v17.9.0), it is seen that the test parameter tables of sections A.6.2.1 are still not complete for FR1. Same for the tables for FR2 in A.7.2.1.

The test parameter tables of R17 TS 38.133 sections A.6.2.1 and A.7.2.1 are incomplete.

For FR1, W1 is given as min(640ms, M1*TDRX), same as for W2, which is min(640ms, 1*320ms) = 320ms.

For FR2, W1 is given as max(480ms, 8*SMTC periodicity), same as for W2, which is max(480, 8*20ms) = 480ms.

Assuming the UE test loop mode B is closed very close to the RRC release, the time from TC to TF is given as an integer number of seconds. Therefore, whether FR1 or FR2 measurement windows are within the TC to TF period, does not change the fact it needs to be > 1s, at least, and leaving at least 2 measurement periods for both RSRP1 and RSRP2 (in case of errors in UE implementation) in T3, T3 ≥ 2*(W1 + W2), meaning T3 ≥ 2s for both FR1 and FR2 testing.

To ensure the right TE, T3 shall be defined as (T_delay_modeB - W2 - W1). T_delay_modeB shall at least be the closest integer to hold the above minimum value for T3 + W1 + W2. This gives T_delay_modeB ≥ 3s for both FR1 and FR2 testing.

T_delay_modeB = 3s for both FR1 and FR2 testing.

T3 = T_delay_modeB - W1 - W2. This means 2.36s for FR1 and 2.04s for FR2.

T1 should be at least 2 measurement periods as well, so T1 = 1s will work for both FR1 and FR2.

T1 > 2*measurement period ≈ 1s for FR1 and same for FR2.

T2 shall be defined as : T2 = 2*W1.

T2 = 2*W1 = 640ms for FR1 and 960ms for FR2.

T4 is more difficult to establish. If the first CG-SDT transmission fails, how should the UE receive a new test command and an RRC release? In this case, what solution should be implemented in the test to solve this problem as the test is not failed until it is established whether the test requirement is fulfilled, more than 90% of TA validations correct, or not.

In case of a successful CG-SDT transmission, T4 can be as short as (2*W1 + W2 + W3 + a margin for additional SDT signaling). If not, there must be a mitigation to make signaling possible, which must be added as additional margin for potentially even enter connected mode for the signaling to happen. It is clear that T4 > (2*320ms + 320ms + 640ms + margin) which implies T4 > (1.6s + margin) for FR1 and T4 > (2.1s + margin) for FR2. Therefore, a value of 4s is suggested.

T4 = 4s for FR1 and same for FR2.

The discussion for T5 is similar to the discussion for T3, except that there must be time after TK to check for erroneous transmission and it must include W2 + W3 + a margin compared to T3. The margin in this case shall be larger than twice of CG-SDT resource period. So, T5 = T3 + W2 + W3 + 2*CG-SDT resource period.

T5 = T3 + W2 + W3 + 2*CG-SDT resource period = 2.36s + 0.32s + 0.64s + 2*0.64s = 4.6s for FR1 and 3.24s for FR2.

For FR1 and FR2, there is a very large difference in the values used for the CG-SDT period. As we cannot predict which CG-SDT resource is chosen for transmission of the device, there is no longer a reason for selecting large values.

For this reason, we suggest to align the values between the two tests.

Change the FR1 CG-SDT resource period to align with FR2 test to 40ms

It is suggested to update the tables of 38.133, tables A.6.2.1.2-2 and A.7.2.1.1.1-2 for T_delay_modeB, T1, T2, T3, T4 and T5 as in the following table.

|  | Formula | FR1 | FR2 | Unit |
| --- | --- | --- | --- | --- |
| W1 |  | 320 | 480 | ms |
| W2 |  | 320 | 480 | - |
| W3 |  | 640 | 640 | - |
| T_delay_modeB |  | 3000 | 3000 | - |
| CG-SDT ressource period |  | 40 | 40 | - |
| T1 | 2xmeasurement period | 1000 | 1000 | - |
| T2 | 2xW1 | 640 | 960 | - |
| T3 | T_timer_modeB-W1-W2 | 2360 | 2040 | - |
| T4 |  | 4000 | 4000 | - |
| T5 | T3+W2+W3+2xCG-SDT ressource period | 3400 | 3240 | - |
| T1+T2+T3+T4+T5 |  | 11400 | 11240 | - |

Table 1: Suggestion for T_delay_modeB and calculation of T1, T2, T3, T4 and T5 based on W1, W2, W3 and CG-SDT period already defined.

Conclusion

This paper has presented Nokia’s view on the missing RRM performance test parameter values for SDT TA testing. As part of this discussion, we have reached the following observations and proposals:

Observation 1: The test parameter tables of R17 TS 38.133 sections A.6.2.1 and A.7.2.1 are incomplete.

Proposal 1: T_delay_modeB = 3s for both FR1 and FR2 testing.

Proposal 2: T3 = T_delay_modeB - W1 - W2. This means 2.36s for FR1 and 2.04s for FR2.

Proposal 3: T1 > 2*measurement period ≈ 1s for FR1 and same for FR2.

Proposal 4: T2 = 2*W1 = 640ms for FR1 and 960ms for FR2.

Proposal 5: T4 = 4s for FR1 and same for FR2.

Proposal 6: T5 = T3 + W2 + W3 + 2*CG-SDT resource period = 2.36s + 0.32s + 0.64s + 2*0.64s = 4.6s for FR1 and 3.24s for FR2.

Proposal 7: Change the FR1 CG-SDT resource period to align with FR2 test to 40ms

Proposal 8: It is suggested to update the tables of 38.133, tables A.6.2.1.2-2 and A.7.2.1.1.1-2 for T_delay_modeB, T1, T2, T3, T4 and T5 as in the following table:

|  | Formula | FR1 | FR2 | Unit |
| --- | --- | --- | --- | --- |
| W1 |  | 320 | 480 | ms |
| W2 |  | 320 | 480 | - |
| W3 |  | 640 | 640 | - |
| T_delay_modeB |  | 3000 | 3000 | - |
| CG-SDT ressource period |  | 40 | 40 | - |
| T1 | 2xmeasurement period | 1000 | 1000 | - |
| T2 | 2xW1 | 640 | 960 | - |
| T3 | T_timer_modeB-W1-W2 | 2360 | 2040 | - |
| T4 |  | 4000 | 4000 | - |
| T5 | T3+W2+W3+2xCG-SDT ressource period | 3400 | 3240 | - |
| T1+T2+T3+T4+T5 |  | 11400 | 11240 | - |

- References

- RP-222364, Updated Work Item on NR small data transmissions in INACTIVE state.

- R4-2302761, Topic summary for [106][203] R17_maintenance_RRM, Intel Corporation