3GPP TSG-RAN WG4 Meeting #107 	R4-2307196

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on PUSCH demodulation requirements for the extension to 71 GHz

Agenda item:	5.2.9.6.2

Document for:	Discussion

Introduction

This paper presents discussion on remaining issues and PUSCH simulation results for the definition of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Discussion

In the previous RAN4 meeting the MCS for 2x2 cases and 16 QAM was reviewed according to the following agreement [1]:

MCS and number Tx/Rx branches for 16 QAM PUSCH requirements <Agreement> - For 1x2 Low, use TDLA30-650 for 100MHz CBW, TDLA10-650 for 400MHz CBW. - For 2x2 Low, use TDLA30-200 for 100MHz CBW, TDLA10-200 for 400MHz CBW - Consider MCS16 for the requirements - Update requirements on CRs with - TDLA30-650 - 120 kHz and 100 MHz– add requirements in [] - TDLA10-650 - 120 kHz and 400 MHz – add requirements in [] - TDLA10-650 - 480 kHz and 400 MHz – add requirements in [] - Bring simulation results next meeting for - MCS16 - TDLA30-200 - 120 kHz and 100 MHz - TDLA10-200 - 120 kHz and 400 MHz - TDLA10-200 - 480 kHz and 400 MHz - MCS12 - TDLA30-650 - 120 kHz and 100 MHz - TDLA10-650 - 120 kHz and 400 MHz - TDLA10-650 - 480 kHz and 400 MHz - Decide between MCS12 and MCS16 for 2x2 requirements based on the results provided

As part of this discussion, it was open whether to use MCS 12 or MCS 16 for the 2x2 requirements. Based on our simulation results shown in the annex of this paper, MCS12 has about 4 to 5 dB lower SNR than MCS 16. For this reason, since the margin to the testable SNR limit for this MCS is larger it is fine to agree to use MCS 12.

Define PUSCH requirements for 2x2 Low MCS12 using use TDLA30-200 for 100MHz CBW, TDLA10-200 for 400MHz CBW.

Conclusion

In the paper, the following Observations and Proposals were made:

Proposal 1: Define PUSCH requirements for 2x2 Low MCS12 using use TDLA30-200 for 100MHz CBW, TDLA10-200 for 400MHz CBW

- References

- R4-2302971, WF on BS demodulation requirements for FR2-2, Nokia, Nokia Shanghai Bell, RAN4 #106

- R4-2217372, WF on PUSCH demodulation requirements for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2217371, WF on general aspects for FR2-2 demodulation requirements, Huawei,HiSilicon

- R4-2210664, WF on general and BS aspects for FR2-2 demodulation requirements, Intel Corporation

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation

Annex: simulation results

Simulation assumptions are shown below and are derived considering the agreements on the WF [1].

Table 1 Common simulation parameters

| Parameter | Value |
| --- | --- |
| Transform precoding | Disabled and enabled |
| CP | Normal CP |
| Carrier frequency | 70 GHz |
| DMRS symbols | [1 9] |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| SCS | 120, 480, and 960 kHz |
| TDD pattern | 120 kHz: 3D1S1U, S=10D:2G:2U 480 kHz: 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U 960 kHz: 29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U |
| DMRS type | Type 1 with single-symbol DM-RS (l0=2) |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups with data | 2 |
| Symbol length | 10 |
| Start symbol index | 0 |
| PRTS | On |
| HARQ | ON |
| Time domain resource allocation type | type B |
| Frequency domain resource without transform precoding | For 120 kHz SCS and - CHW= 100 MHz: 66 - CHW= 400 MHz: 264 For 480 kHz SCS and - CHW= 400 MHz: 66 |
| Frequency domain resource without transform precoding | For 120 kHz SCS and - CHW= 100 MHz: 64 For 480 kHz SCS and - CHW= 400 MHz:64 |
| MCS index | MCS 4, 16, 18, 20 |
| Propagation condition | TDLA30-200 TDLA30-650 TDLD30-200 TDLA10-200 TDLA10-650 TDLD10-200 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz; CBW = 400 MHz SCS = 480 kHz; CBW = 400 MHz |
| Phase noise model | 38808 Set2 |
| Number of HARQ transmissions | 4 |
| RV sequence | {0.2.3.1} |
| Testing metric | SNR at 70% of the maximum throughput |
| Phase noise compensation | CPE only |

Note: Simulation results are being updated in accordance to the agreed WF. Previous simulation results are shown here for information.

Table 2 Simulation results with transform precoding disabled

| SCS | CBW | MCS | Channel model | Antenna configuration | SNR @ 70% TPUT |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Ideal | Impair |
| 120 | 100 | 12 | TDLA30-650 | 2x2 | 10,5 | 13,8 |
| 120 | 100 | 16 | TDLA30-650 | 2x2 | 15,3 | 18,6 |
| 120 | 100 | 16 | TDLA30-200 | 2x2 | 14,6 | 17,9 |
| 120 | 100 | 16 | TDLD30-200 | 2x2 | 10,36 | 13,9 |
| 120 | 400 | 12 | TDLA10-650 | 2x2 | 10,8 | 14,1 |
| 120 | 400 | 16 | TDLA10-650 | 2x2 | 16,8 | 20,1 |
| 120 | 400 | 16 | TDLA10-200 | 2x2 | 16,1 | 19,4 |
| 120 | 400 | 16 | TDLD10-200 | 2x2 | 10,36 | 13,9 |
| 480 | 400 | 12 | TDLA10-650 | 2x2 | 10,0 | 13,3 |
| 480 | 400 | 16 | TDLA10-650 | 2x2 | 15,1 | 18,4 |
| 480 | 400 | 16 | TDLA10-200 | 2x2 | 14,5 | 17,8 |
| 480 | 400 | 16 | TDLD10-200 | 2x2 | 10,19 | 13,7 |