3GPP TSG-RAN4 Meeting #107	R4-2307206

# Incheon, Korea, May 22nd – May 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 1498 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance CR to RF requirements for n263 (Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2023-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There is a Note for max EIRP on band n263 indicating the requirment is average EIRP, which seems implying max EIRP for other bands are not average EIRP. Based on the analysis in R4-2307205, max EIRP is average power on all the bands, therefore the note could be removed. - In some test cases the requirements are specified for FR2-1 and FR2-2 respectively, while some table titles only mentioned ‘FR2’, which might be interpreted as applying to both FR2-1 and FR2-2. Some cleaning up is needed. - There are mistakes in the measurement period of PRACH time mask. - The Additional spurious emission requirement for NS_204 is still missing. R4-2307205 provides the proposal of specific requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removing Note for Max EIRP on band n263. - Updating table titles to clarify the requirements only apply to FR2-1. - Correcting measurement period of PRACH time mask. - Adding Additional spurious emission requirement for NS_204 based on analysis in R4-2307205. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for n263 are incorrect or incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1, 6.2.1.3, 6.2.2.1, 6.3.3.4, 6.4.2.1, 6.4.2.2.5, 6.4.2.2.6, 6.5.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2.1.1	UE maximum output power for power class 1

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.1-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1-1: UE minimum peak EIRP for power class 1

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 40.0 |
| n258 | 40.0 |
| n260 | 38.0 |
| n261 | 40.0 |
| n262 | 34.2 |
| n263 | 30.6 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.1-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1-2: UE maximum output power limits for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 35 | 55 |
| n258 | 35 | 55 |
| n260 | 35 | 55 |
| n261 | 35 | 55 |
| n262 | 35 | 55 |
| n263 | 25 | 40 |

The minimum EIRP at the 85th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.1-3 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2.1.1-3: UE spherical coverage for power class 1

| Operating band | Min EIRP at 85 %-tile CDF (dBm) |
| --- | --- |
| n257 | 32.0 |
| n258 | 32.0 |
| n260 | 30.0 |
| n261 | 32.0 |
| n262 | 26.0 |
| n263 | 19.1 |
| NOTE 1: Minimum EIRP at 85 %-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. |  |

6.2.1.2	UE maximum output power for power class 2

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.2-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.2-1: UE minimum peak EIRP for power class 2

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 29 |
| n258 | 29 |
| n259 | 25 |
| n261 | 29 |
| n262 | 22.9 |
| n263 | 22.7 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.2-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.2-2: UE maximum output power limits for power class 2

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n259 | 23 | 43 |
| n261 | 23 | 43 |
| n262 | 23 | 43 |
| n263 | 23 | 43 |

The minimum EIRP at the 60th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.2-3 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2.1.2-3: UE spherical coverage for power class 2

| Operating band | Min EIRP at 60 %-tile CDF (dBm) |
| --- | --- |
| n257 | 18.0 |
| n258 | 18.0 |
| n259 | 12.5 |
| n261 | 18.0 |
| n262 | 11.0 |
| n263 | 7.6 |
| NOTE 1: Minimum EIRP at 60 %-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. |  |

6.2.1.3	UE maximum output power for power class 3

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.3-1. The requirement is verified with the test metric of total component of EIRP (Link=TX beam peak direction, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 6.2.1.3-1. The requirement for the UE which supports multiple FR2 bands is specified in both Table 6.2.1.3-1 and Table 6.2.1.3-4.

Table 6.2.1.3-1: UE minimum peak EIRP for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n259 | 18.7 |
| n260 | 20.6 |
| n261 | 22.4 |
| n262 | 16.0 |
| n263 | 14.1 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Void |  |

The maximum output power values for TRP and EIRP are found on the Table 6.2.1.3-2. The max allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and the total component of EIRP (Link=TX beam peak direction, Meas=Link angle.

Table 6.2.1.3-2: UE maximum output power limits for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Max EIRP (dBm/MHz) | Notes |
| --- | --- | --- | --- | --- |
| n257 | 23 | 43 |  |  |
| n258 | 23 | 43 |  |  |
| n259 | 23 | 43 |  |  |
| n260 | 23 | 43 |  |  |
| n261 | 23 | 43 |  |  |
| n262 | 23 | 43 |  |  |
| n263 | 25 | 43 |  | Default for “NS_200” |
|  | 25 | 40 | 23 | Applies when “NS_204” is indicated in the cell |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.3-3 below. The requirement is verified with the test metric of the total component of EIRP (Link=Beam peak search grids, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 6.2.1.3-3. The requirement for the UE which supports multiple FR2 bands is specified in both Table 6.2.1.3-3 and Table 6.2.1.3-4.

<Unchanged Text Skipped>

6.2.2.1	UE maximum output power reduction for power class 1

For power class 1, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB ≤ 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in table 5.3.2-1, and waveform types. MPRWT is defined in Tables 6.2.2.1-1 and 6.2.2.1-2 for FR2-1 and in Tables 6.2.2.1-3 and 6.2.2.1-4 for FR2-2.

Table 6.2.2.1-1 MPRWT for power class 1, BWchannel ≤ 200 MHz in FR2-1

| Modulation |  | MPRWT (dB), BWchannel ≤ 200 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.0 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 7.0 | ≤ 5.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |

Table 6.2.2.1-2 MPRWT for power class 1, BWchannel = 400 MHz in FR2-1

| Modulation |  | MPRWT (dB), BWchannel = 400 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 | ≤ 9.0 |

Table 6.2.2.1-3 MPRWT for power class 1, BWchannel = 100 MHz in FR2-2

| Modulation |  | MPRWT (dB), BWchannel = 100 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.5 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 7.0 | ≤ 2.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 8.0 | ≤ 8.0 |
| CP-OFDM | QPSK | ≤ 8.0 | ≤ 1.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 3.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 9.5 | ≤ 9.5 |

Table 6.2.2.1-4 MPRWT for power class 1, BWchannel >= 400 MHz in FR2-2

| Modulation |  | MPRWT (dB), BWchannel = 400, 800, 1600, 2000 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6.0 | ≤ 1.0 | ≤ 3.5 |
|  | QPSK | ≤ 6.0 | ≤ 1.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 3.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 8.0 | ≤ 8.0 | ≤ 8.0 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 1.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

<Unchanged Text Skipped>

6.3.3.4	PRACH time mask

The PRACH ON power is specified as the mean power over the PRACH measurement period excluding any transient periods as shown in Figure 6.3.3.4-1. The measurement period for different PRACH preamble format is specified in Table 6.3.3.4-1.

Table 6.3.3.4-1: PRACH ON power measurement period

| Format | SCS | Measurement period | Note |
| --- | --- | --- | --- |
| A1 | 60 kHz | 0.035677 ms |  |
|  | 120 kHz | 0.017839 ms |  |
|  | 480 kHz | 0.004460 ms |  |
|  | 960 kHz | 0.002230 ms |  |
| A2 | 60 kHz | 0.071354 ms |  |
|  | 120 kHz | 0.035677 ms |  |
|  | 480 kHz | 0.008919 ms |  |
|  | 960 kHz | 0.004460 ms |  |
| A3 | 60 kHz | 0.107031 ms |  |
|  | 120 kHz | 0.053516 ms |  |
|  | 480 kHz | 0.013379 ms |  |
|  | 960 kHz | 0.006690 ms |  |
| B1 | 60 kHz | 0.035091 ms |  |
|  | 120 kHz | 0.0175455 ms |  |
|  | 480 kHz | 0.004386 ms |  |
|  | 960 kHz | 0.002193 ms |  |
| B4 | 60 kHz | 0.207617 ms |  |
|  | 120 kHz | 0.103809 ms |  |
|  | 480 kHz | 0.025952 ms |  |
|  | 960 kHz | 0.012976 ms |  |
| A1/B1 | 60 kHz | 0.035677 ms for front X1 occasion 0.035091 ms for last occasion | X1 = [2,5] |
|  | 120 kHz | 0.017839 ms for front X1occasion 0.017546 ms for last occasion |  |
|  | 480 kHz | 0.004460 ms for front X1 occasion 0.004386 ms for last occasion |  |
|  | 960 kHz | 0.002230 ms for front X1occasion 0.002193 ms for last occasion |  |
| A2/B2 | 60 kHz | 0.071354 ms for front X2 occasion 0.069596 ms for last occasion | X2 = [1,2] |
|  | 120 kHz | 0.035677 ms for front X2 occasion 0.034798 ms for last occasion |  |
|  | 480 kHz | 0.008919 ms for front X2 occasion 0.008700 ms for last occasion |  |
|  | 960 kHz | 0.004460 ms for front X2 occasion 0.004350 ms for last occasion |  |
| A3/B3 | 60 kHz | 0.107031 ms for first occasion 0.104101 ms for second occasion |  |
|  | 120 kHz | 0.053515 ms for first occasion 0.052050 ms for second occasion |  |
|  | 480 kHz | 0.013379 ms for first occasion 0.013013 ms for second occasion |  |
|  | 960 kHz | 0.006690 ms for first occasion 0.006506 ms for second occasion |  |
| C0 | 60 kHz | 0.026758 ms |  |
|  | 120 kHz | 0.013379 ms |  |
|  | 480 kHz | 0.003345 ms |  |
|  | 960 kHz | 0.001672 ms |  |
| C2 | 60 kHz | 0.083333 ms |  |
|  | 120 kHz | 0.0416667 ms |  |
|  | 480 kHz | 0.010417 ms |  |
|  | 960 kHz | 0.005208 ms |  |
| NOTE: For PRACH on PRACH occasion start from begin of 0ms or 0.5 ms boundary, the measurement period will plus 0.032552 μs |  |  |  |

Figure 6.3.3.4-1: PRACH ON/OFF time mask

<Unchanged Text Skipped>

6.4.2.1	Error vector magnitude

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM, the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further equalised using the channel estimates subjected to the EVM equaliser spectrum flatness requirement specified in sub-clauses 6.4.2.4 and 6.4.2.5. For DFT-s-OFDM waveforms, the EVM result is defined after the front-end FFT and IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %. For CP-OFDM waveforms, the EVM result is defined after the front-end FFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH and one slot for PUCCH and PUSCH in the time domain. The EVM measurement interval is reduced by any symbols that contains an allowable power transient in the measurement interval as as defined in clause 6.3.3.

The RMS average of the basic EVM measurements over 10 subframes for the average EVM case, and over 60 subframes for the reference signal EVM case, for the different modulation schemes shall not exceed the values specified in Table 6.4.2.1-1 for the parameters defined in Table 6.4.2.1-2 or 6.4.2.1-3, depending on UE power class. For EVM evaluation purposes, all 13 PRACH preamble formats and all 5 PUCCH formats are considered to have the same EVM requirement as QPSK modulated.

The requirement is verified with the test metric of EVM (Link=TX beam peak direction, Meas=Link angle).

Table 6.4.2.1-1: Minimum requirements for error vector magnitude

| Parameter | Unit | Average EVM level | Reference signal EVM level |
| --- | --- | --- | --- |
| Pi/2 BPSK | % | 30.0 | 30.0 |
| QPSK | % | 17.5 | 17.5 |
| 16 QAM | % | 12.5 | 12.5 |
| 64 QAM | % | 8.0 | 8.0 |

Table 6.4.2.1-2: Parameters for Error Vector Magnitude for power class 1 in FR2-1

| Parameter | Unit | Level |
| --- | --- | --- |
| UE EIRP | dBm | 4 |
| UE EIRP for UL 16 QAM | dBm | 7 |
| UE EIRP for UL 64 QAM | dBm | 11 |
| Operating conditions |  | Normal conditions |

Table 6.4.2.1-2a: Parameters for Error Vector Magnitude for power class 1 in FR2-2

|  |  | Level |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| UE EIRP | dBm | 4 | 2 | 5 | 8 | 9 |
| UE EIRP for UL 16 QAM | dBm | 7 | 5 | 8 | 11 | 12 |
| UE EIRP for UL 64 QAM | dBm | 11 | 9 | 12 | 15 | 16 |
| Operating conditions | Normal Conditions |  |  |  |  |  |
| NOTE 1: PTRS is configured for 16 QAM and 64 QAM |  |  |  |  |  |  |

Table 6.4.2.1-3: Parameters for Error Vector Magnitude for power class 2, 3, 4 and 7 in FR2-1

| Parameter | Unit | Level |
| --- | --- | --- |
| UE EIRP | dBm | -13 |
| UE EIRP for UL 16 QAM | dBm | -10 |
| UE EIRP for UL 64 QAM | dBm | -6 |
| Operating conditions |  | Normal conditions |

Table 6.4.2.1-3a: Parameters for Error Vector Magnitude for power class 3 in FR2-2

|  |  | Level |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| UE EIRP | dBm | -13 | -11 | -8 | -5 | -4 |
| UE EIRP for UL 16 QAM | dBm | -10 | -8 | -5 | -2 | -1 |
| UE EIRP for UL 64 QAM | dBm | -6 | -4 | -1 | 2 | 3 |
| Operating conditions | Normal Conditions |  |  |  |  |  |
| NOTE 1: PTRS is configured for 16 QAM and 64 QAM |  |  |  |  |  |  |

Table 6.4.2.1-3b: Parameters for Error Vector Magnitude for power class 2 in FR2-2

|  |  | Level |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| UE EIRP | dBm | -13 | -11 | -8 | -5 | -4 |
| UE EIRP for UL 16 QAM | dBm | -10 | -8 | -5 | -2 | -1 |
| UE EIRP for UL 64 QAM | dBm | -6 | -4 | -1 | 2 | 3 |
| Operating conditions | Normal Conditions |  |  |  |  |  |
| NOTE 1: PTRS is configured for 16 QAM and 64 QAM |  |  |  |  |  |  |

Table 6.4.2.1-4: Parameters for Error Vector Magnitude for power class 5 in FR2-1

| Parameter | Unit | Level |
| --- | --- | --- |
| UE EIRP | dBm | -6 |
| UE EIRP for UL 16 QAM | dBm | -3 |
| UE EIRP for UL 64 QAM | dBm | 1 |
| Operating conditions |  | Normal conditions |

<Unchanged Text Skipped>

6.4.2.2.5	Carrier leakage for power class 4

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2.5-1 for power class 4.

Table 6.4.2.2.5-1: Minimum requirements for relative carrier leakage power for power class 4 in FR2-1

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 11 dBm | -25 |
| -13 dBm ≤ EIRP ≤ 11 dBm | -20 |

6.4.2.2.6	Carrier leakage for power class 5

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2.6-1 for power class 5.

Table 6.4.2.2.6-1: Minimum requirements for relative carrier leakage power for power class 5 in FR2-1

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 7 dBm | -25 |
| -6 dBm ≤ EIRP ≤ 7 dBm | -20 |

<Unchanged Text Skipped>

6.4.2.3.5	In-band emissions for power class 4

The average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4.2.3.5-1 for power class 4 UEs.

Table 6.4.2.3.5-1: Requirements for in-band emissions for power class 4 in FR2-1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 21 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 21 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 11 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 11 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Clause 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB = 1 or RB = -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

6.4.2.3.6	In-band emissions for power class 5

The average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4.2.3.6-1 for power class 5 UEs.

Table 6.4.2.3.6-1: Requirements for in-band emissions for power class 5 in FR2-1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 17 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 17 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 7 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -6 dBm ≤ Output power ≤ 7 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Clause 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB = 1 or RB = -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

<Unchanged Text Skipped>

6.5.3.2.5	Additional spurious emission requirements for NS_204

When "NS_204" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.5-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.5-1: Additional requirements (NS_204)

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 12.75 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |
| 87,5 MHz ≤ f ≤ 118 MHz | -54 dBm | 100 kHz |
| 174 MHz ≤ f ≤ 230 MHz | -54 dBm | 100 kHz |
| 470 MHz ≤ f ≤ 694 MHz | -54 dBm | 100 kHz |

<End of Changes >