3GPP TSG-RAN WG4 Meeting # 107                                                                 R4-2307243

Incheon, 22 - 26 May, 2023

Title: 	On reply to GSMA LS

Source: 	Huawei, HiSilicon

Agenda item:	8.16.1

Document for:	discussion

# Introduction

In RAN4 #106bis, WF [1] suggests further discussion.

Issue 1-1-1: Test parameters for 3GPP NR TRP/TRS OTA testing

Agreements:

- Keep current test parameters in the TR and TS.

Issue 1-1-3: How to handle the request from GSMA as in LS R4-2304023

Agreements:

- RAN4 further discuss how to address GSMA request.

This contribution investigates further and provides options.

# Discussion

Firstly, TRP as a function of channel bandwidth is investigated in case other organizations have concerns on TRP measurement under different channel bandwidths. Table 6.5.1-1 in 38.101-1 [2] shows that occupied channel bandwidth is the same as the NR channel bandwidth as follows:

Table 6.5.1-1: Occupied channel bandwidth

|  | NR channel bandwidth (MHz) |
| --- | --- |
|  | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Occupied channel bandwidth (MHz) | Same as NR channel bandwidth |

In addition, section 6.5.1 of [2] defines that 99% of the transmit power is contained within the occupied channel bandwidth. Namely 99% of the transmit power is always contained within the occupied channel bandwidth regardless of the bandwidth values.

Antennas in devices cover TDD channel bandwidth or both UL and DL spectra in FDD. In other words, the antenna efficiency does not change as a function of channel bandwidth. This leads to the conclusion that TRP is independent of channel bandwidth. This applies to TRP from a single antenna as TRP measurement procedures under TxD and UL MIMO are still under investigation.

Observation 1: TRP is independent of channel bandwidth.

Secondly, TRS values can be scaled from that at a given channel bandwidth to any channel bandwidth using Table 7.3.2-1b of [2] with NRB in REFSENS column being the maximum transmission bandwidth configuration as defined in Table 5.3.2-1 of [2].

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n77 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) |  |

Although Table 7.3.2-1b of [2] is for two antenna port reference sensitivity, the scaling mechanism is equally applicable to four antenna port reference sensitivity.

Observation 2: TRS at various channel bandwidths can be scaled using the formula in Table 7.3.2-1b in TS 38.101-1.

Proposal 1: state in reply to GSMA LS that TRP is independent of channel bandwidth and TRS values at various channel bandwidth can be scaled from that at a given channel bandwidth using Table 7.3.2-1b in TS 38.101-1.

# Conclusions

This contribution makes the following proposal and observations.

Observation 1: TRP is independent of channel bandwidth.

Observation 2: TRS at various channel bandwidths can be scaled using the formula in Table 7.3.2-1b in TS 38.101-1.

Proposal 1: state in reply to GSMA LS that TRP is independent of channel bandwidth and TRS values at various channel bandwidth can be scaled from that at a given channel bandwidth using Table 7.3.2-1b in TS 38.101-1.

A draft reply to GSMA LS is given in the appendix.

# References

- R4-2305904 WF for Rel-18 TRP/TRS requirements

- TS 38.101-1 v18.1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

# Appendix

3GPP TSG RAN WG4 Meeting #107  	                                                               R4-230xxxx

Incheon, 22 - 26 May, 2023

Title:	Reply LS to ETSI TC MSG/TFES on NR TRP and TRS requirements

Release:	Rel-18

Work Item:	NR_FR1_TRP_TRS_enh

Source:	3GPP RAN4

To:	GSMA

Cc:	3GPP RAN, 3GPP RAN WG5

Contact person:	Ruixin Wang

ruixin.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

3GPP RAN4 would like to thank GSMA for the LS (R4-2304023 TSGAP76_004) on bandwidth used for TRS measurement.

Table 5.3-2 in TS 38.161 specifies channel bandwidths for each band, ranging from 5 MHz to 100 MHz. TRS values measured at a specific bandwidth can be converted to a different bandwidth using the formula in Table 7.3.2-1b of TS 38.101-1.

In addition, TRP measurement is independent of channel bandwidth because 99 % of the transmit power is contained within channel bandwidth as specified in section 6.5.1 of TS 38.101-1.

RAN WG4 is therefore of the view that no change is needed in RAN WG4 and WG5 specification as TRS comparisons between different bandwidths can be readily made using the conversion mentioned above.

3GPP RAN4 looks forward to further cooperation with GSMA on NR FR1 TRP TRS work.

# 2	Actions

To ETSI TC MSG/TFES:

ACTION: 	3GPP RAN4 respectfully asks GSMA to take the above feedback into account.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #108	21-25 August, 2023			Toulouse, FR

TSG-RAN WG4 Meeting #108bis                           	9-13 October, 2023    		     Xiamen, CN