3GPP TSG-RAN WG4 Meeting #107 					                  R4-2307256

Incheon, KR, 22 - 26 May, 2023

Title:	Draft reply LS on the report of switching periods in Rel-18 UL Tx switching

Response to:	R2-2304567

Release:		Release 18

Work Item:	NR_MC_enh-Core

Source:	RAN4

To:	RAN2

CC: 	RAN1

Contact person:	Shan YANG

yangshan@chinatelecom.cn

Attachments:	none

- Overall description

RAN4 would like to thank RAN2 for their LS in R2-2304567. RAN4 discussed the question 1 to question from RAN2, and has agreed the following answers:

RAN2 Question 1:

RAN2 could not achieve conclusion, but has agreed an intention below in RAN2#121bis-e:

In support of RAN4 agreement, RAN2 intend to introduce support for two per-band-pair UE capabilities, a length of a switching period, for 1Tx-2Tx switching (like Rel-16) and that for 2Tx-2Tx switching (like Rel-17).

Question 1. (To RAN1 and RAN4)

RAN2 respectfully asks RAN1 and RAN4 to take above agreement on RAN2 intention into account and asks for feedback if there is any issue.

RAN4 Answer to Question 1:

RAN4 agrees with the above agreement on RAN2 intention. In addition, as described in an earlier RAN4 LS in R4-2303507, RAN4 also discussed the scenario of 1Tx-1Tx switching, i.e., the UL carriers in both bands before and after switching are capable of one transmit antenna connector, and agreed to apply the same length of switching period for 1Tx-1Tx switching and 1Tx-2Tx switching. It means that for the band pair supporting 1Tx-2Tx switching, the UE always support 1Tx-1Tx switching with the same length of switching period.

RAN2 Question 2:

RAN2 could not conclude whether the UE needs to explicitly report if it supports 2Tx-2Tx switching for every band pair used for Rel-18 UL Tx switching.

RAN2 is not sure which is the correct understanding:

- The UE always supports 2Tx-2Tx switching on a pair of bands if the UE supports 2 layers/ports UL MIMO on the two bands

- The UE may not support 2Tx-2Tx switching on a pair of bands even if the UE supports 2 layers/ports UL MIMO on the two bands (i.e., per-band-pair UE capability to report whether to support 2Tx-2Tx switching is needed, e.g. based on the presence/absence of 2Tx-2Tx switching period).

Question 2. (To RAN4)

RAN2 respectfully asks RAN4 to take below RAN2 assumptions into account and asks for feedback if there is any issue:

- For the band pair supporting 2Tx-2Tx switching, the UE always support 1Tx-2Tx switching.

- The UE reports whether it supports 2Tx-2Tx switching via per-band-pair UE capability.

RAN4 Answer to Question 2:

RAN4 agrees with the above two bullets in RAN2 assumptions.

RAN2 Question 3:

RAN2 has discussed how the gNB knows which of the reported switching periods (for 1Tx-2Tx switching or for 2Tx-2Tx) should be applied for every switching but could not conclude.

Question 3. (To RAN4)

RAN2 respectfully asks RAN4 which of the options below matches RAN4 understanding on the selection of applied switching periods when both switching periods of 2Tx-2Tx switching and 1Tx-2Tx switching can be reported for the same band pair.

Option 1: Based on implicit rules, e.g. 2Tx-2Tx switching period is only applicable when performing UL switching between two bands (e.g. 2P+0P<=>0P+2P) and 1Tx-2Tx period is applied for the other switching cases (e.g. UL Tx switching that involves 3 or 4 bands, such as band A + band B<=>band C, band A+ band B <=>band C + band D). FFS on the switching case of 2P+0P<=>1P+1P.

Option 2: Based on explicit RRC configuration, i.e., gNB configures which period is applied. FFS on the granularity of the configuration.

RAN4 Answer to Question 3:

The option 2 above matches RAN4 understanding. As described in an earlier RAN4 LS in R4-2214464, it is RAN4 understanding that:

- For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured.

# 2 Actions

To 3GPP RAN2

ACTION: RAN4 respectfully asks RAN2 to take into account the above information in their work.

# 3	Dates of next TSG RAN WG4 meetings

TSG-RAN WG4 Meeting #108			21 - 25 Aug, 2023		 Toulouse, FR

TSG-RAN WG4 Meeting #108-bis		09 - 13 Oct, 2023		 Xiamen, CN