3GPP TSG-RAN WG4 Meeting # 107		R4-2307282

Incheon, KR, May 22 – May 26, 2023

Source: 		Qualcomm Incorporated

Title: 	SSB based scheduling and measurement restrictions

Agenda item:	8.8.3.4

Document for:		Approval

# Introduction

In this paper, we present our view on the following aspects:

- Measurement Restriction

- Scheduling Availability

# Discussion

Although we do not support simultaneous Rx beam based fast RLM and BFD evaluation, for CSI-RS based measurement and scheduling restriction, we can consider the requirement relaxation.

Measurement Restriction

When an L1 measurement resource is associated with one of active TCI states for group-based simultaneous mTRP support, UE may be able to receive and process another resource, e.g. CSI-RS for RLM/BFD, from the other TRP.

Proposal 1: A measurement restriction for mTRP can be defined such that the following can be supported:

- UE can receive SSB for L1-RSRP measurement from one TRP while performing CSI-RS based RLM and BFD from the other TRP if the resources are associated with active TCI states for simultaneous reception from the two TRPs

- UE can receive CSI-RS for L1-RSRP measurement from one TRP while performing CSI-RS based RLM and BFD from the other TRP if the resources are associated with active TCI states for simultaneous reception from the two TRPs

- The CSI-RSs are not in a resource set configured with repetition ON

Scheduling Availability

According to RAN1 spec (relevant statements are quoted in Figure 1), PDSCHs can be scheduled to a UE on OFDM symbols overlapping with SSB if the SSB is associated with one of the PDSCHs, which is illustrated in Figure 1.

Fig. 1 Scheduling availability in FR2 mTRP

Observation 1: The following two statements from TS38.214 allow a simultaneous reception of PDSCHs on SSB symbols if at least one MD-RS port for the PDSCH(s) is associated with the SSB in terms of QCL-TypeD:

- If at least one TCI codepoint indicates two TCI states and the UE receives the DM-RS for PDSCH and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that at least one DM-RS port for the PDSCH and SS/PBCH block are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable.

- If the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, and the UE receives the DM-RS for PDSCH(s) and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that at least one DM-RS port for the PDSCH(s) and SS/PBCH block are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable.

With Observation 1, the legacy scheduling restriction rule can be relaxed such that a simultaneous reception of PDSCHs from two TRPs on the SSB symbols can be supported. And for such scheduling relaxations, a proper set of side conditions needs to be defined, e.g. the relaxation is applied only when UE is in a simultaneous dual beam based mTRP mode.

Proposal 2: A scheduling availability during SSB based L1 measurements from two TRPs can be defined such that a reception of PDSCH from one TRP on the SSB symbols overlapping with the SSB from the other TRP can be supported if at least the DM-RS port for the PDSCH is associated with one of SSBs configured for R17 group-based L1-RSRP measurement in terms of QCL-TypeD. FFS on side conditions, e.g.

- UE is configured with active TCI states from two TRPs, and the association between the TCI states and the TRPs is explicitly known to the UE, i.e.

  - (single DCI based mTRP) at least one of the codepoints in the active TCI list for PDSCH includes two reference resources for qcl-TypeD from respective TRPs

  - (multi DCI based mTRP) two CORESETs QCL’ed with two reference resources for qcl-TypeD are configured

  - [SNR > XdB for each TRP, where rank > 2 is expected]

- Group-based L1-RSRP measurement is configured based on L3 measurements for the same measurement resources

# Conclusions

In this paper, we proposed the following.

Measurement Restriction

Proposal 1: A measurement restriction for mTRP can be defined such that the following can be supported:

- UE can receive SSB for L1-RSRP measurement from one TRP while performing CSI-RS based RLM and BFD from the other TRP if the resources are associated with active TCI states for simultaneous reception from the two TRPs

- UE can receive CSI-RS for L1-RSRP measurement from one TRP while performing CSI-RS based RLM and BFD from the other TRP if the resources are associated with active TCI states for simultaneous reception from the two TRPs

- The CSI-RSs are not in a resource set configured with repetition ON

Scheduling Availability

Observation 1: The following two statements from TS38.214 allow a simultaneous reception of PDSCHs on SSB symbols if at least one MD-RS port for the PDSCH(s) is associated with the SSB in terms of QCL-TypeD:

- If at least one TCI codepoint indicates two TCI states and the UE receives the DM-RS for PDSCH and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that at least one DM-RS port for the PDSCH and SS/PBCH block are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable.

- If the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, and the UE receives the DM-RS for PDSCH(s) and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that at least one DM-RS port for the PDSCH(s) and SS/PBCH block are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable.

Proposal 2: A scheduling availability during SSB based L1 measurements from two TRPs can be defined such that a reception of PDSCH from one TRP on the SSB symbols overlapping with the SSB from the other TRP can be supported if at least the DM-RS port for the PDSCH is associated with one of SSBs configured for R17 group-based L1-RSRP measurement in terms of QCL-TypeD. FFS on side conditions, e.g.

- UE is configured with active TCI states from two TRPs, and the association between the TCI states and the TRPs is explicitly known to the UE, i.e.

  - (single DCI based mTRP) at least one of the codepoints in the active TCI list for PDSCH includes two reference resources for qcl-TypeD from respective TRPs

  - (multi DCI based mTRP) two CORESETs QCL’ed with two reference resources for qcl-TypeD are configured

  - [SNR > XdB for each TRP, where rank > 2 is expected]

- Group-based L1-RSRP measurement is configured based on L3 measurements for the same measurement resources