3GPP TSG-RAN WG4 Meeting # 107	R4-2307338

# Incheon, Korea, May 22 – May 26, 2023

Agenda item:	8.19.2

Source:	Apple

Title:	On Simulation Results for Physical layer throughput

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#106bis-e requirements for physical layer throughput were discussed and way forward [1] was approved. It was encouraged to bring simulation results for alignment and requirements definition. In this paper we present alignment and impairment results for defining requirements for phy layer TP.

# 2. Simulation Results

For the simulation parameters in the table below we provide we simulation results in the sub-sections below.

Table 1: Simulation Parameters for Physical layer throughput

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 4 | 4 | 4 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

FR1 FDD

Table 2: Simulation results for FDD 2x2

| SNR (dB) | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER |  |  |
| 0 | 3.57 | 0.07 | 3 | 1 |
| 2 | 5.07 | 0.07 | 3 | 1 |
| 4 | 6.85 | 0.07 | 4 | 1 |
| 6 | 8.86 | 0.07 | 5 | 1 |
| 8 | 10.98 | 0.06 | 6 | 1 |
| 10 | 13.46 | 0.06 | 6 | 1 |
| 12 | 16.19 | 0.06 | 7 | 1 |
| 14 | 19.01 | 0.07 | 8 | 2 |
| 16 | 22.12 | 0.07 | 8 | 2 |
| 18 | 25.19 | 0.09 | 9 | 2 |
| 20 | 28.52 | 0.10 | 9 | 2 |

Table 3: Max TP% vs SNR for FDD 2RX

| %Max TP | SNR @ %Max TP |  |
| --- | --- | --- |
|  | Alignment | Impairment |
| 10% | 3.6 | 5.8 |
| 15% | 6.8 | 9.0 |
| 20% | 9.6 | 11.8 |
| 25% | 12.0 | 14.2 |
| 30% | 14.2 | 16.5 |
| 35% | 16.3 | 18.6 |
| 40% | 18.4 | 20.6 |

Table 4: Simulation results for FDD 2x4

| SNR (dB) | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER |  |  |
| 0 | 6.00 | 0.19 | 4 | 1 |
| 2 | 8.00 | 0.11 | 4 | 2 |
| 4 | 9.92 | 0.14 | 4 | 2 |
| 6 | 13.23 | 0.14 | 5 | 2 |
| 8 | 17.12 | 0.11 | 5 | 2 |
| 10 | 21.27 | 0.07 | 6 | 2 |
| 12 | 25.26 | 0.07 | 7 | 2 |
| 14 | 29.51 | 0.08 | 8 | 2 |
| 16 | 34.38 | 0.08 | 9 | 2 |
| 18 | 39.49 | 0.07 | 10 | 2 |
| 20 | 43.73 | 0.08 | 11 | 2 |

Table 5: Max TP% vs SNR for FDD 4RX

| %Max TP | SNR @ %Max TP |  |
| --- | --- | --- |
|  | Alignment | Impairment |
| 10% | 0.5 | 2.7 |
| 15% | 3.7 | 6.0 |
| 20% | 5.8 | 8.1 |
| 25% | 7.5 | 9.7 |
| 30% | 9.1 | 11.3 |
| 35% | 10.7 | 12.9 |
| 40% | 12.3 | 14.5 |
| 45% | 13.8 | 16.0 |
| 50% | 15.1 | 17.4 |
| 55% | 16.4 | 18.7 |
| 60% | 17.7 | 19.9 |
| 65% | 19.2 | 21.4 |

FR1 TDD

Table 6: Simulation results for TDD 2x2

| SNR (dB) | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER |  |  |
| 0 | 10.83 | 0.05 | 3 | 1 |
| 2 | 15.54 | 0.05 | 3 | 1 |
| 4 | 21.27 | 0.06 | 4 | 1 |
| 6 | 27.49 | 0.06 | 5 | 1 |
| 8 | 34.20 | 0.05 | 6 | 1 |
| 10 | 41.84 | 0.04 | 6 | 1 |
| 12 | 50.11 | 0.06 | 7 | 1 |
| 14 | 58.56 | 0.07 | 7 | 2 |
| 16 | 67.88 | 0.08 | 8 | 2 |
| 18 | 78.00 | 0.09 | 8 | 2 |
| 20 | 87.58 | 0.11 | 8 | 2 |

Table 7: Max TP% vs SNR for TDD 2RX

| %Max TP | SNR @ %Max TP |  |
| --- | --- | --- |
|  | Alignment | Impairment |
| 10% | 3.8 | 6.1 |
| 15% | 7.1 | 9.3 |
| 20% | 9.9 | 12.2 |
| 25% | 12.4 | 14.7 |
| 30% | 14.8 | 17.0 |
| 35% | 16.9 | 19.2 |
| 40% | 19.0 | 21.3 |

Table 8: Simulation results for TDD 2x4

| SNR (dB) | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER |  |  |
| 0 | 18.63 | 0.14 | 4 | 1 |
| 2 | 25.14 | 0.06 | 4 | 2 |
| 4 | 30.74 | 0.07 | 4 | 2 |
| 6 | 42.25 | 0.09 | 4 | 2 |
| 8 | 54.40 | 0.07 | 5 | 2 |
| 10 | 66.97 | 0.05 | 6 | 2 |
| 12 | 79.93 | 0.04 | 7 | 2 |
| 14 | 94.71 | 0.04 | 8 | 2 |
| 16 | 108.80 | 0.05 | 9 | 2 |
| 18 | 121.21 | 0.08 | 10 | 2 |
| 20 | 135.53 | 0.08 | 11 | 2 |

Table 9: Max TP% vs SNR for TDD 4RX

| %Max TP | SNR @ %Max TP |  |
| --- | --- | --- |
|  | Alignment | Impairment |
| 10% | 0.6 | 2.9 |
| 15% | 4.1 | 6.3 |
| 20% | 5.9 | 8.1 |
| 25% | 7.6 | 9.8 |
| 30% | 9.2 | 11.5 |
| 35% | 10.9 | 13.1 |
| 40% | 12.4 | 14.7 |
| 45% | 13.8 | 16.1 |
| 50% | 15.3 | 17.5 |
| 55% | 16.9 | 19.1 |
| 60% | 18.5 | 20.7 |
| 65% | 19.9 | 22.2 |

FR2

Table 10: Simulation results for FR2 TDD 2x2

| SNR (dB) | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER |  |  |
| 0 | 17.2 | 0.2 | 5 | 1 |
| 2 | 23.2 | 0.2 | 6 | 1 |
| 4 | 30.8 | 0.2 | 7 | 1 |
| 6 | 39.4 | 0.2 | 8 | 1 |
| 8 | 47.7 | 0.2 | 9 | 1 |
| 10 | 56.8 | 0.2 | 9 | 1 |
| 12 | 70.5 | 0.2 | 9 | 2 |
| 14 | 86.0 | 0.1 | 9 | 2 |
| 16 | 99.9 | 0.1 | 9 | 2 |

Table 11: Max TP% vs SNR for FR2 TDD 2RX

| %Max TP | SNR @ %Max TP |  |
| --- | --- | --- |
|  | Alignment | Impairment |
| 10% | 2.3 | 4.7 |
| 15% | 5.3 | 7.7 |
| 20% | 8.2 | 10.6 |
| 25% | 10.6 | 13.0 |
| 30% | 12.3 | 14.7 |
| 35% | 13.9 | 16.3 |
| 40% | 15.6 | 18.0 |

# 4. Conclusion

In this paper, we provide simulation results for alignment for defining physical layer throughput requirements.

# Reference

- R4-2305915, “WF for Absolute physical layer throughput requirements”, Intel.