3GPP TSG-RAN WG4 Meeting # 107	R4-2307340

# Incheon, Korea, May 22 – May 26, 2023

Agenda item:	8.22.2

Source:	Apple

Title:	On RAN4 requirements for use cases for AI/ML

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#106-bis-e RAN4 started the discussion on RAN4 impact with AI/ML in the air interface and way forward [1] was agreed.  In this contribution we present our views on issues related to use cases for AI/ML.

# 2. Discussion

In RAN4#106bis-e it was agreed that the following use cases will be further handled and studied in RAN4:

- 2.2.1 Use cases and sub-use cases to be handled Agreement: - Following use cases and sub-use cases will be handled in RAN4: - CSI feedback enhancement - time domain CSI prediction - spatial-frequency domain CSI compression - Beam management - Spatial-domain DL beam prediction - Temporal DL beam prediction - Positioning accuracy enhancements - direct AI/ML positioning - AI/ML assisted positioning

For Core requirements for AI/ML, the following was agreed:

- 2.2.2 Core requirements for AI/ML - Definition of RAN4 core requirements for the following procedures will be studied based on progress in RAN1/RAN2: - Performance monitoring procedure, including performance evaluation and decision making procedure for AI/ML functionalities/models - Functionality/Model management procedure, including functionality/model selection/activation/deactivation, and functionality/model switching/fallback/transfer/delivery/update - Latency/interruption requirement for above procedures - FFS is any other aspects should be studied

RAN1/RAN2 will are likely to define core requirements for various aspects of LCM in the WI phase. The core requirements would be RRC/ MAC-CE or DCI based. RAN4 could use the legacy framework for core requirements as a starting point. The details of the requirements can be discussed in the WI phase based on what procedures are defined in RAN1/RAN2 – such as latency and interruption requirements.

Proposal #1: Use the legacy framework for RRC/MAC-CE/DCI based core requirements.

Proposal #2: RAN4 can discuss details of the requirements in WI phase based on what is defined in RAN1/RAN2.

For the test metrics to be considered for different use cases the following was agreed:

- 2.2.3 KPIs/ Test Metrics for use cases Following KPIs are to be considered in the RAN4 study for different use cases. - KPIs/Test Metrics for CSI prediction and compression - Throughput - Other options could also be considered depending on work in other work groups For e.g., SGCS/NMSE and accuracy of CSI prediction, latency of CSI feedback/prediction - KPIs/Test Metrics for beam management - Beam prediction accuracy (absolute or relative) - other KPIs could also be considered: e.g., link throughput, beam measurement accuracy, prediction confidence etc. - KPIs/Test Metrics for positioning - Measurement accuracy - FFS whether latency can also be considered - other KPIs could also be considered:

RAN4 has to study defining requirements for 2 aspects – the performance requirement for the features that use AI/ML based on model inference, and requirements if deemed necessary for LCM procedures, such as model monitoring.

Observation #1: RAN4 shall study to define requirements for features that use AI/ML and if necessary for LCM procedures.

The test metrics that are used for performance requirement with features with AI/ML would be the legacy metrics that RAN4 uses for requirements definition. Such as:

- For CSI reporting – Throughput

- For Beam management – RSRP

Proposal #3: The test metrics for performance requirement with features with AI/ML would be – Throughput for CSI reporting, RSRP for beam management.

For the LCM procedures, RAN1 is discussing the intermediate/eventual KPIs that can be used for model monitoring.

From RAN1#112

From RAN1#112:

In RAN1 the discussion for model monitoring is still ongoing and RAN4 needs to wait for progress in RAN1 before studying the procedures for requirements associated with model monitoring.

Observation #3: RAN1 is still discussing the test metrics/ KPIs for model monitoring.

Proposal #4: RAN4 shall study the procedures for requirements associated with model monitoring based on RAN1 progress.

For LCM related requirements, depending on RAN1/RAN2 progress the following was agreed:

- 2.2.4 LCM Related requirements Agreements: - Following LCM related requirements are to be studied: - Model/Functionality select/switch/activate/deactivate/fallback - Model/Functionality monitoring - FFS if requirements for data collection (in particular for training) could/need be defined - FFS if requirements for transfer/delivery/update - NOTE: RAN4 study should be aligned with the agreements in other working groups. - Further study under LCM related tests, if they are defined. - how the framework can address the possibility of updates/activation/deactivation/switching to the functionalities/models after the deployment of the devices in the filed

The LCM related requirements would be the core requirements discussed in 2.2.2 or the performance requirements for model monitoring discussed pertaining to the test metrics defined in 2.2.3.

- Model/Functionality select/switch/activate/deactivate/fallback -> belongs to Core requirements.

- Model/Functionality monitoring -> belongs to performance requirements.

Observation #4: The LCM related requirements could be core or performance requirements.

It is not clear what requirements can be defined for data collection for offline training since it is delay insensitive. Hence, we propose not to define any requirements for data collection.

Observation #5: Data collection for offline training is delay insensitive.

Proposal #5: Do not define requirements for data collection.

For transfer/delivery/update RAN2 is discussing a framework using control or user place solution. We don’t see the necessity to define requirements for this in RAN4.

Proposal #6: Do not define requirements for transfer/delivery/update.

# 3. Conclusion

In this paper, we provide our views on issues related to use cases for AI/ML. Our observations and proposals are captured below:

Proposal #1: Use the legacy framework for RRC/MAC-CE/DCI based core requirements.

Proposal #2: RAN4 can discuss details of the requirements in WI phase based on what is defined in RAN1/RAN2.

Observation #1: RAN4 shall study to define requirements for features that use AI/ML and if necessary for LCM procedures.

Proposal #3: The test metrics for performance requirement with features with AI/ML would be – Throughput for CSI reporting, RSRP for beam management.

Observation #3: RAN1 is still discussing the test metrics/ KPIs for model monitoring.

Proposal #4: RAN4 shall study the procedures for requirements associated with model monitoring based on RAN1 progress.

Observation #4: The LCM related requirements could be core or performance requirements.

Observation #5: Data collection for offline training is delay insensitive.

Proposal #5: Do not define requirements for data collection.

Proposal #6: Do not define requirements for transfer/delivery/update.

# Reference

- R4-2306299, “WF on AI/ML RAN4 studies”, Qualcomm Incorporated.