3GPP TSG-RAN WG4 Meeting #107                                                                                    R4-2307386

Incheon, KR, May 22 – May 26, 2023

Source: 	CATT

Title: 	Discussion on BS RF requirement for network energy saving for NR

Agenda item:	8.35.2

Document for:	Approval

- Introduction

In 106-bis meeting, A WF [1, 3] on RF feasibility study of Network Energy Savings for NR was agreed. Some remaining issues in WF need further discussion in next meeting.

In this contribution, we provide our views on BS RF core requirement for network energy saving for NR.

- Discussion

BS feasibility for inter-band CA for SSB-less Scell operation

As per the WF [1], the remaining open issue concerning BS feasibility for inter-band CA for SSB-less Scell operation is shown as below:

Issue 2-1-1: Priorities for feasibility study - Agreement： - RAN4 prioritize the discussion on SSB-less Scell operation for inter-band CA for FR1 and co-located cell.

For SSB-less Scell operation,  the TAE requirement is more important factor for Scell timing, the matter is TAE at UE (generally referred as MRTD), if TAE requirements for inter-band CA is discussed isolated at the base station, this can lead to wrong or unbalanced requirements. The MRTD will be discussed in RRM session, so BS inter-band CA TAE requirement for SSB-less operation can wait for the conclusion of RRM discussion. After this, we can further discuss the BS achievable inter-band CA TAE.

Observation 1: BS inter-band CA TAE requirement for SSB-less operation can wait for the conclusion of RRM discussion.

BS RF requirement for inter-band CA for SSB-less Scell operation

As per the WF [1], the remaining open issue concerning BS RF requirement for inter-band CA for SSB-less Scell operation is shown as below:

Issue 2-2-6: How to define the RF requirements for inter-band CA for SSB-less Scell operation - Agreement: - No impact to the legacy requirements of inter-band CA TAE defined in TS38.104. Whether or not to define feature-specific TAE requirements for SSB-less operation (NW enabling energy savings) for FR1 co-located inter-band CA is FFS.

The inter-band CA TAE for SB-less operation can be optional and manufacturer declared.

Observation 2: The inter-band CA TAE for SB-less operation can be optional and manufacturer declared.

- Conclusion

This contribution provides our consideration on BS RF core requirement for network energy saving for NR. The following observation is concluded as follows:

Observation 1: BS inter-band CA TAE requirement for SSB-less operation can wait for the conclusion of RRM discussion.

Observation 2: The inter-band CA TAE for SB-less operation can be optional and manufacturer declared.

- References

- RP-223540, New WID: Network energy savings for NR, Huawei, RAN#98-e

- TR 38.864 V18.1.0, Study on network energy savings for NR(Release 18)

- R4-2306639, WF on RF feasibility study of Network Energy Savings for NR, Moderator (Huawei),RAN4#106-bis-e