3GPP TSG-RAN WG4 Meeting # 107                                                        R4- 2307391			R4-22xxxxx

# Incheon, KR, May 22 – May 26, 2023

Title: 	SBFD adjacent channel co-existence simulation results

Source: 	CATT

Agenda item:	8.20.2.1

Document for:	Discussion

- Background

This contribution provides the SBFD adjacent channel co-existence simulation results according to the update of the simulation assumption in last RAN4 meeting [1] and [2], also the WF[3] in RAN4#106 meeting.

- Discussion

  - SBFD adjacent channel co-existence simulation scenarios

SBFD adjacent channel co-existence simulation calibration scenarios summarized in table 1, SBFD subband configuration is {DU}.

Table 1: SBFD adjacent channel co-existence simulation calibration scenarios

| FR1 SBFD Urban Macro UL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| --- | --- |
| FR1 SBFD Urban Macro DL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| FR1 legacy TDD Urban Macro UL | Victim: NR TDD UL SINR baseline: NR TDD UL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: Low |
| FR1 legacy TDD Urban Macro DL | Victim: NR TDD DL SINR baseline: NR TDD DL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: High |
| FR2 SBFD Urban Macro UL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| FR2 SBFD Urban Macro DL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| FR2 legacy TDD Urban Macro UL | Victim: NR TDD UL SINR baseline: NR TDD UL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: Low |
| FR2 legacy TDD Urban Macro DL | Victim: NR TDD DL SINR baseline: NR TDD DL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: High |

  - Simulation calibration results

2.2.1 FR1

      - FR1 SBFD Urban Macro UL

Table 2: FR1 SBFD Urban Macro UL SINR and Throughput

| FR1 SBFD Urban Macro UL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | -7.84 | 11.65 | 0.09 | 1.59 |
| Baseline + ACI | -8.46 | 11.17 | 0.08 | 1.53 |
| ((Baseline + ACI) - Baseline)/Baseline | 0.0791 | 0.0415 | 0.1246 | 0.0378 |

Figure1: FR1 SBFD Urban Macro UL SINR and Throughput

The simulation results show 50% throughput degrades 3.78%,  5% throughput degrades 12.46%.

      - FR1 SBFD Urban Macro DL

Table 3: FR1 SBFD Urban Macro DL SINR and Throughput

| FR1 SBFD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 7.33 | 24.18 | 1.61 | 4.82 |
| Baseline + ACI | 6.63 | 23.57 | 1.49 | 4.70 |
| ((Baseline + ACI) - Baseline)/Baseline | 0.0951 | 0.0250 | 0.0720 | 0.0249 |

Figure2: FR1 SBFD Urban Macro DL SINR and Throughput

The simulation results show 50% throughput degrades 2.49%,  5% throughput degrades 7.20%.

      - FR1 legacy TDD Urban Macro UL

Table 4: FR1 legacy TDD Urban Macro UL SINR and Throughput

| FR1 legacy TDD Urban Macro UL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | -8.32 | 12.49 | 0.08 | 1.69 |
| Baseline + ACI | -8.70 | 12.06 | 0.07 | 1.64 |
| ((Baseline + ACI) - Baseline)/Baseline | 0.0462 | 0.0340 | 0.0795 | 0.0315 |

Figure3: FR1 legacy TDD Urban Macro UL SINR and Throughput

The simulation results show 50% throughput degrades 3.15%,  5% throughput degrades 7.95%.

      - FR1 legacy TDD Urban Macro DL

Table 5: FR1 legacy TDD Urban Macro DL SINR and Throughput

| FR1 legacy TDD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 5.96 | 22.74 | 1.38 | 4.54 |
| Baseline + ACI | 5.77 | 22.63 | 1.35 | 4.51 |
| ((Baseline + ACI) - Baseline)/Baseline | 0.0322 | 0.0050 | 0.0219 | 0.0050 |

Figure4: FR1 legacy TDD Urban Macro DL SINR and Throughput

The simulation results show 50% throughput degrades 0.50%,  5% throughput degrades2.19%.

2.2.2 FR2

- 2.2.2.1 FR2 SBFD Urban Macro UL

Table 6: FR2 SBFD Urban Macro UL SINR and Throughput

| FR2 SBFD Urban Macro UL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | -7.39 | 12.14 | 0.10 | 1.65 |
| Baseline + ACI | -7.47 | 12.02 | 0.10 | 1.63 |
| ((Baseline + ACI) - Baseline)/Baseline | 0.0115 | 0.0100 | 0.0178 | 0.0092 |

Figure5: FR2 SBFD Urban Macro UL SINR and Throughput

The simulation results show 50% throughput degrades 0.92%,  5% throughput degrades1.78%.

- 2.2.2.2 FR2 SBFD Urban Macro DL

Table 7: FR2 SBFD Urban Macro DL SINR and Throughput

| FR2 SBFD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 10.91 | 25.64 | 2.24 | 5.11 |
| Baseline + ACI | 10.08 | 24.81 | 2.09 | 4.95 |
| ((Baseline + ACI) - Baseline)/Baseline | 0.0767 | 0.0322 | 0.0683 | 0.0321 |

Figure6: FR2 SBFD Urban Macro DL SINR and Throughput

The simulation results show 50% throughput degrades 3.21%,  5% throughput degrades 6.83%.

- 2.2.2.3 FR2 legacy TDD Urban Macro UL

Table 8: FR2 legacy TDD Urban Macro UL SINR and Throughput

| FR2 legacy TDD Urban Macro UL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | -5.04 | 12.21 | 0.16 | 1.66 |
| Baseline + ACI | -5.28 | 12.07 | 0.15 | 1.64 |
| ((Baseline + ACI) - Baseline)/Baseline | 0.0485 | 0.0120 | 0.0482 | 0.0110 |

Figure7: FR2 legacy TDD Urban Macro UL SINR and Throughput

The simulation results show 50% throughput degrades 1.10%,  5% throughput degrades 4.82%.

- 2.2.2.4 FR2 legacy TDD Urban Macro DL

Table 9: FR2 legacy TDD Urban Macro DL SINR and Throughput

| FR2 legacy TDD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 10.77 | 26.00 | 2.22 | 5.18 |
| Baseline + ACI | 10.62 | 25.83 | 2.19 | 5.15 |
| ((Baseline + ACI) - Baseline)/Baseline | 0.0141 | 0.0065 | 0.0125 | 0.0065 |

Figure8: FR2 legacy TDD Urban Macro DL SINR and Throughput

The simulation results show 50% throughput degrades 0.65%,  5% throughput degrades 1.25%.

  - Co-existence simulation results

Table 10: Simulation cases for SBFD

| Victim | Aggressor | Figures: Aggressor(left) and Victim(right) | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- |
| NR TDD DL | SBFD (DU) | Case 1 | NR TDD DL | High |
| NR TDD UL | SBFD (DU) | Case 2 | NR TDD UL | Low |
| SBFD (DU) | NR TDD DL | Case 3 | No system in adjacent channel | High |
| SBFD(DU) | NR TDD UL | Case 4 |  | Low |

Table 11 provides 5% and 50% throughput degrades of different ACIR for FR1 and FR2, Simulation cases is specified in Table 10. The table also provide the throughput loss for 5% and 50% observation point for the case when TDD interfere TDD using the same parameters as SBFD system when SBFD is victim.

Table 11: SBFD adjacent channel co-existence simulation results

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | … | - 4dB | - 2dB | Relative ACIR | + 2dB | + 4dB | … | with relative ACIR Note1 |  |
| FR1 Uma-Uma |  | 1 | 5% |  | 2.57 | 2.35 | 2.19 | 0.52 | 0.09 |  |  |  |
|  |  |  | 50% |  | 0.96 | 0.70 | 0.50 | 0.34 | 0.22 |  |  |  |
|  |  | 2 | 5% |  | 12.66 | 10.89 | 7.95 | 3.11 | 0.52 |  |  |  |
|  |  |  | 50% |  | 4.93 | 3.97 | 3.15 | 2.59 | 2.13 |  |  |  |
|  |  | 3 | 5% |  | 17.99 | 13.84 | 7.20 | 5.51 | 4.18 |  | 3.15 |  |
|  |  |  | 50% |  | 3.78 | 3.02 | 2.49 | 1.88 | 1.46 |  | 1.27 |  |
|  |  | 4 | 5% |  | 16.16 | 13.89 | 12.46 | 10.89 | 5.26 |  | 4.26 |  |
|  |  |  | 50% |  | 5.71 | 4.63 | 3.78 | 3.02 | 2.42 |  | 0.85 |  |
| FR2 Uma-Uma |  | 1 | 5% |  | 19.09 | 5.54 | 1.25 | 0.07 | 0.05 |  |  |  |
|  |  |  | 50% |  | 1.10 | 0.83 | 0.65 | 0.43 | 0.29 |  |  |  |
|  |  | 2 | 5% |  | 100.00 | 100.00 | 4.82 | 2.12 | 0.91 |  |  |  |
|  |  |  | 50% |  | 2.47 | 1.86 | 1.10 | 0.85 | 0.82 |  |  |  |
|  |  | 3 | 5% |  | 31.31 | 17.91 | 6.83 | 3.16 | 0.07 |  | 3.37 |  |
|  |  |  | 50% |  | 5.00 | 3.95 | 3.21 | 2.43 | 1.90 |  | 1.67 |  |
|  |  | 4 | 5% |  | 100.00 | 3.47 | 1.78 | 0.70 | 0.07 |  | 0.94 |  |
|  |  |  | 50% |  | 1.97 | 1.27 | 0.92 | 0.83 | 0.53 |  | 0.44 |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: |  |  |  |  |  |  |  |  |  |  |  |  |
| - The relative means was derived from the legacy and baseline ACS assumptions for legacy TDD and SBFD BS and UE; |  |  |  |  |  |  |  |  |  |  |  |  |
| - The -4/-2/+2/+4 are the offset based on that relative ACIR. |  |  |  |  |  |  |  |  |  |  |  |  |
| - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; |  |  |  |  |  |  |  |  |  |  |  |  |
| - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; |  |  |  |  |  |  |  |  |  |  |  |  |
| - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

In Table 10, when SINR is less than SINRmin which is -10dB, throughput degrades 100%.

- Conclusion

This contribution summarizes the simulation results from our company. For FR1, it needs higher ACIR when NR TDD/SBFD UL is victim than NR TDD/SBFD DL is victim. For FR2, it is same as FR1 when NR TDD UL is victim, but it’s opposite when SBFD UL is victim.

Reference

[1] R4-2305921, “ WF on SBFD co-existence simulation”, RAN4#106bis-e

[2] R4-2305923, “Summary of co-existence simulation parameters”, RAN4#106bis-e

[3] R4-2302888, “WF for SBFD adjacent channel co-existence study”, RAN4#106