3GPP TSG-RAN WG4 Meeting # 107	            R4-2307404

# Incheon, KR, May 22th – May 26th, 2023

Agenda Item:	8.29.5

Source: 	CATT

Title: 	Discussion on RRM core requirements for NR Network-controlled Repeaters

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, WF on NCR-MT RRM requirements was approved [1]. In this contribution, some open issues on NCR-MT RRM requirements are further discussed.

2. Discussion

In the last meeting, RAN4 has agreed many agreements for NCR-MT and a few open issues are as follows:

Issue 4-2: adaptive beamforming for NCR access link

Agreements

FFS whether to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency.

Considering the beam is fixed, so it is not necessary to define requirements for periodic beam configuration, and switching latency is also unimportant. We prefer to not to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency.

Proposal 1: RAN4 not to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency.

BFD/BFR/RLM for NCR-MT

Issue 5-1:  BFD/BFR/RLM for NCR-MT

Agreements

  - For LA NCR-MT

    - Define RLM/BFD/BFR requirements

  - For WA NCR-MT

    - Do not define BFD/BFR requirements

    - FFS for RLM requirements.

For LA NCR-MT, it was agreed to define RLM/BFD/BFR requirements. For RLM requirements, we agree that link recovery though re-establishment is one of the most important NCR-MT procedures compared with BFD/BFR [2]. Though the link of WA could be less impacted by interruptions compared with LA, we don’t see any strong reason to exclude RLM requirements for WA NCR-MT. So, we can compromise to define RLM requirements for WA NCR-MT.

Proposal 2: RAN4 to define RLM requirements for WA NCR-MT.

3. Conclusion

In this paper, we provide our views on NCR-MT RRM requirements. From this discussion we have derived the following proposals:

Proposal 1: RAN4 not to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency.

Proposal 2: RAN4 to define RLM requirements for WA NCR-MT.

4. Reference

- R4-2306360 WF on NCR-MT RRM requirements, ZTE

- R4-2306253 Topic summary for NCR-MT RRM requirements, ZTE