3GPP TSG-RAN4 Meeting #107	                                                            R4-2307410

Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on the RRM impact of option A, B-1-1, B-1-2 and C for the support for bandwidth part operation without restriction

Source:	CATT

Agenda Item:	8.11.2

Document for:	Discussion

1 Introduction

In last meeting, the WI to complete the specification support for BandWidth Part operation without restriction in NR was discussed and the conclusions were captured in the approved WF [1].

This contribution further discusses the RRM requirements and specification support for the above options and presents our understandings and proposals.

2 Discussion

It was agreed in last meeting that the existing requirements for SSB-based RLM/BFD/BM can be reused for option A/B-1-1/ C. And the possible clarification is FFS. Based on our discussion in last meeting, we think no additional clarifications are needed for option A and B-1-1. For option C, it needs to clarify that the current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support.

Proposal 1: No additional clarification is needed for timing requirements for option A and B-1-1.

Proposal 2: For option C, the current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support.

For option B-1-2, the measurement is performed without gap but interruption is needed. For the measurement requirements, the same conclusions as option B-1-1 can be used. For interruption requirements, similar as that for other measurements in existing specification, the interruption for option B-1-2 can also be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate. And the existing interruption ratio in the specification for other measurements can be reused (i.e. missed ACK/NACK is up to 0.5%). And the length for each interruption should be defined based on RF retuning time (0.5ms for FR1 and 0.25ms for FR2).

Proposal 3: For option B-1-2, existing SSB based BM/RLM/BFD measurement requirements apply and no additional measurement requirements are needed.

Proposal 4: For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%]. And the length for each interruption shall not exceed the RF retuning time (0.5ms for FR1 and 0.25ms for FR2).

In last meeting, there were some other issues raised:

## Issue 3-6: Whether to define requirements for supporting multiple options <Way forward >: - Proposals - Option 1: - Requirements/UE behaviour for UE supporting both option B-1-1 and C - Option 2: - Requirements for UE supporting both option B-1-1 and A.

Based on the discussing, all the options are optional capability. We understand to support the BWP operation without restriction, one of the options is enough. And even if multiple options are supported, the measurement should be based on network configuration. We think it is not typical for NW to configuration multiple options, so there is no need to define requirements for supporting multiple options.

Proposal 5: No need to define requirements for supporting multiple options.

<Agreement >: - It is a common RAN4 understanding that L3 measurements requirements impact for options B-1-1 and B-1-2 are not explicitly included in the WI objectives. - RAN4 work will focus on enabling L1 measurements for options B-1-1 and B-1-2 in Q2’2023. - In RAN4#107 RAN4 can continue discussion on whether any further clarifications are needed on intra-frequency measurements for UEs supporting options B-1-1 and B-1-2. ## Issue 4-4: Impact to intra-frequency measurement requirements for supporting option C During GTW on 17th April, following agreements were reached. <Agreement >: - It is a common RAN4 understanding that support of L3 measurements based on NCD-SSB for Option C can be beneficial, but it is not explicitly included in the WI objectives. Whether to support L3 measurements based on NCD-SSB is up to RAN decision. - RAN4 work will focus on enabling L1 measurements based on NCD-SSB in Q2’2023.

In last meeting, the handling of L3 measurement requirements impact was discussed, some companies think it is not explicitly included in the WI objective and should be left to RAN plenary to decide whether to include L3 measurement into the WID objective. But we understand when we discuss a new feature e.g. MG enhancement, the measurement requirements and impact on L1/L3 measurements are naturally included in the discussion and no need to state everything in the WI objective. Similarly, for this feature, when we introduce new UE implementation, the L1/L3 measurements should be clarified even if it is not explicitly stated in the WID.

Proposal 6: RAN4 can continue the discussions on L3 impact from Q3 directly with the current WID and allocated TU budget.

3 Summary

This contribution discusses the RRM requirements and specification support for option A, B-1-1, B-1-2 and C for the support for bandwidth part operation without restriction, and the following is proposed:

Proposal 1: No additional clarification is needed for timing requirements for option A and B-1-1.

Proposal 2: For option C, the current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support.

Proposal 3: For option B-1-2, existing SSB based BM/RLM/BFD measurement requirements apply and no additional measurement requirements are needed.

Proposal 4: For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%]. And the length for each interruption shall not exceed the RF retuning time (0.5ms for FR1 and 0.25ms for FR2).

Proposal 5: No need to define requirements for supporting multiple options.

Proposal 6: RAN4 can continue the discussions on L3 impact from Q3 directly with the current WID and allocated TU budget.

4 References

- R4-2306397 WF on BWP operation without restriction, vivo, RAN4#106bis-e

- R4-2306393 Topic summary for [106-bis-e][208] NR_BWP_wor, vivo, RAN4#106bis-e

- RP-222725 (R4-2220437) LS on BWP operation without bandwidth restriction, vivo

- R4-2304428 Discussion on the RRM impact of option A, B-1-1, B-1-2 and C for the support for bandwidth part operation without restriction, CATT, RAN4#106bis-e