3GPP TSG-RAN4 Meeting #107	                                                            R4-2307412

Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on RRM requirements of sidelink positioning

Source:	CATT

Agenda Item:	8.23.3.2

Document for:	Discussion

1 Introduction

In last meeting, RAN4 has discussed general RRM impact of the sidelink positioning with conclusions captured in [1]. RAN1#112bis-e meeting has also further discussed the work item and the agreements were captured in [2]. In this paper, we further discuss the potential RRM requirements for sidelink positioning and give our proposals.

2 Discussion

Based on RAN1#112bis-e meeting, the definitions of SL-PRS based RTOA and SL-PRS based AOA/ZOA were further discussed and agreed as Annex. In Rel-17 positioning, no RTOA and AOA/ZOA measurement period requirements are defined since it is for uplink measurement, but for SL positioning, all the measurement are performed by UE and the measurement period should be defined.

Proposal 1: Besides agreed RAN4 SL-based RSTD, Rx-Tx time difference, RSRP, RSRPP measurements, RAN4 also to define the SL based measurement period requirements for SL-PRS based RTOA and AOA/ZOA.

Based on RAN1 agreements, both first path and additional path report can be supported for SL-based RSTD, Rx-Tx time difference, RTOA, AOA and RSRPP measurement, although no performance requirements are expected, the report mapping for both first path and additional path needs to be defined. And as agreed in last meeting, the existing report mapping can be reused as baseline which should include both first path and additional path.

Agreement Support SL-based RSTD, Rx-Tx time difference, RToA, AoA, RSRPP measurement and report for the first path and optionally additional path. - No specification impact for how to set the additional path measurements - From RAN1 perspective, no performance requirements are expected to be defined for the additional-path measurements in Rel-18.

For the measurement period requirements, taking SL-PRS based RSRP as example, we understand the existing PRS-RSRP measurement period requirements without gap can be used as baseline, i.e. the following factors can be considered and included:

- Available SL-PRS periodicity considering muting

- Available time duration of SL-PRS resource

- UE processing capability

- The number of carriers

- The number of samples

- The number of beams

- Scaling factor due to TEG

- The definition of measurement window

- The measurement duration of the last sample

In last meeting, it was agreed to define UE requirements for SL PRS-based positioning measurements performed within a single SL BWP and carrier. So the SL-based measurement period requirements are defined for single carrier without gap.

In last meeting, the applicable number of samples for SL based measurement period requirements was discussed. Based on the following RAN1 agreement, AGC symbol is not considered as part of the SL PRS resource, so we understand there is no need to increase the number of samples reserved for AGC for measurement period requirements. And we think the SL based measurement period requirements can be defined based on single sample and further study whether to define requirements for multiple samples based on performance evaluation and RAN1 further progress on UE capability.

Agreement An AGC symbol preceding a SL PRS resource is not considered as part of the SL PRS resource itself.

For the number of beams, since the SL based measurement requirements are only defined for FR1, the number of Rx beam is 1.

For the impact of TEG, we think the Rel-17 TEG feature can be used for SL positioning and the scaling factor for Rx TEG defined in existing PRS measurement requirements can be reused.

If the requirements are defined for single sample, the SL-PRS periodicity, the definition of measurement window are not needed to be considered, but the available time during on the resource, the measurement duration on last sample and UE capability are still needed and need to wait further RAN1 progress.

Proposal 2: The following factors should be considered for SL-PRS based measurement requirements:

- Available SL-PRS periodicity considering muting

- Available time duration of SL-PRS resource

- UE processing capability

- The number of carriers

- The number of samples

- The number of beams

- Scaling factor due to TEG

- The definition of measurement window

- The measurement duration of the last sample

Proposal 3: RAN4 to define the SL based measurement period requirements based on single sample. FFS whether to define requirements for multiple samples based on performance evaluation and RAN1 further progress on UE capability.

Proposal 4: The scaling factor for Rx TEG defined in existing PRS measurement requirements can be reused for SL positioning.

Proposal 5: Wait for further RAN1 progress on SL-PRS resource configuration and UE processing capability.

In the current specification, the timing error limit requirements for sidelink UE based on different synchronization reference source including GNSS, NR cell, E-UTRAN cell and SyncRef UE. We can see that most of the requirements can be comparable to the Uu requirements. So we think the existing Te requirements can be reused for SL positioning, at least for the case when GNSS or SyncRef UE as synchronization reference source and no new transmission time adjustment requirements are needed.

For the impact when synchronization reference source change, we think we should firstly clarify the use case. We didn’t see the typical case for synchronization reference source change and it is not needed for single sample measurement. And even this case is considered, we understand the transmission timing requirements should be met for each source and would not impact the measurement period requirements. Then the impact would depend on whether different performance requirements are defined for different Te requirements.

Proposal 6: Reuse the existing Te requirements for SL positioning in Rel-18 and also no new transmit timing adjustment requirements for SL positioning are needed.

Proposal 7: The measurement period requirements are not impacted due to synchronization reference source change.

Proposal 8: The impact of synchronization reference source change on performance depends on whether to define different accuracy requirements for different Te requirements and should be discussed in performance part.

For the impact of SL-DRX, we prefer to prioritize non-DRX case for measurement period requirements same as the existing PRS measurement.

For the impact of mobility, we understand there is the same issue for Uu measurement, but we didn’t define the limitation on the requirements but report the measurement with time stamp and leave the implementation to LMF. For SL positioning, we may not need to define the requirements for this impact either. Although there is one difference for SL positioning that the coverage scenario is possible to changed, since we only define the requirements for PC5-only, we understand the requirements are not different for different coverage scenarios. So we think there no need to consider the scenario change, i.e. the requirements should still apply.

Proposal 9: RAN4 to prioritize non-DRX case for the measurement period for SL PRS-based measurements.

Proposal 10: The measurement period requirements still apply when there is change of network coverage scenario.

3 Summary

In this paper, we discuss the RRM aspects on sidelink positioning and the following proposals are given:

Proposal 1: Besides agreed RAN4 SL-based RSTD, Rx-Tx time difference, RSRP, RSRPP measurements, RAN4 also to define the SL based measurement period requirements for SL-PRS based RTOA and AOA/ZOA.

Proposal 2: The following factors should be considered for SL-PRS based measurement requirements:

- Available SL-PRS periodicity considering muting

- Available time duration of SL-PRS resource

- UE processing capability

- The number of carriers

- The number of samples

- The number of beams

- Scaling factor due to TEG

- The definition of measurement window

- The measurement duration of the last sample

Proposal 3: RAN4 to define the SL based measurement period requirements based on single sample. FFS whether to define requirements for multiple samples based on performance evaluation and RAN1 further progress on UE capability.

Proposal 4: The scaling factor for Rx TEG defined in existing PRS measurement requirements can be reused for SL positioning.

Proposal 5: Wait for further RAN1 progress on SL-PRS resource configuration and UE processing capability.

Proposal 6: Reuse the existing Te requirements for SL positioning in Rel-18 and also no new transmit timing adjustment requirements for SL positioning are needed.

Proposal 7: The measurement period requirements are not impacted due to synchronization reference source change.

Proposal 8: The impact of synchronization reference source change on performance depends on whether to define different accuracy requirements for different Te requirements and should be discussed in performance part.

Proposal 9: RAN4 to prioritize non-DRX case for the measurement period for SL PRS-based measurements.

Proposal 10: The measurement period requirements still apply when there is change of network coverage scenario.

4 References

- R4-2306351 WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT, RAN4#106bis-e

- Draft Report of 3GPP TSG RAN WG1 #112bis-e v0.1.0

Annex RAN1 agreements on SL positioning

RAN1#112bis-e

Reference signals

Agreement

For SL PRS sequence generation, no additional parameters other than the following input parameters are used: slot number, symbol number, and the parameter nID,seqSL-PRSnID,seqSL-PRS.

Agreement

TDM-based multiplexing in a slot of SL PRS from different UEs is NOT supported for a shared resource pool.

Agreement

SL PRS resource sets are not defined in Rel-18.

Agreement

(M, N) patterns with M > N with full staggering are supported.

- In the last (M-N) symbols, the SL PRS symbols are repeated with same order of comb offsets as in the first N symbols.

Agreement

An AGC symbol preceding a SL PRS resource is not considered as part of the SL PRS resource itself.

Agreement

For the SL PRS open-loop power control, a UE can be configured to use DL pathloss (between TX UE and gNB) only, SL pathloss (between TX UE and RX UE) only, or both DL pathloss and SL pathloss.

- The same principle as for PSSCH power control is applied for deciding which (i.e., SL, DL, or SL and DL) pathloss to use.

- FFS: SL pathloss reference for open-loop power control for SL PRS.

Agreement

At least for dedicated SL PRS resource pools, in addition to already-agreed (M, N) = (2, 2), (4, 4), fully staggered pattern with (M, N) = (6, 6) is supported.

- FFS: Other values of (M, N).

- FFS: Applicability to shared resource pools.

Agreement

- A SL PRS resource refers to a time-frequency resource within a slot of a dedicated SL PRS resource pool that is used for SL PRS transmission.

  - FFS: for a shared resource pool

- Characteristics associated with a SL PRS resource include at least:

  - SL PRS resource ID,

  - SL PRS comb offset and associated SL PRS comb size (N),

  - SL PRS starting symbol and number of SL PRS symbols (M),

  - SL PRS frequency domain allocation,

  - Note: Additional parameters can be included as/when identified.

  - FFS: other time domain aspects, if any

- A SL PRS resource is identified by a SL PRS resource ID that is unique within a slot of a dedicated SL PRS resource pool.

NOTE 1: The above does not imply need for signalling/(pre-)configuration of all these parameters

Agreement

For SL PRS sequence generation, one of the following options is down-selected to define the parameter nID,seqSL-PRS nID,seqSL-PRS:

- Option 1:nID,seqSL-PRS nID,seqSL-PRS is a higher layer parameter.

  - FFS: How the higher layer parameter is obtained, e.g., (pre-)configuration or via LPP/SLPP, etc.

- Option 2:nID,seqSL-PRS nID,seqSL-PRS is based on 12 LSB bits CRC of PSCCH associated with the SL PRS.

- Option 3: based on a combination of higher layer parameter from a configured ID list and 12 LSB bits of CRC of PSCCH associated with the SL PRS.

  - FFS: How the higher layer parameter/ID list is determined/obtained, e.g., (pre-)configuration or via LPP/SLPP, etc.

Agreement

For a dedicated SL PRS resource pool, options for SL pathloss reference for OLPC for SL PRS are (to be down-selected from):

- Option 1: SL PRS as pathloss reference

- Option 2: PSCCH DMRS as pathloss reference

- Option 3: Both Options 1 and 2

  - FFS: Selection between Option 1 and Option 2, including (pre-)configuration.

Agreement

For shared resource pools, a UE does not map SL-PRS and PSSCH DMRS in the same OFDM symbol(s).

Agreement

For SL pathloss-based OLPC for SL PRS in unicast, filtered RSRP is reported by a receiving UE.

Measurement and reporting:

Conclusion

- For SL-PRS based RTOA definition, the selection between SFN0 vs DFN0 and SFN vs DFN is determined based on the selection of the synchronization source for SL PRS transmission, as in legacy specifications.

  - When the UE selects a cell as the synchronization reference source, SFN0/SFN is used for SL-PRS based RTOA.

  - Otherwise, DFN/DFN0 is used for the definition of the SL-PRS based RTOA.

- FFS on indication of whether SFN or DFN is used along with the RTOA measurement reporting

Agreement

- SFN/DFN Initialisation Time can be provided by UE or by LMF.

  - FFS: which UEs can provide initialisation time (note: which may be decided by other WGs)

- FFS: further details of the definition of DFN Initialisation Time. For the definition of SL-PRS based RTOA, update the definition of reference timing as:

  - the reference timing is defined as T0+tSL-PRS, where

    - T0 is the nominal beginning time of SFN 0 or DFN0 provided by SFN/DFN Initialisation Time

    - tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively

    - FFS: The timing of SFN0/SFN for RTOA reference timing is determined by advancing DL SFN0/SFN with TA/2 when the UE is in-coverage and in connected state

Agreement

Support both the case with and without translation of the LCS to GCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement.

Agreement

SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a transmitting UE with respect to a reference direction, wherein the reference direction is defined:

- In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction

- In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901.

The SL AoA is determined at the receiving UE’s antennas for a SL channel corresponding to the transmitting UE.

Agreement

Support SL-based RSTD, Rx-Tx time difference, RToA, AoA, RSRPP measurement and report for the first path and optionally additional path.

- No specification impact for how to set the additional path measurements

- From RAN1 perspective, no performance requirements are expected to be defined for the additional-path measurements in Rel-18.

Agreement

For provision of assistance information for absolute SL positioning, the anchor UE location information can be provided to LMF or UE.

FFS: which UEs can receive the anchor UE location information (note: which may be decided by other WGs)

FFS on quality information of anchor UE location information.

Agreement

Support per ARP based measurement in sidelink positioning. The ARP related information can be reported along with the SL measurement.

FFS on details of ARP related information, including whether TEG ID can be reused for such purpose.

Agreement

For definition of SL-PRS based Rx-Tx measurement, further consider Alt1 and Alt3 until RAN1#113:

- Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement

- Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu

Agreement

SL reference signal time difference (SL RSTD) is the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi, where:

- TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j.

- TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j.

FFS: whether or not impact due to mobility or synchronization timing change should be considered for SL RSTD

Agreement

Support higher layer signaling for sidelink positioning measurement report and report triggering.

- Up to RAN2 to discuss detailed signaling design.

FFS on SCI based report triggering.

Resource allocation:

Agreement

For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a

- Dynamic grant

  - FFS Reuse DCI format 3_0 for signalling SL-PRS resource allocation or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point

- Configured grant type 1

  - the SL-PRS transmission(s) follows the higher layer configuration

- Configured grant type 2

  - Support activating and releasing the configured grant using a new DCI format 3_X or 3_0 (to be down-selected between the two DCI formats)

- The above mechanisms use NR Rel-16 mode-1 signaling as a starting point

- FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool.

- FFS: Further details

Agreement

For a dedicated resource pool for positioning:

- No additional slots are needed to be supported.

Agreement

For SL-PRS transmission, either dedicated resource pool(s) or shared resource pool(s) or both can be (pre-)configured in the only SL BWP of a carrier.

- A UE can be (pre-)configured with one or more dedicated SL resource pools.

- A UE can be (pre-)configured with one or more shared SL resource pools.

Agreement

Confirm the working assumption: Sensing-based and random selection can be allowed in the same resource pool.

- Note: It is possible to (pre-)configure a resource pool to exclusively use sensing-based resource allocation.

Agreement

For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point.

- Study at least the following aspects on potential changes over the existing congestion control mechanisms:

  - CBR and CR definition for SL-PRS

  - Which parameters  of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters

  - CR and CBR measurement time window

  - Congestion control processing time

  - Number of CBR ranges

  - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool

Agreement

With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits

- Opt. 1: SCI1-A & a 2nd stage SCI format are used for SL-PRS indication

  - FFS: Details including a new or existing 2nd stage SCI

Agreement

In a shared resource pool:

- SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot

  - With regards to PSSCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting:

    - Alt. A.1: Only TDMing is supported

    - Alt. A.2: Only FDMing of PSSCH and SL-PRS is supported

      - FFS: Rate-matched around SL-PRS REs and/or PRB/sub-channel-level FDMing are supported potentially for different cases

      - Note: Rate-matched around SL-PRS REs is not applicable to comb-1 SL-PRS

    - Alt. A.3: Both Alt. A.1 and A.2 are supported in the specification

  - With regards to PSCCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting:

    - Alt. B.1: Only TDMing is supported

    - Alt. B.2: TDMing or PRB/sub-channel-based FDMing is supported

  - The PSSCH is used for (downselect one of the following alternatives in RAN1#113 meeting):

    - Alt. C.1: 2nd SCI only

    - Alt. C.2: 2nd SCI and SL-SCH

    - Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH”

  - FFS: Handling of PT-RS and SL-PRS

Agreement

For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot.

- FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH

Agreement

For the scheme 2 sensing-based resource allocation, resolve the brackets below by:

- Alt. 2: Rel-16 resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Note: This means that Rel-17 partial sensing is not considered a starting point for the design

Agreement

For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications:

- Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion:

  - Option 1: SL-PRS

  - Option 2: PSCCH DMRS

  - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool)

- Modification 2: For the resource selection window:

  - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget

  - Option 2: the selection window is provided by higher layers

- Modification 3: For the SL-PRS priority:

  - Option 1: A single L1 SL-PRS priority is allowed in a resource pool

  - Option 2: Multiple L1 SL-PRS priority are allowed  in a resource pool

- Modification 4: For the definition of a candidate resource within the resource selection window:

  - Options TBD

- Modification 5: For the reservation interval of SL-PRS:

  - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

- Modification 6: For the sensing window length (T0):

  - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

  - Option 2: Equal to or larger than the largest reservation interval

  - Option 3: Provided by higher layers with values TBD

- Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%):

  - Options TBD

- Modification 8: For the pre-emption of the reserved resources:

  - Options TBD

- Note 1: Other potential modifications and/or other options within each modification are not precluded

- Note 2: Multiple options for each potential modification may be supported

Agreement

In Scheme 2, with regards to the triggering of SL-PRS,

- Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers

- Working assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A.

  - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A

  - FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS