3GPP TSG-RAN4 Meeting #107	                                                            R4-2307419

Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on RRM requirements of network energy saving

Source:	CATT

Agenda Item:	8.35.4

Document for:	Discussion

1 Introduction

In last meeting, the RRM requirements for network energy saving were discussed and the conclusions were captured in the approved WF [1]. In this paper, we have some further discussions on the possible RRM impact of SSB-less SCell operation based on the agreed WF and give our views.

2 Discussion

Scenarios:

In last meeting, RAN4 discussed the scenarios for SSB-less SCell operation and reached following agreement:

< Agreement >Issue 2-1-1: Which scenario to be considered for SSB-Less SCell operation for FR1 inter-band collocated CA Online session (Friday April 21, 2023) - Agreements - The following scenarios are considered for inter-band SSB-less SCell requirements definition - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell. - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis

It may be possible to set some condition to make all the cases feasible for SSB-less SCell activation. But we understand scenario 2 and 2a are not typical case and even for SSB-less SCell in intra-band CA, the TRS which is QCLed with the SSB of activated serving cell is needed. So we think scenario 1 should be prioritized for defining the requirements. Even considering scenario 2 and 2a, we think 2a is one of special case of scenario 2, and there is no need to define separate requirements for the two cases.

Proposal 1: Prioritize scenario 1 to define the requirements for SSB-less SCell operation.

Proposal 2: Do not define separate requirements for scenario 2 and 2a.

Conditions for SSB-less SCell operation:

< Agreement >Issue 2-1-2: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-1? Online session (Monday April 24, 2023) Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-1 - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

For the conditions under which SSB-less SCell can work, it seems straightforward to reuse the existing conditions on the reference timing difference and power imbalance for intra-band CA. But for inter-band CA, there is one issue that it is hard to make RTD smaller than 260ns even under co-located deployment. Considering the demands and implementation, maybe a middle value between 260ns and CP can be used, e.g. 3us for 15kHz.

Proposal 3: The conditions under which SSB-less SCell operation for scenario 1 are as below:

  - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell is within 3us.

  - The difference of the reception power with the FR1 inter-band active serving cell is within 6dB.

  - The TRS(s) of the SSB-less SCell is (are) QCLed with SSB(s) of the FR1 inter-band active serving cell.

SCell activation requirements:

It was agreed in last meeting that the SSB-less SCell activation delay requirements are to be defined for inter-band CA for FR1 co-located cells. When the conditions defined above can be met, the SSB-less SCell activation delay can be 3ms similar to intra-band continuous CA.

Proposal 4: For FR1 co-located SSB-less inter-band CA, when the conditions defined above are met, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms).

L1/L3 measurement

For SSB-less SCell activation, when the conditions about RTD, power imbalance and TRS are met, since the SSB-less SCell is leveraged the timing information from activated serving cell, the L1/L3 measurement can be skipped. If the conditions are not met but CSI-RS based measurement is supported and configured, L1/L3 measurement needs to be specified for SSB-less SCell operation.

Proposal 5: For SSB-less SCell activation, when the conditions about RTD, power imbalance and TRS are met, the L1/L3 measurement can be skipped. If the conditions are not met but CSI-RS based measurement is supported and configured, L1/L3 measurement needs to be specified for SSB-less SCell operation.

Others:

For scenario 1, when the conditions are met, the timing of activated serving cell can be used for SSB-less SCell and the TRS can be used for fine time tracking. And when the power imbalance condition is met, the AGC info of the activated inter-band FR1 serving cell can also be used for the target SSB-less SCell.

For the UE capability of SCellwithoutSSB, both reusing existing signaling and introducing new signaling can work, and it can be left to RAN2.

Proposal 6: For SSB-less SCell, when the power imbalance condition is met, the AGC info of the activated inter-band FR1 serving cell can also be used for the target SSB-less SCell.

Proposal 7: It is left to RAN2 to decide whether to reuse existing SCellwithoutSSB or introduce new signaling.

3 Summary

In this paper, we have some discussions on possible RRM impact for network energy saving, the following observations and proposals are given:

Proposal 1: Prioritize scenario 1 to define the requirements for SSB-less SCell operation.

Proposal 2: Do not define separate requirements for scenario 2 and 2a.

Proposal 3: The conditions under which SSB-less SCell operation for scenario 1 are as below:

  - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell is within 3us.

  - The difference of the reception power with the FR1 inter-band active serving cell is within 6dB.

  - The TRS(s) of the SSB-less SCell is (are) QCLed with SSB(s) of the FR1 inter-band active serving cell.

Proposal 4: For FR1 co-located SSB-less inter-band CA, when the conditions defined above are met, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms).

Proposal 5: For SSB-less SCell activation, when the conditions about RTD, power imbalance and TRS are met, the L1/L3 measurement can be skipped. If the conditions are not met but CSI-RS based measurement is supported and configured, L1/L3 measurement needs to be specified for SSB-less SCell operation.

Proposal 6: For SSB-less SCell, when the power imbalance condition is met, the AGC info of the activated inter-band FR1 serving cell can also be used for the target SSB-less SCell.

Proposal 7: It is left to RAN2 to decide whether to reuse existing SCellwithoutSSB or introduce new signaling.

4 References

- R4-2306369 WF on RRM requirements for NR network energy saving, Huawei, RAN4#106bis-e

- R4-2306260 Topic summary for [106-bis-e][229] Netw_Energy_NR, Huawei, RAN4#106bis-e