3GPP TSG-RAN WG4 Meeting #107	R4-2307464

Incheon, Korea, May 22 – May 26, 2023

Agenda item:	8.9.2.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on L3 part enhancement for FR2 SCell activation

Document for:	Discussion

- Introduction

At the last RAN4 #106bis-e meeting, RRM requirements for L3 part enhancement for FR2 SCell activation was discussion and WF was agreed [1]. There are four remaining issues as following.

- L3 reporting during SCell activation

- Beam related enhancement for L3 part

- RS related enhancement for L3 part

- Others for L3 par

In this contribution, we will pick up some remaining issues and propose our view on each remaining issue.

- Discussion

  - 1.	L3 reporting during SCell activation

During last meeting, the discussion that L3 reporting during SCell activation and RAN4 RRM session send LS to RAN2 for signalling design (network triggering report of L3 measurement result) [2]. The LS content is duplicated as follows:

- How to configure the report of L3 measurement result - How to trigger the report of L3 measurement result - Contents of the report of L3 measurement result and how to send the report.

RAN4 following agreements in RAN4 #106-bis-e meeting, part of L3 reporting SCell activation is duplicated as follows[1].

- Agreements (GTW, Monday Apr 17, 2023) - The L3 measurement report for unknown FR2 Scell activation after Scell activation command is triggered by Scell activation command. FFS whether to have additional events for report triggering

part of L3 reporting during SCell activation is duplicated as follows [1]

Issue 1-1-6: If measurement results are available, the UE will report them to the NW. How to determine the measurement result is available? FFS: - Option 1 (Apple, OPPO, Huawei, Xiaomi, vivo): - No need to define criteria to determine the L3 measurement result is available or not for FR2 unknown SCell activation enhancement. - Option 2 (Nokia, Ericsson): - The L3 measurement report is considered as valid only if it fulfils the measurement requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2). - Option 3 (Intel): - If UE already has available L3 measurement result when Scell activation command is triggered, the measurement result can be reported only if it satisfied the following conditions: - Measurement should satisfy the requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2) - the time duration between measurement and Scell activation command is less than [X]s. - Option 4 (MTK): - RAN4 to define a new window [W]s to determine the freshness of the available L3-RSRP measurements at the UE prior to the reception time of Scell activation command. - Proposal 6: If L3-RSRP measurements are within [W], then the UE can send L3-RSRP report. - Option 6 (vivo): - The criteria to determine whether a measurement result is valid (i.e. available), including the signal quality thresholds and the maximum time-delta of the measurement before the reporting, are configured by network - Option 7 (CMCC, ZTE): - for L3 measurement report after Scell activation command, the reported SS-RSRP, SS-RSRQ, and SS-SINR measurements need to meet the accuracy requirements spedified in Clause 10. - Option 8 (ZTE): - To address the concern of the quick report is valid or not, the verification lies in two aspects: - 1)Whether the measurement report is fresh enough. - 2)Whether the quality of measurement result in the report is good or available enough. - The 1st aspect can be ignored since same issue exists in legacy L3 report. The 2nd aspect should be considered to guarantee the NW can obtain useful RSRP and SSB index info from the report. - Option 9 (Apple, CMCC, LGE, CTC, QC, OPPO): - Option 1 and the valid reporting must meet the existing measurement delay/accuracy requirement.

We support with Option 2. The essence of this issue is that if a UE has a "valid" L3 measurement report, it should report it to the NW. In other words, the criteria for validity need to be clearly defined. If the criteria for validity when reporting to the NW are unclear, the shortening effect may be ambiguous depending on the UE implementation. Therefore, it is possible to communize the effectiveness quantitatively by considering the current regulations as a base.

Observation 1: If the availability of L3 measurements is not explicitly defined, the effect of L3 measurements may be "depending on the UE implementation" or "ambiguity in activation reduction effect".

Proposal 1: The L3 measurement report is considered as valid only if it fulfils the measurement requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2).

Issue 1-1-7: FFS on necessity of L3 measurement reporting if UE has no valid measurement results?

FFS: - Option 1 (Apple, ZTE, CTC, OPPO, Ericsson, Xiaomi, HW, vivo, QC): No need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results. - Option 2 (Nokia, Intel, vivo, Ericsson): The UE may continue the measurement and report a valid L3 measurement result when it is ready, if there is no valid L3 measurement result at the time of Scell activation. - Option 3 (LGE, CTC, MTK): If a UE has no valid measurement results, UE needs to report even the lowest RSRP reported value to notice whether a UE has valid measurement results to the network. - Option 4 (vivo, MTK): If the reporting of the results is carried by uplink L1L2 signaling, and UE has no valid measurement results at the L1L2 reporting occasion, UE may report -Inf value, e.g. ‘RSRP<-156dBm’. - Option 5 (MTK): when report triggering conditions are met, but UE has no fresh L3-RSRP measurements within [W] period, then instead of sending the report UE indicates unknown status for the target Scell.

This issue is the behaviour when the UE does not have a valid L3 report. If the UE does not have a valid L3 report by a certain time, it may be better to be able to continue measurements and report valid L3 measurements when ready, which may lead to a shorter SCell activation delay.

Proposal 1: The UE may continue the measurement and report a valid L3 measurement result when it is ready, if there is no valid L3 measurement result at the time of Scell activation.

- Conclusion

In this contribution, we proposed our views on L1 part enhancement for FR2 SCell activation.

Observation 1: If the availability of L3 measurements is not explicitly defined, the effect of L3 measurements may be "depending on the UE implementation" or "ambiguity in activation reduction effect".

Proposal 1: The L3 measurement report is considered as valid only if it fulfils the measurement requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2).

Proposal 2: The UE may continue the measurement and report a valid L3 measurement result when it is ready, if there is no valid L3 measurement result at the time of Scell activation.

References

[1] 3GPP, R4-2306315, Apple, “WF on R18 RRM enhancement- FR2 SCell activation delay reduction”

[2] 3GPP, R4-2306321, RAN WG 4, “LS on FR2 unknown SCell activation enhancement”