3GPP TSG-RAN WG4 Meeting #107 	R4-2307469

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Advanced Receivers - Simulation results

Agenda item:	8.19.1.2

Document for:	Discussion

Introduction

This document details Nokia’s current simulation results for alignment on the WID for advanced receivers for MU-MIMO scenario. Agreements were made in RAN4#106-bis [1] for facilitating companies to provide simulation results for Phase I.

Simulation assumptions for Phase I

Table 1 provides an overview of the already agreed simulation assumptions for Phase I taken from the WF [1]:

Table 1: Agreed simulation assumptions for Phase I

| Reference receiver assumption for E-MMSE-IRC | WRX,1k,l=H1H(k,l)R-1 R= P1H1k,lH1Hk,l+Pjj=2NUEHjk,lHjHk,l+1Nrek,l∈DMRS of NUEr(k,l)r(k,l)H, rk,l=rk, l-j=1NUEHjk,ldjk,l . |
| --- | --- |
| Reference receiver for phase I simulation | Consider both R-ML and E-IRC in initial evaluation stage |
| Rank allocation for the target and co-scheduled UEs, with 1 co-scheduled UE | 2Rx UE: 1+1 4Rx UE: 2+2 |
| DMRS port configurations for the target and co-scheduled UEs | - Use different CDM groups for: - rank 2 (DMRS port 0, 1) + 2 (DMRS port 2, 3) - rank 1 (DMRS port 0 for target UE) +1 (port 1) +1 (port 2) - Use the same CDM group for rank 1+1 |
| Antenna configuration | For rank 1+1: cover 2T2R For rank 2+2, rank 1+1+1: 4T4R |
| Channel model | Use TDLC300-100 when the rank of the target UE is 1 Use TDLA30-10 when the rank of the target UE is 2 |
| Antenna correlation | Rank 1+1: ULA medium Rank 2+2: ULA Low Rank 1+1+1: XPL medium |
| Precoder selection target and co-scheduled UEs | Single panel Type 1 For rank 1+1: Random PMI selection For rank 2+2: Orthogonal PMI selection |
| QCL assumptions | Assume all scheduled DMRS ports have same QCL assumptions |
| Evaluation metric | Reuse the Rel-17 MMSE-IRC phase I evaluation assumptions captured in TR38.833 as a start point. |
| The number of co-scheduled UEs | Option 1: 1 co-scheduled UE as higher priority Option 2: Cover 2 co-scheduled UEs at least in phase I study |
| DMRS sequence for the co-scheduled UE | For initial simulation in phase I, assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s) |
| MCS for the target UE | For rank 1: MCS 13 For rank 2: MCS 13 and MCS 17 |
| Modulation order for the co-scheduled UE | For rank 1+1: QPSK (high priority) For rank 2+2: 64QAM (high priority) For rank 2+2: QPSK (high priority) For rank 1+1 (64QAM) +1 (QPSK) (lower priority) For rank 1+1 (64QAM) +1 (16QAM) (lowest priority) |
| PDSCH resource allocation for the target and co-scheduled UE | Maximum transmission bandwidth configuration for all UEs. Full OFDM symbol allocation. |
| Assumptions on the required information | Assume the needed parameters of the co-scheduled UE are all known to UE (upper bound for the potential performance gain). |
| MCS | Table 1 |

Based on the information in Table 1, we provide the simulation results on the highest priority cases for this meeting, listed in Table 2.

Table 2 - Simulation Overview – high priority cases

| Case | # Co-scheduled UE | Rank target UE | Rank Co-scheduled UE | Modulation order target UE | Modulation order co-scheduled UE | MIMO | Channel model | Precoder selection for the Co-scheduled UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1A | 1 | 1 | 1 | QPSK (MCS 4) | QPSK (MCS 4) | 2Tx 2Rx ULA medium | TDLC300-100 | random |
| 1 |  |  |  | 16QAM (MCS 13) |  |  |  |  |
| 2 |  | 2 | 2 |  | 64QAM (MCS19) | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal |
| 3 |  |  |  |  | QPSK (MCS 4) |  |  |  |
| 4 |  |  |  | 64QAM (MCS 17) | 64QAM (MCS19) |  |  |  |
| 5 |  |  |  |  | QPSK (MCS 4) |  |  |  |
| Note1: Case 1A is not a high-priority case but is listed in [1] as a potential addition left for discussion in the RAN#107 meeting. Note2: All tests consider the use of MCS index Table 1. Note3: FDRA allocation is taken to occupy the full CHBW (52 PRBs). |  |  |  |  |  |  |  |  |

Simulation Results

This section displays our current simulation results. Normalized throughput curves are shown for the baseline receiver MMSE-IRC (IRC) against the E-MMSE-IRC (E-IRC) and R-ML receivers. A black reference line is included to highlight the SNR @ 70% of maximum throughput metric for each receiver, captured in Table 3.

Table 3 – Summary of Receiver performance.

| Case | # Co-Sch UE* | Rank target UE | Rank Co-Sch UE | MO* Target UE1 | MO Co-Sch UE1 | MIMO | Channel model | Precoder Co-Sch UE | Nokia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | SNR@70%maxTP (dB) |  |  | Gain of R-ML | Gain of E-IRC | Gain of R-ML over E-IRC |
|  |  |  |  |  |  |  |  |  | R-ML | E-IRC | IRC (baseline) |  |  |  |
| 1A | 1 | 1 | 1 | MCS4 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | 5.89 | 8.53 | 13.68 | 7.79 | 5.15 | 2.65 |
| 1 |  |  |  | MCS13 |  |  |  |  | 14.22 | 22.48 | 25.73 | 11.51 | 3.25 | 8.26 |
| 2 |  | 2 | 2 |  | 64QAM | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal | 12.69 | 13.23 | 14.23 | 1.54 | 1.00 | 0.54 |
| 3 |  |  |  |  | QPSK |  |  |  | 9.95 | 13.24 | 14.22 | 4.27 | 0.98 | 3.29 |
| 4 |  |  |  | MCS17 | 64QAM |  |  |  | 17.98 | 17.45 | 18.32 | 0.34 | 0.87 | -0.53 |
| 5 |  |  |  |  | QPSK |  |  |  | 12.90 | 17.45 | 18.32 | 5.42 | 0.87 | 4.55 |
| Note1: All tests consider the use of MCS index Table 1. Note2: FDRA allocation is taken to occupy the full CHBW (52 PRBs). Note*: Co-scheduled UE (Co-Sch UE) and Modulation order (MO). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3 offers a summary of the results above, along with each receiver’s gain over the baseline IRC. Some observations follow below:

- Results show that both E-IRC and R-ML offer improved receiver performance over the IRC baseline for all the tested cases.

- R-ML generally outperforms E-IRC but the improvement margin is reduced for higher modulation orders of the co-scheduled UE.

Case 1A:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1A | 1 | 1 | 1 | MCS4 | MCS4 | 2Tx 2Rx ULA medium | TDLC300-100 | random |

Figure 1 - Case 1A (2T2R Rank 1+1 TDLC300-100, medium correlation, MCS4-MCS4, random precoder).

Case 1:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | MCS13 | MCS4 | 2Tx 2Rx ULA medium | TDLC300-100 | random |

Figure 2 -Case 1 (2T2R Rank 1+1 TDLC300-100, medium correlation, MCS13-MCS4, random precoder).

Case 2:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 2 | 2 | MCS13 | MCS19 | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal |

Figure 3 - Case 2 (4T4R Rank 2+2 TDLA30-10, low correlation, MCS13-MCS19, orthogonal precoder).

Case 3:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 1 | 2 | 2 | MCS13 | MCS4 | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal |

Figure 4 - Case 3 (4T4R Rank 2+2 TDLA30-10, low correlation, MCS13-MCS4, orthogonal precoder).

Case 4:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 1 | 2 | 2 | MCS17 | MCS19 | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal |

Figure 5 – Case 4 (4T4R Rank 2+2 TDLA30-10, low correlation, MCS17-MCS19, orthogonal precoder).

Case 5:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 1 | 2 | 2 | MCS17 | MCS4 | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal |

Figure 6 - Case 5 (4T4R Rank 2+2 TDLA30-10, low correlation, MCS17-MCS4, orthogonal precoder).

Conclusion

This paper presents Nokia's current simulation results on the WID for advanced receivers for MU-MIMO scenario. It includes the configurations agreed in RAN#106-bis to be highest priority.

- References

- R4-2305914 WF on advanced receiver for MU-MIMO scenario