3GPP TSG-RAN WG4 Meeting #107 	R4-2307474

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On MultiRx Demodulation performance and CSI requirements - PMI

Agenda item:	8.8.4.3

Document for:	Discussion

Introduction

In RAN4#106-bis-e good progress was made on the topic of requirements for PMI (see WF [1]).

In the following we will provide our view on the remaining open issues and make new observations and proposals where needed.

Discussion

Simulation assumptions

Number of CSI-RS ports

In RAN4#106-bis the number of CSI RS ports were discussed with the following remaining options in the WF [1]:

Issue 3-3-2: Number of CSI-RS ports <Way forward > - Proposals - Option 1: 8-port per TRP. - Option 2: Define CSI requirements with 4-port per TRP as a minimum. In addition to requirements with 4-port per TRP, requirements with 8-port per TRP can be included. - Option 3: 4-port per TRP. - Option 4: 2-port per TRP.

We see a minimum of 2 CSI-RS ports per TRP to be required (i.e. 4 CSI-RS ports in total), however in a practical deployment we see it more likely a gNB will select to use 4 CSI-RS ports per TRP. As the practical deployment will use 4 CSI-RS ports per TRP CSI requirements should be defined for this higher complexity case.

4 CSI-RS ports per TRP will be seen for most practical deployments.

Use 4 CSI-RS ports per TRP (option 3)

Whether to assume LI in report quantity for multi-Rx FR2

In RAN4#106-bis it was discussed weather to assume LI in report quantity for multi Rx (WF [1]):

Issue 3-3-4: Whether to assume LI in report quantity for multi-Rx FR2 <Way forward > - Proposals - Option 1: Include LI in the report quantity to facilitate PTRS port mapping. - Option 2: No

It seems not useful in testing to have a dynamic PT-RS port allocation, assuming PT-RS will be configured in the first place. As such, we see it sufficient to fix the PT-RS port as per usual.

It seems sufficient to fix the PT-RS port as per usual.

Do not assume LI in report quantity for MultiRx (option 2)

Conclusion

This paper presents Nokia's view on the open issues with relation definition of PMI requirements for MultiRx Demodulation performance.

In the paper, the following Observations and Proposals were made:

- 4 CSI-RS ports per TRP will be seen for most practical deployments.

- Use 4 CSI-RS ports per TRP (option 3)

It seems sufficient to fix the PT-RS port as per usual.

Do not assume LI in report quantity for MultiRx (option 2)

- References

- R4-2306000 - WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception

- 38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements v17.8.0