3GPP TSG-RAN WG4 Meeting #107		R4-2307480

# Incheon, KR, May 22 – May 26, 2023

Source:	LG Electronics

Title:	TP to TR on con-current operation on Uu and sidelink

Agenda item:	8.31.2.2

Document for:	Approval

- Introduction

The requirement of con-current operation on Uu and sidelink has been discussed with some general aspects and some requirements were agreed in the WF [1].

Based on the WF, the TP to TR captures the agreed requirements. Table 1.1 is for better checking as below:

Table 1.1: Status of issues for con-current operation on Uu and sidelink

| Issue | Status |
| --- | --- |
| 1-1-1: Limited number of example band combination for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-1-2: Example band combination for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-2-1: Con-current operation of Uu@Un-licensed and SL@Un-licensed | Agreed |
| 1-2-1: Con-current operation of Uu@Un-licensed and SL@licensed | Agreed |
| 1-2-3: Con-current operation of intra-band SL@Un-licensed | Agreed |
| 1-3-1: Basis of SL-U for con-current operation | Agreed |
| 1-3-2: Applicable channel bandwidth for con-current operation of Uu@Licensed and SL@Un-licensed | Agreed |
| 1-3-3: RF architecture reference of Uu@Licensed and SL@Un-licensed | FFS |
| 1-4-1: Targeted power class for Uu@Licensed and SL@Un-licensed | Initial agreement |
| 1-4-2: MPR/A-MPR for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-4-3: Configured transmitted power for Uu@Licensed and SL@Un-licensed | Initial agreement |
| 1-4-4: UE coexistence requirements for Uu@Licensed and SL@Un-licensed | Initial agreement |
| 1-4-5: General spurious emission for Uu@Licensed and SL@Un-licensed | Initial agreement |
| 1-4-6: Additional spurious emission for Uu@Licensed and SL@Un-license | FFS |
| 1-4-7: Other Tx requirements for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-5-1: MSD for Uu@Licensed and SL@Un-licensed | Initial agreement |
| 1-5-2: Other Rx requirements for Uu@Licensed and SL@Un-licensed | Agreed |

- Reference

[1] R4-2306636 		WF on NR SL con-current operation and co-channel coexistence scenario of LTE V2X and NR V2X, LG Electronics

- Annex TP to TR 38.786

< START OF CHANGE #1 >

- 5.1	Operating bands

5.1.1	Operating bands for single carrier operation in unlicensed band

5.1.2	Operating band combinations for inter-band con-current operation

It has been agreed to use  n78@licensed band + n46@unlicensed band as the example band combination for Uu@Licensed and SL@Un-licensed.

The below inter-band con-current operation has been agreed not to be supported in Rel-18.

  - Uu@Un-licensed and SL@Un-licensed

  - Uu@Un-licensed and SL@licensed

  - Intra-band SL@Un-licensed

NR SL operation is designed to operate concurrently with NR uplink/downlink on the operating band combinations listed in Table 5.1.2-1.

- Table 5.1.2-1 Band combinations of inter-band con-current operating bands

| NR SL inter-band con-current operating Band | NR Band | Interface |
| --- | --- | --- |
| SL_n78-n46 | n78 | Uu |
|  | n46 | PC5 |

- 5.2	Channel bandwidth

5.2.1	Channel bandwidth for single carrier operation in unlicensed band

5.2.2	Channel bandwidth for inter-band con-current operation

It has been agreed to apply each channel bandwidth that is supported in a band for Uu and a band for SL-U.

For NR SL inter-band con-current operation in FR1, the NR channel bandwidths and SL channel bandwidths for each operating band are specified in Table 5.2.2-1.

Table 5.2.2-1: Inter-band con-current operation configurations

| NR SL inter-band con-current operating configuration | NR Band | Interface | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| SL_n78A-n46A | n78 | Uu | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | n46 | PC5 | 20, 40, 60, 80, 100 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 in TS38.101-1. |  |  |  |  |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

- 6.2	Tx requirements for inter-band con-current operation

It has been agreed to consider the functionality and requirements of single carrier SL-U operation as a basis of SL-U  and the functionality and requirements of single carrier Uu operation as a basis of Uu in con-current operation.

6.2.1	Maximum output power for inter-band con-current operation

It has been agreed to target the following power classes.

  - PC3 Uu@Licensed + PC5 SL@Un-licensed

  - PC3 Uu@Licensed + PC3 SL@Un-licensed can be considered as 2nd priority if PC3 SL is agreed in a single carrier at unlicensed band

6.2.2	UE maximum output power reduction for inter-band con-current operation

It has been agreed to refer to the existing requirement for NR Uu and refer to the SL-U requirement if specified.

6.2.3	UE additional maximum output power reduction for inter-band con-current operation

It has been agreed to refer to the existing requirement for NR Uu and refer to the SL-U requirement if specified.

6.2.4	Configured transmitted power for inter-band con-current operation

It has been agreed to consider the existing requirement for NR V2X con-current operation (6.2E.4.2 in TS38.101-1)  as starting point.

6.2.5	Minimum output power for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

6.2.6	Transmit OFF power for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

6.2.7	ON/OFF time mask for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

6.2.8	Power control for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

6.2.9	Transmit signal quality for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

6.2.10	Spectrum emission mask for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

6.2.11	ACLR requirements for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

6.2.12	Spurious emissions for inter-band con-current operation

6.2.13	Spurious emission band UE co-existence for inter-band con-current operation

6.2.14	Transmit intermodulation for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

< END OF CHANGE #2 >

< START OF CHANGE #3 >

- 7.2	Rx requirements for inter-band con-current operation

7.2.1	Reference sensitivity power level for inter-band con-current operation

7.2.2	Maximum input level for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

7.2.3	Adjacent Channel Selectivity for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

7.2.4	Blocking characteristics for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

7.2.5	Spurious response for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

7.2.6	Intermodulation characteristics for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

< END OF CHANGE #3 >