3GPP TSG-RAN WG4 Meeting #107	R4-2307529

Incheon, South Korea, 22– 26 May, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on BS RF requirement of network Energy savings

Agenda Item:	5.35.2

Document for:	Approval

Introduction

There were some initial discussion on the R18 WI Network energy savings for NR in the last meeting. The WF was agreed as following [1]:

Issue 2-1-1: Priorities for feasibility study

- Agreement：

  - RAN4 prioritize the discussion on SSB-less Scell operation for inter-band CA for FR1 and co-located cell.

Issue 2-2-6: How to define the RF requirements for inter-band CA for SSB-less Scell operation

- Agreement:

  - No impact to the legacy requirements of inter-band CA TAE defined in TS38.104. Whether or not to define feature-specific TAE requirements for SSB-less operation (NW enabling energy savings) for FR1 co-located inter-band CA is FFS.

Issue 2-3-1: RF requirements for Cell DTX

- Agreement:

  - FFS on the necessity to define ON-OFF power requirement waiting for more inputs from RAN1/2.

Issue 2-4-1: RF requirements for spatial domain techniques

- Agreement:

  - FFS on the necessity to the updates in manufacturer declarations, switching period and DL EVM requirement waiting for more inputs from RAN1/2.

This paper we further discuss these issues and give our proposals.

Discussion

Feasibility of SSB-less Scell operation for FR1 co-located inter-band CA

In the revised WID of Network energy savings for NR [2] , RAN4 aims to extend SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible.

It is worth noting that the current specification has already supported SSB-less SCell operation for intra-band contiguous CA. From RF perspective, the prerequisites are as follows:

1) RTD（Receive Timing Difference）between the SSB-less SCell and the FR1 intra-band contiguous active serving cell is within 260ns.

2) The received power difference between target SCell and the active serving cell with SSB is <= 6dB.

To initiate the feasibility study of SSB-less SCell operation for FR1 co-located inter-band CA, we can use these two conditions as a starting point. If certain scenarios involving FR1 co-located inter-band CA satisfy the same two conditions, it can be inferred that these scenarios are also capable of supporting SSB-less SCell operation.

We know RTD is composed of three factors: the TAE at the BS, the propagation delay over the air interface, and the time delay at the UE. In the case of intra-band contiguous CA, the TAE at the BS must not exceed 260ns. Since the two component carriers are contiguous in frequency, they are usually implemented with the same RU, and naturally the same baseband, the propagation delay over the air interface and the time delay at the UE is negligible. Therefore, the RTD is primarily dominated by the TAE at the BS.

- Figure 1 the three factors of RTD

The received power difference between target SCell and the active serving cell with SSB, is primarily affected by the output power and the path loss in transmission. For intra-band contiguous CA , the two CA component carriers are close to each other in frequency (no gap at all) and their target coverage areas usually are the same or similar. Therefore, the received power difference can be limited to 6 dB. In theFR1 co-located scenario, the target coverage areas can be assumed to be the same or similar. For some inter-band CA combinations, although their frequency spectrums are not very close to each other, their frequency gap still can be small enough to meet <=6dB as the propagation loss can be different about 20*log10(f).

- Table 1 Preliminary estimated power difference for FR1 and co-located cells

| CA combinations | f_low (MHz) | f_high (MHz) | 20*log10(f_high/f_low) (dB) | note |
| --- | --- | --- | --- | --- |
| CA_n1-n3 | 1805 | 2170 | 1.6 | 1.8G+2.1G |
| CA_n1-n7 | 1805 | 2690 | 3.5 | 2.1G+2.6G |
| CA_n2-n10 | 1990 | 2170 | 0.8 | 1.9G+2.1G |
| CA_n5-n8 | 869 | 960 | 0.9 | 850M+900M |
| CA_n5-n14 | 758 | 869 | 1.2 | 700M+850M |
| CA_n8-n28 | 758 | 960 | 2.1 | 700M+900M |
| CA_n8-n20 | 791 | 960 | 1.7 | 800M+900M |
| CA_n41 | 2496 | 2690 | 0.7 | 2.6G |

From Table 1, it is can be observed that for many inter-band CA combinations, the received power difference can be meet within <= 6dB.The list can go on and on.

There are various co-located deployment scenarios on the live network, as the following Figure 2:

- Figure 2 Scenarios for inter-band CA for FR1 and co-located cells

In Case 1, inter-band CA, at least some combinations, can be implemented with shared BB and RU, typically a BS supporting multi-band operation, which means that the majority of the components are shared by the component carriers. Currently, BSs supporting multi-band operation are widely available in the market, including 700M+800M, 700M+850M, 700M+900M, 850M+900M, 1.8G+2.1G, 1.9G+2.1G and 2.1G+2.6G and so on. Therefore, the TAE could be controlled in the same way as intra-band contiguous carrier aggregation, ensuring it does not exceed 260ns.

Although these band combinations are not so close in frequency as intra-band contiguous CA, as examples, for CA_n8-n20（900M+800M）, CA_n8-n28 (900M+700M) , CA_n5-n14 (850M+700M), their frequency gap（separately 169M, 202M, 136M）is no much larger than some of the intra-band contiguous CA, such as CA_n41 which the frequency span is 194 MHz. So we can conclude that inter-band CA, at least for some combinations, propagation delay is no more than that of intra-band contiguous CA. Therefore, it can be considered that the air interface propagation delay difference can be ignored at least in some combinations.

So far, in Case 1 where the CA is implemented with the same BB and RF module, it is feasible to support SSB-less Scell operation. This is because the TAE could be controlled in the same way as intra-band contiguous carrier aggregation, ensuring it does not exceed 260ns. Additionally, the air interface propagation delay difference can be ignored.

Proposal 1: In In the case of inter-band CA implemented with the same BB and RF module, the TAE could be controlled within 260ns. Additionally, the air interface propagation delay difference can be ignored. The received power difference can be meet within <= 6dB. So it is feasible to support SSB-less Scell operation.

Neither Case 2A nor case 2B inter-band CA are implemented with the same RU. Their CA component carriers are from independent RUs. In Case 2A, BB are shared but RUs are not shared. In Case 2B, neither BB nor RU are shared. Compared with Case 1, Case 2A and Case 2B may be a little more complex TAE/RTD, which depended on implementation.

The LTE inter-band CA TAE requirements (including co-located and non -o-located) in [3]:

-	For inter-band carrier aggregation, with or without MIMO or TX diversity, TAE shall not exceed 260ns.

Besides that, it is worth noting that many NR-capable RUs in the network also support MSR (including LTE), so we believe it is naturally they support the 260 ns TAE requirement of LTE inter-band CA. Therefore, a large number of RF modules can meet the LTE inter-band CA TAE requirement of 260 ns.

As for NR, the TAE of inter-band carrier aggregation is relaxed to 3µs considering especially non-co-located scenarios. However, in co-located scenarios, the TAE performance is better than 3µs and 260 ns is feasible.

Observation 1: Many RUs in the network that support NR also support LTE, which makes it natural for them to support the 260 ns inter-band CA TAE.

Observation 2: The TAE for NR inter-band CA has been relaxed to 3µs, particularly in non-co-located scenarios. However, in co-located scenarios, the TAE performance is much better than 3µs and achieving 260 ns is feasible.

Based on the above observations, we believe that at least MSR BS which supporting both LTE and NR, can meet the TAE requirement of 260 ns in NR FR1 co-located inter-band CA scenarios.

Proposal 2: Most, if not all, of the NR-capable RUs in the network also support MSR, including LTE. As a result, the TAE requirement of 260 ns can be met in at least some of co-located inter-band CA scenarios for NR FR1.

We noticed that in the previous meeting, it was proposed to categorize the SSBless inter-band CA scenario as follows [4]:

Scenario 1) Strong correlated gNB beam is applied for SSB-less Scell where small frequency distance between reference cell and the SSBless Scell is considered.

Scenario 2) Weak correlated gNB beam is applied for SSB-less SCell where general inter-band CA is considered.

In our opinion, Scenario 1) with small frequency distance, can typically be implemented with BS supporting multi-band operation. On the other hand, Scenario 2) for general inter-band CA, can be implemented with single-band BSs.

RF requirements for Cell DTX

The Cell DTX (discontinuous transmission) network saving technology involves switching the transmission between turn on and turn off for both the BS and the UE. This is similar to TDD, but without the switch between transmission and reception.

DTX is usually implemented through various shutdowns, such as shutdown of a carrier, a PA, a TRX channel, or others. These requirements for DTX are related to the switch response time of the PA, TX channel. However, this is not a significant issue as it occurs within the transmitter period or frequency band and does not interfere with others.

Proposal 3: For Cell DTX, there is no need to define switch time and spectrum as it operates within its own transmitter period or frequency band and does not cause interference with other signals.

RF requirements for spatial adaptation domain techniques

As for the spatial domain techniques，it is possible to flexibly enable or disable individual or multiple antenna elements that are associated to a logical antenna port. This is similar somewhat to Cell DTX. It's in the transmitter period or frequency band and doesn't interfere with others.

In terms of spatial domain techniques, it is possible to selectively enable or disable individual or multiple antenna elements that are linked to a logical antenna port. This functionality is akin to Cell DTX to some extent, it is only occurred during the transmitter period or frequency band without causing interference to other signals.

Proposal 4: For Spatial adaptation domain techniques, there is no need to define new requirement within its own transmitter period or frequency band and does not cause interference with other signals.

Conclusions

In this contribution, we discuss the Network energy savings, especially for the SSB-less SCell operation for inter-band CA for FR1 and co-located cells. Our observations and proposals are as following:

Observation 1: Many RUs in the network that support NR also support LTE, which makes it natural for them to support the 260 ns inter-band CA TAE.

Observation 2: The TAE for NR inter-band CA has been relaxed to 3µs, particularly in non-co-located scenarios. However, in co-located scenarios, the TAE performance is much better than 3µs and achieving 260 ns is feasible.

Proposal 1: In In the case of inter-band CA implemented with the same BB and RF module, the TAE could be controlled within 260ns. Additionally, the air interface propagation delay difference can be ignored. The received power difference can be meet within <= 6dB. So it is feasible to support SSB-less Scell operation.

Proposal 2: Most, if not all, of the NR-capable RUs in the network also support MSR, including LTE. As a result, the TAE requirement of 260 ns can be met in at least some of co-located inter-band CA scenarios for NR FR1.

Proposal 3: For Cell DTX, there is no need to define switch time and spectrum as it operates within its own transmitter period or frequency band and does not cause interference with other signals.

Proposal 4: For Spatial adaptation domain techniques, there is no need to define new requirement within its own transmitter period or frequency band and does not cause interference with other signals.

Reference

[1] R4-2306639，WF on RF feasibility study of Network Energy Savings for NR，Huawei, RAN4#106bis, April 17-26, 2023

[2] RP-230566, WID revision: Network energy savings for NR, Huawei, RAN#99, March 20-23, 2023

[3] 3GPP TS 36.104 V18.1.0 (2023-03), Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception

[4] R4-2304387, Discussion on requirements of SSB-less Scell at inter-band CA for FR1, Qualcomm Incorporated, RAN4# 106bis-e April 2023