3GPP TSG-RAN WG4 Meeting # 107		R4-2307599

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.27.5

Source: 	CMCC

Title: 			 Discussion on RRM core requirements for NR NTN enhancement

Document for:	Discussion

- Introduction

In RAN4#106bis-e meeting, RAN4 discussed the RRM core requirements for NR NTN enhancement, the WF has been approved in [1].

In this contribution, we further discuss the RRM impaction of NR NTN enhancement based on the progress of RAN2.

- NTN-TN and NTN-NTN cell reselection enhancements

Issue 4-1: NTN-NTN Cell reselection enhancements for earth moving cell

RAN2 is parallelly discussing the cell reselection enhancements for earth moving cell, including how to provide coverage information, whether to support location-based cell reselection and/or time-based cell reselection initiation/criteria, and related mechanisms. Following agreements has been achieved so far:

Agreements from RAN2#119b: For NTN-NTN cell reselection with earth moving cell, RAN2 will consider providing parameters of serving cell to UE, for UE to estimate when the serving cell stops providing coverage at the present UE location (FFS whether this will be an optional UE feature) (this does not exclude any time-based or location-based approach) (other solutions can also be considered)

Agreements from RAN2#121: A serving cell reference location and a distance threshold/radius will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris). For cell selection/reselection, location-based measurement initiation is supported in earth-moving cell For earth-moving cell, the location-based cell measurement rules of quasi-fixed cell is reused, i.e., for cell reselection in earth-moving cell, UE initiates measurements when its location to serving cell reference location is larger than the configured distance threshold.

Agreements from RAN2#121bis: 1. RAN2 understands that for earth-moving cell reselection, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (FFS whether a new epochTime IE is needed). RAN2 understanding is that both PVT and orbital parameters can be used for this. FFS if additional information is needed to allow more accurate measurements. 2. For earth-moving cell, new IE is introduced to indicate the reference location of serving cell. 3. For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement initiation, which reuses distanceThresh in SIB19. 4. For cell (re)selection in earth-moving system, time-based measurement initiation is used to address feeder-link switch case. 5. Time-based cell reselection criteria is not pursued in R18.

Issue 4-1-A: Time-based cell reselection in earth moving cell NTN deployments

Based on RAN2’s agreement, time-based measurement initiation will be used in earth-moving system. The time-based cell reselection mechanism and requirement applicability for quasi-fixed cell could be referred if similar ‘t-service’ mechanism is introduced by RAN2.

Proposal 1: The time-based cell reselection mechanism and requirement applicability for quasi-fixed cell could be referred if similar ‘t-service’ mechanism is introduced by RAN2.

Issue 4-1-B: For location-based cell reselection in earth moving cell NTN deployments

Agreement:

- For location-based cell reselection in earth moving cell NTN deployments:

- Whether the coverage information of serving cell is (absolutely) necessary:

  - No consensus in the group on whether serving cell coverage information is absolutely necessary.

  - Consider further progress and conclusion from RAN2, if any

- Whether and to what extent restrict the use of the values of DRX cycle:

  - FFS: Do not further restrict DRX cycle beyond Rel-17 NR NTN.

Cell reselection requirement

RAN4 should define requirements for location-based cell reselection. The RRM requirement for location-based cell reselection for quasi-earth fixed cell in Rel-17 as follows can be considered as the starting point for NTN-NTN location-based cell reselection requirement with earth moving cell.

- UE initiates the measurement for cell-reselection in IDLE/Inactive mode if the distance between UE and serving cell reference location is longer than distanceThresh.

- The definition of reference location should further link to RAN2’s design.

Proposal 2: For location-based measurement requirements with earth moving cell, the RRM requirement for location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point, which is:

- UE initiates the measurement for cell-reselection in IDLE/Inactive mode if the distance between UE and serving cell reference location is longer than distanceThresh.

- The definition of reference location should further link to RAN2’s specification.

Cell coverage information

For earth-moving cell, the location-based cell measurement rules of quasi-fixed cell is reused, which means UE will initiate the measurement when the distance if the distance between UE and serving cell reference location is longer than distanceThresh. Some companies observed that the remaining service time may not enough for UE to finish the measurement and evaluation because of the moving cell, and introducing the cell coverage information could help UE to derive this remaining service time information.

In our view, even though UE could acquire the cell coverage information, the fact that insufficient measurement time will not be changed. The only benefit is that RAN4 could define the applicability rule, for instance, if the time span between measurement initiation timing and serving cell service stop timing is less than Ttrigger, then the requirement is not applicable.

Observation 1: The fact that insufficient measurement time due to earth-moving cell will not be changed, whether UE could acquire the coverage information or not.

Observation 2: The only benefit of introducing the coverage information is that RAN4 could define the applicability rule, for instance, if the time span between measurement initiation timing and serving cell service stop timing is less than Ttrigger, then the requirement is not applicable.

Restriction of DRX cycle

We think there is no need to further restrict the DRX cycle configuration. Network could configure the DRX cycle based on satellite speed, cell coverage and so on, it is up to network implementation.

Proposal 3: No need to further restrict the DRX cycle configuration.

Issue 4-2: NTN-TN Cell reselection enhancements (to reduce UE power consumption)

- No agreement. Wait for further progress from RAN2. Companies can provide further analyses in the next meeting as usual. The analyses and discussions may depend on the following aspects:

- The direction of cell reselection, e.g. from TN to NTN, from NTN to TN

- The type of satellites, e.g. GEO, GSO, NGSO

- The type of NTN cell deployment, e.g. earth fixed cell vs. earth moving cell

- Accuracy of TN coverage information provided by NTN cell.

- Whether assistance information of neighbour NTN cells ( e.g. ephemeris data, frequency layer and PCI) provided by serving TN cell is mandatory in terms of RRM requirement definition/applicability

Based on the discussion in RAN2 so far, several basic assumptions have been achieved:

- measurements for TN’s coverage are performed only when relevant - UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage - NTN UE could be provided with assistance information of TN coverage - NTN UE could differentiate when camping in an area only covered by NTN network (earth-moving or earth-fixed) vs an area where TN network(s) is/are also available. - No additional cell reselection prioritization rules for NTN vs TN in Rel-18 (e.g. per service type, per mobility state, or per UE type) on top of what specified in Rel-17

Agreements from RAN2#121bis-e: 1. For signaling the TN coverage, the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one). FFS on the SIB. FFS on whether additional information in dedicated signalling is needed/useful 2. Area center location and its radius for TN coverage information is signalled using Ellipsoid-Point and radius separately. FFS if Rel-17 referenceLocation and distanceThresh are directly reused 3. Decision on the size of TN coverage area list is postponed until more is known on the format of this information and how is it sent. 4. The discussion on how to indicate the frequency information for each TN coverage area should be combined with the discussion on which SIB will be used to indicate the TN coverage area, possibly based on evaluation of the signalling overhead 5. The acquired TN area coverage information remains valid until the next system information update of the SIB including TN coverage info 6. On a frequency band number shared by TN and NTN (e.g., n1), if NTN-specific assistance information is NOT provided for a neighbour cell configured in SIB3/SIB4, UE assumes this is a TN neighbour cell. This understanding is also applicable for Rel-17 and it does not need any spec update

We propose to consider the cell re-selection from NTN to TN as the first priority. The mechanism designed by RAN2 is as follows:

- UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage

- For signaling the TN coverage, the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one).

Based on our understanding of RAN2’s agreement, the NTN-TN cell re-selection enhancement mechanism is applicable for all type of satellites and all the type of NTN deployment. There is no need to set additional restriction in RAN4. Moreover, no need to consider the accuracy of TN coverage information.

Regarding the impaction on NTN-TN cell re-selection requirement, we think the definition of Kcarrier should be revised to capture the NTN-TN cell re-selection enhancement mechanism. For example, Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell except for the TN only carriers when UE is in coverage of none TN cells under this carrier. The detailed condition to exclude the TN carriers depends on RAN2 decision.

Proposal 4: Consider the cell re-selection from NTN to TN as the first priority.

Proposal 5: The NTN-TN cell re-selection enhancement mechanism is applicable for all type of satellites and all the type of NTN deployment.

Proposal 6: No need to consider the accuracy of TN coverage information

Proposal 7: NTN-TN cell re-selection enhancement mechanism will impact the definition of Kcarrier.

- NTN-NTN handover enhancements

### Issue 5-1: RACH-less (C)HO

Agreement:

- RAN4 to define RACH-less NTN HO requirements based on the RACH-less LTE HO requirements. NR and NTN specific adjustments shall be made. The adjustments to investigate may include:

  - FFS on known/unknown condition

  - FFS on necessity of fine time tracking if target cell is known

  - FFS on others if identified

- Note: some aspects would need to wait for further RAN2 progress.

Several agreements of RACH-less handover have been approved in RAN2#121 and RAN2#121bis-e.

Agreements from RAN2#121 - Support RACH-less Handover in Rel-18. - RACH-less Handover in NR NTN is a L3 mobility procedure (FFS if this is combined with the unchanged PCI approach, if supported) and uses the LTE’s RACH-less Handover procedure as a baseline. FFS on TA acquisition - In NTN RACH-less handover, network indicates (implicitly or explicitly) whether NTA in the target cell is identical to the source cell or explicitly provided by the NW. - Support dynamic grant from the target cell for RACH-less PUSCH transmission to reduce random access congestion in the target cell. FFS whether to limit the solution to same feeder link/gateway scenario

Agreements from RAN2#121bis-e 1. In Rel-18 we don’t aim at RACH-less HO for NTN-TN mobility 2. For initial UL transmission in RACH-less HO, support pre-allocated grant in RACH-less HO command 3. NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB; 4. NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB. 5. RAN2 confirms the general UE procedure for NTN RACH-less HO 1. receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC) 2. start timer T304 for the target cell (RRC) 3. perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC) 4. start time alignment timer (MAC) 5. monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY) 6. send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY) 7. consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC) 8. stop timer T304 for the target cell. (RRC) FFS whether to release UL grant if pre-allocated after RACH-less HO completion FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported 6. The pre-allocated grant is provided as type-1 CG 7. Send an LS to RAN1 informing RAN2 agreements on NTN RACH-less HO and check RAN1 views on the following aspects: 1. whether the pre-allocated grant is provided with association to SSBs; if so, whether a RSRP threshold is configured for SSB selection. 2. to monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam indication can be provided in RACH-less HO command. 3. power control for initial UL transmission 8. At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant 9. Consider to support combining RACH-less HO with time-based CHO for NTN, taking into account the 1) validity of pre-allocated grant and potential waste of reserved resource; 2) when/how to provide dynamic grant in PDCCH.

First, we think both known target cell and unknown target cell condition could be covered. RAN2 didn’t limit the RACH-less HO to known cell.

Seconds, even if the target cell is known, the fine time tracking is needed. As designed by RAN2, the UE should perform DL and UL synchronization.

Last, the definition of TIU should cover both cases of pre-allocated grant and dynamic grant. For instance:

TIU is the interruption uncertainty in acquiring the first PUSCH transmission occasion when UE is configured with RACH-less handover in the new SAN cell.

- TIU can be up to [10] ms if pre-allocated UL grant is configured in RRC command.

  - NOTE:	The actual value of TIU shall depend upon the pre-allocated UL grant configuration in RRC command.

- TIU can be up to TUL_grant if pre-allocated UL grant is not configured in RRC command.

  - NOTE:	TUL_grant is the time required to acquire and process uplink grant from the target cell.

Proposal 8: For RACH-less HO, consider both known target cell and unknown target cell, the delay for fine time tracking is needed.

Proposal 9: For RACH-less HO, the definition of TIU should be revised, which should cover both cases of pre-allocated UL grant and dynamic UL grant. Further revision is not precluded based on further agreements from RAN2.

### Issue 5-2: HO for satellite switching without PCI change

- No agreement. Wait for further progress from RAN2.

Agreements from RAN2#121bis-e: 1. In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1 (as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18) 2. Remove the part in brackets “as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18” in the LS to RAN1. The action to RAN1 will also ask for feedback for the hard satellite switch (not only the soft satellite switch case), e.g. action to RAN1 is to see if there are any major technical issues (as in the agreement).

The feasibility of satellite switching without PCI change is basically confirmed by RAN2. The satellite switching delay requirement for satellite switching without PCI change feature can be considered by RAN4. Based on our understanding, the delay requirement should at least cover TSI-NR and TPRACH.

- TSI-NR: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for the target cell of target satellite.

- TPRACH: It is the delay uncertainty in acquiring the first available PRACH occasion in the target cell of target satellite.

Proposal 10: The satellite switching delay requirement for satellite switching without PCI change feature can be considered, which should at least cover TSI-NR and TPRACH.

- TSI-NR: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for the target cell of target satellite.

- TPRACH: It is the delay uncertainty in acquiring the first available PRACH occasion in the target cell of target satellite.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Proposal 1: The time-based cell reselection mechanism and requirement applicability for quasi-fixed cell could be referred if similar ‘t-service’ mechanism is introduced by RAN2.

Proposal 2: For location-based measurement requirements with earth moving cell, the RRM requirement for location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point, which is:

- UE initiates the measurement for cell-reselection in IDLE/Inactive mode if the distance between UE and serving cell reference location is longer than distanceThresh.

- The definition of reference location should further link to RAN2’s specification.

Observation 1: The fact that insufficient measurement time due to earth-moving cell will not be changed, whether UE could acquire the coverage information or not.

Observation 2: The only benefit of introducing the coverage information is that RAN4 could define the applicability rule, for instance, if the time span between measurement initiation timing and serving cell service stop timing is less than Ttrigger, then the requirement is not applicable.

Proposal 3: No need to further restrict the DRX cycle configuration.

Proposal 4: Consider the cell re-selection from NTN to TN as the first priority.

Proposal 5: The NTN-TN cell re-selection enhancement mechanism is applicable for all type of satellites and all the type of NTN deployment.

Proposal 6: No need to consider the accuracy of TN coverage information

Proposal 7: NTN-TN cell re-selection enhancement mechanism will impact the definition of Kcarrier.

Proposal 8: For RACH-less HO, consider both known target cell and unknown target cell, the delay for fine time tracking is needed.

Proposal 9: For RACH-less HO, the definition of TIU should be revised, which should cover both cases of pre-allocated UL grant and dynamic UL grant. Further revision is not precluded based on further agreements from RAN2.

Proposal 10: The satellite switching delay requirement for satellite switching without PCI change feature can be considered, which should at least cover TSI-NR and TPRACH.

- TSI-NR: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for the target cell of target satellite.

- TPRACH: It is the delay uncertainty in acquiring the first available PRACH occasion in the target cell of target satellite.

- Reference

[1] R4-2306358, WF on NR NTN enhancement RRM requirements, Qualcomm Incorporated.