3GPP TSG-RAN WG4 Meeting # 107	R4-2307604

Incheon, KR, May 22 – May 26, 2023

Agenda Item: 8.30.3.1

Source: MediaTek Inc.

Title: Discussion on R18 MIMO for RRM requirements impacts

Document for: Discussion

- Introduction

According to WF [1] and discussion summary [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WI is divided into three agenda items to be discussed: (1) RRM requirements impacts, (2) Timing requirements for UL multi-DCI multi-TRP with two TAs, (3) Unified TCI framework. The discussion in this paper focus on the “RRM requirements impacts”.

- Discussion

Based on our preliminary study, we suggest RAN4 can further study following topics.

- RRM requirements for UL beam indication for simultaneous multi-panel UL transmission

- SRS antenna port switch

2.1 RRM requirements for UL beam indication and simultaneous multi-panel UL transmission

In last meeting, there’re some discussion [2] below on the requirements for UL beam indication.

Issue 1-3-1: Do you think there are RRM impacts by UL precoding indication for multi-panel transmission? Agreement: - No RRM impacts by UL precoding indication for multi-panel transmission. Issue 1-3-2: Do you think there are RRM impacts by UL beam indication in objective 6 for simultaneous multi-panel UL transmission? Proposals - Option 1(Nokia): - Simultaneous UL transmission with multiple panels have RRM impact on unified TCI extension and MTTD requirements. - Reuse similar method as GBBR in downlink reception for uplink pairs of TCI states. - Option 2(Apple/Samsung): - No RRM requirements are introduced for enhanced simultaneous multi-panel UL transmission in this release - Option 3(Ericsson/Apple/Huawei/MTK/Samsung): - Wait for further RAN1 progress - Option 4(Huawei/MTK): - Discuss whether to have impacts on MTTD and TCI requirements under Topic#2 and Topic#3 respectively

In our understanding, there may be some impact for UL beam indication on unified TCI state switching and this will be discussed in another AI. In addition, whether UE support simultaneous multi-panel UL transmission also be an impact should be considered in timing requirement with two TAs in another AI. So, we suggest the following proposal:

Proposal 1: The impacts on MTTD of simultaneous multi-panel UL transmission can be discussed in another agenda item (Timing requirements for UL multi-DCI multi-TRP with two TAs).

Proposal 2: Whether UL beam indication for simultaneous multi-panel UL transmission have RRM impacts can be discussed in another agenda item (Unified TCI framework).

2.2 SRS antenna port switch

In last meeting, there’re some discussion [2] below for the RRM impacts by SRS enhancement.

Issue 1-2-1: Do you think there are RRM impacts by SRS enhancement to enable 8 TX UL operation? GTW conclusion: - Discuss the following 2 solutions separately - Rel-18 SRS enhancements for 8 TX UL - Rel-17 Full slot SRS transmission

In R17, RAN4 defined the interruption requirement for SRS antenna port switching based on R15 SRS patten, i.e., only consider the case when SRS is transmitted in last 6 symbols in one slot. However, from R16 to R18, RAN1 continuously specify SRS enhancement. For example, SRS is no longer limited in last 6 symbols in one slot.

Let us further recap R16/17 SRS resource enhancement before discussing whether to specify new interruption requirement in R18.

- In R16, SRS resources can be in any symbol position within one slot.

- In R17, one SRS resource can occupy 14 symbols within one slot for coverage enhancement.

- No matter R16/17, all SRS resources within one or multiple SRS resource set could support inter-slot transmission depend on NW configuration.

At least, the legacy interruption requirements for SRS antenna port switching cannot cover all the enhanced SRS patterns in R16/R17. RAN4 could specify new interruption requirements of SRS antenna port switching, based on SRS resource enhancement. We prefer to define a unified requirement, i.e., consider the worst case (E.g., 1T8R SRS resources for antenna switching and one SRS resource with 14 symbols) to define the interruption length.

In addition, if we define Rel-17 full slot SRS transmission, it could also support Rel-18 SRS enhancement for 8TX UL and xTyR to transmit any symbol within one slot. This is more flexible configuration to allow UE perform SRS antenna switching. So, based on above analysis, we have the following observation and proposal.

Observation 1: In R17, RAN4 only defined the interruption requirement for SRS antenna port switching base on R15 SRS pattern (SRS resources in the last 6 symbols in one slot).

Proposal 3: RAN4 to discuss the interruption requirements for Rel-17 full slot SRS transmission.

- Summary

In this paper, the discussion of R18 MIMO is provided. We have the following proposal:

Proposal 1: The impacts on MTTD of simultaneous multi-panel UL transmission can be discussed in another agenda item (Timing requirements for UL multi-DCI multi-TRP with two TAs).

Proposal 2: Whether UL beam indication for simultaneous multi-panel UL transmission have RRM impacts can be discussed in another agenda item (Unified TCI framework).

Observation 1: In R17, RAN4 only defined the interruption requirement for SRS antenna port switching base on R15 SRS pattern (SRS resources in the last 6 symbols in one slot).

Proposal 3: RAN4 to discuss the interruption requirements for Rel-17 full slot SRS transmission.

- Reference

- R4-2306362, WF on R18 NR MIMO RRM requirements, Samsung, RAN#106bis-e

- R4-2306254, Topic summary for [106bis-e][223] NR_MIMO_evo_DL_UL, Samsung, RAN#106bis-e