3GPP TSG-RAN WG4 Meeting # 107	R4-2307610

Incheon, KR, May 22 – May 26, 2023

Agenda Item: 8.25.2.3

Source: MediaTek Inc.

Title: Discussion on L1/L2 inter-cell mobility delay requirements

Document for: Discussion

- Introduction

In RAN4#106bis-e, some companies had some proposals on cell switch delay requirements. Due to limited time in last meeting, there are still a number of the issues that are open [1]. We will provide our views on cell switch delay requirements in this contribution.

- Discussion

2.1 Cell switch delay for PCell/PSCell

2.2.1 End point of RACH-less cell switch delay for PCell/PSCell

Regarding the ending point if RACH-less cell switch, RAN4#106bis-e has reached the agreement to wait for more progress in RAN2 and RAN2 had agreed that “In RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE”. So for RACH-less cell switch, we can follow RAN2 and use the first UL transmission as the ending point.

Issue 3-2-1: Ending point of RACH-less cell switch delay for PCell/PSCell < Agreement>: - Wait for more progress in RAN2

- RAN2#121bis-e Agreement In RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE

Proposal 1: For RACH-less cell switch, cell switch delay for PCell/PSCell ends at UE transmitting the first UL to the target cell.

2.2.2 known cell conditions and known TCI state conditions

Before going to the detail of cell switch delay requirements, we would like to discuss known cell conditions and known TCI state conditions at first.

For known cell condition, we suggest using the conditions for L3 HO with a bit modification:

The target cell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the cell switch command: - the UE has sent a valid L1 [or L3] measurement report for the target cell and - One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell or 9.3 for inter-frequency cell, - One of the SSBs measured from the target cell also remains detectable during the cell switch delay according to the cell identification conditions specified in clause 9.2 for intra-frequency cell or 9.3 for inter-frequency cell. otherwise it is unknown.

In last meeting, some company pointed out that the side condition discussed for L1-RSRP measurement may have impact on the known cell condition. We are open to further discuss whether to align the side condition with L1-RSRP measurement. In our understanding, NW can choose to trigger cell switch based on L1-RSRP measurement report or L3 measurement report. Even when the actual SNR is lower than -3dB and higher than -6dB, UE would keep measuring L1-RSRP as requested by NW. The known cell condition is to make sure the cell is actually detectable to UE. From this point, we prefer to use the same side condition as L3 HO.

For known TCI state condition, we suggest using legacy known TCI state conditions with a bit modification:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of cell switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the cell switch command - The TCI state remains detectable during the cell switching period - The SSB associated with the TCI state remain detectable during the cell switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

Proposal 2: Use the following known cell conditions as a baseline in LTM:

The target cell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the cell switch command: - the UE has sent a valid L1 [or L3] measurement report for the target cell and - One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell or 9.3 for inter-frequency cell, - One of the SSBs measured from the target cell also remains detectable during the cell switch delay according to the cell identification conditions specified in clause 9.2 for intra-frequency cell or 9.3 for inter-frequency cell. Otherwise, it is unknown.

Proposal 3: Known TCI state conditions in LTM:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of cell switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the cell switch command - The TCI state remains detectable during the cell switching period - The SSB associated with the TCI state remain detectable during the cell switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

2.2.3 Procedure of cell switch for PCell/PSCell

RAN2 has agreed the timeline of LTM in R2-2209256 as Fig.1 and the meaning of some components are listed in Table 1.

Figure 1

Table 1

| Components | Meaning |
| --- | --- |
| Tcmd | Time for processing L1/L2-command (HARQ and parsing) |
| Tprocessing,2 | Time for UE processing. This may include L2/3 reconfiguration, RF retuning, baseband retuning, security update if needed, etc. |
| Tsearch | Time required to search the target cell |
| TΔ | Time for fine tracking and acquiring full timing information |
| Tmargin | Time for SSB or CSI-RS post-processing |
| TIU | interruption uncertainty in acquiring the first available PRACH occasion in the new cell |

Similar as L3 HO, the interruption time would start from Tprocessing,2 as shown in Fig.2. For the component TΔ: In our understanding, if the TCI state to use for the target cell is already in the active TCI state list, UE may not need T/F fine tracking during cell switch. As analyzed in our another contribution [2], considering that the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms, if there are too many cells and beams to monitor or SSB period is too long, it may not be guaranteed that UE has a chance to monitor the target cell every 160ms. For inter-frequency, it would be more difficult to guarantee this as MG is shared between L3 and L1. So UE may still need T/F fine tracking for the following initial transmission even the TCI state to use for the target cell is already in the active TCI state list. In addition, pre-synchronization of DL on a neighbor cell may require UE capability. If UE has to perform T/F fine tracking after cell switch command, the interruption time would be as long as L3 HO.

Proposal 4: TΔ will still be non-zero if 1) UE is not configured to perform pre-T/F tracking by network, or 2) UE is configured to perform pre-T/F tracking by network but the target SSB is not available at least once in every 160ms period.

Figure 2

Observation 1: TΔ may equals to Tfirst-RS. The interruption time of cell switch may be as long as L3 handover.

We are thinking to shorten the interruption time with a bit change on cell switch procedure. As shown in Fig.2, UE would perform L1/L2/L3 processing at first (Tprocessing,2) and then T/F fine tracking (TΔ) if needed. When UE performs L1/L2/L3 processing, UE is not able to receive or transmit dada in the source cell. If UE performs T/F fine tracking (TΔ) at first and then L1/L2/L3 processing (Tprocessing,2) as shown in Fig.3, the interruption would start from the RS symbol used for T/F fine tracking or one symbol earlier. When SSB period is larger, the gain will be considerable.

Figure 3

Proposal 5: Further discuss whether UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

2.2.4 Components of cell switch delay requirements for PCell/PSCell:

No matter which procedure (Fig.2 or Fig.3) will be concluded, the components of cell switch delay would be the same. In the next, we would like to discuss each component in table 1.

- L1/L2 inter-cell mobility execution time:

In last meeting, RAN4 reached the consensus on the baseline of the cell switch delay for PCell/PSCell. Regarding the execution time, RAN4 would wait for RAN2 progress. According to RAN2 agreement, UE may perform early ASN.1 decoding and validity/compliance check of candidate cell configuration and may not. As RAN4 defines the requirements for the worst case, Texecution_time for ASN.1 decoding and validity/compliance check of target cell configuration should be added in the delay requirements.

Issue 3-3-1: RACH-based Cell switch delay for Pcell/PSCell < Agreement>: - The baseline of RACH-based cell switch delay requirements is Tdelay = Tcmd + Tprocessing / TLTM-processing + Tsearch + T∆ + Tmargin + Tuncertainity /TIU, where Tuncertainity /TIU is the uncertainty in acquiring the first available PRACH occasion in the new cell. - FFS: the exact value of each component. Some components can be 0 in certain cases, if agreed. - FFS: add/remove/modify other component(s). Note: ‘/’ means ‘or’ here. Issue 3-3-2: RACH-less Cell switch delay for Pcell/PSCell < Agreement>: - The baseline of RACH-less cell switch delay requirements is Tdelay = Tcmd + Tprocessing,2 / TLTM-processing + Tsearch + T∆ + Tmargin + Tuncertainity/TIU, - FFS: the ending point - FFS: the exact value of each component. Some components can be 0 in certain cases, if agreed. - FFS: add/remove/modify other component(s). Note: ‘/’ means ‘or’ here. Issue 3-3-12: Execution time <Agreement> - Wait for RAN2 progress.

RAN2#121bis-e Agreement The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed. - Initial agreements, from RAN2 point of view (may be dep on RAN1 progress).

Proposal 6: Texecution_time for ASN.1 decoding and validity/compliance check of target cell configuration should be added in the cell switch delay requirements.

Regarding the value, we think further discussion is necessary. RAN2 has agreed “reference configuration + delta configuration”. UE can perform ASN.1 decoding and validity/compliance check both for the reference configuration and delta configuration, and then forms a complete configuration after cell switch command. From the point of configuration, “reference configuration” and “delta configuration” may be configured through two different RRC reconfiguration command. Even they are configured through a single command, UE may decode the two parts in sequential. We think it is reasonable that longer execution time is needed compared to the execution time (10ms) in CHO.

Proposal 7: Further discuss whether longer Texecution_time is needed due to “reference configuration + delta configuration”.

- Tprocessing,2: It is the time for L1/L2/L3 reconfiguration and RF retuning. When discussing the value of processing time, the following factors should be considered:

- Intra-DU or inter-DU cell switch

Our view is that the key difference between intra-DU and inter-DU lies in that DU entity remains unchanged in NW for intra-DU. But what really matters to UE is whether there is any L1/L2/L3 reconfiguration. DU entity remains the same does not mean no L2/L3 reconfiguration.

- Switch to active SCell (including role change)

- The active SCell may be DL only SCell, or DL&UL SCell or PUCCH SCell. No matter which kind of Scell, L2/L3 may be reconfigured by NW.

- If the target cell is previous DL only SCell, L1 reconfiguration is necessary at least for UL.

- If the target cell is previous DL&UL SCell, L1 reconfiguration is necessary at least for PUCCH, RACH and paging.

- If the target cell is previous PUCCH SCell, L1 reconfiguration is necessary at least for RACH and paging.

Observation 2: If the target cell is an active SCell, L1 reconfiguration is necessary, and it is up to NW configuration whether L2/L3 reconfiguration is needed.

- The difference between inter-frequency and intra-frequency cell switch

- RF retuning is needed for inter-frequency cell switch. RF retuning may not be needed for intra-frequency cell switch if BWP remains the same.

- If the inter-frequency is different from all the serving cell, L1 reconfiguration is definitely necessary. If the inter-frequency cell is an intra-frequency neighbor cell of SCell, L1 reconfiguration is needed at least for RACH and paging.

- For intra-frequency cell switch, L1 reconfiguration may be needed depending on NW configuration.

Observation 3: If the target cell is an inter-frequency cell of source SpCell, L1 reconfiguration is necessary.

Observation 4: For intra-frequency cell switch, L1 reconfiguration may be needed depending on NW configuration.

In last meeting, some company pointed out that some procedures are not needed for R18 LTM (such as security update) or some procedures had been done before cell switch command such as DL/UL BWP activation due to pre- DL/UL synchronization. So the processing time after cell switch can be shorter.

We have different views on that. According to RAN1 progress, RAN1 has agreed to support beam indication together with cell switch command and TCI state activation of a neighbour cell before cell switch. But RAN1 has not supported beam indication of a neighbour cell before cell switch yet. Therefore, UE is not expected to receiver or transmit data on the neighbour cell even the TCI state is activated. There is no need for UE to activate the BWP of the neighbour cell, either. After the TCI state of a neighbour cell is activated, UE only performs SSB based T/F fine tracking on the corresponding beam.

According to RAN1 agreement, the following two options are both supported.

-	PDCCH ordered RACH without RAR

-	PDCCH ordered RACH with RAR and the RAR is received from the serving cell

As UE will not receive RAR from neighbour cell and UE would not receive or transmit data from the neighbour cell, we don’t think UE needs to get UL or DL BWP ready to transmit RACH to neighbor cell.

Regarding security update, as far as we know, it only takes a small portion of the processing time. As there are too many L1/L2/L3 parameters, and too many scenarios, we admit that there may be some differences in the actual time needed for L1/L2/L3 or RF retuning for different scenarios or certain parameter remains unchanged. But we don’t think it is necessary to consider all the differences especially some minor difference and define a bunch of different Tprocessing,2. We suggest grouping all the scenarios depending on whether extra time is needed for L2/L3 reconfiguration or L1 reconfiguration. As shown in Table 2, we suggest categorizing all the scenarios into at most four groups:

|  | L2/L3 reconfiguration | L1 reconfiguration | Typical scenario |
| --- | --- | --- | --- |
| Group#1 | Y | Y | - intra-DU or Inter-DU, intra-frequency or inter-frequency cell switch with L1 and L2/L3 reconfiguration |
| Group#2 | N | Y | - intra-DU or Inter-DU, intra-frequency or inter-frequency cell switch without L2/L3 reconfiguration but with L1 reconfiguration: - including switch to active SCell without L2/L3 reconfiguration |
| Group#3 | N | N | - intra-frequency cell switch without L1/L2/L3 reconfiguration, maybe intra-DU or inter-DU |
| Group#4 | Y | N | - intra-frequency cell switch with L2/L3 reconfiguration, maybe intra-DU or inter-DU |

Four groups are already too many, we are open to cut down the group number and the scenarios that is not typical. In addition, there are too many L1/L2/L3 parameters, we suggest avoiding the discussion on how much time can be reduced if L2/L3 or L1 is partially reconfigured.

As for the exact values, we propose Tprocessing,2=20ms for intra-FR cell switch & Tprocessing,2=40ms for inter-FR cell switch for Group#1. Although RAN4 defines the value for each step, it is the total delay requirements that really matters to UE. The value of Tcmd is much smaller than RRC procedure delay in L3 HO. The left margin of Tcmd +Tprocessing,2 would be much smaller than RRC procedure delay+ Tprocessing in L3 HO. Thus we suggest using the legacy UE processing time (Tprocessing,2=20ms for intra-FR cell switch, Tprocessing,2=40ms for inter-FR cell switch) for Group#1.

For Group#2/3/4, the processing time can be shorter than L3 HO and we are open to discuss the exact values further.

We suggest focusing on intra-FR cell switch at first. For inter-FR cell switch, the processing time is 20ms more than intra-FR cell switch in general.

Proposal 8: To avoid defining too much Tprocessing,2 values for different scenarios, suggest focusing only on the typical scenarios and classifying the scenarios into limited groups.

Proposal 9: Detailed discussion on the impact of particular parameters on the processing time should be avoided.

Proposal 10: Categorize all the scenarios into at most four groups depending on if L2/L3 reconfiguration or L1 reconfiguration is needed and further discuss which group(s) to define requirements for.

|  | L2/L3 reconfiguration | L1 reconfiguration | Typical scenario |
| --- | --- | --- | --- |
| Group#1 | Y | Y | - intra-DU or Inter-DU, intra-frequency or inter-frequency cell switch with L1 and L2/L3 reconfiguration |
| Group#2 | N | Y | - intra-DU or Inter-DU, intra-frequency or inter-frequency cell switch without L2/L3 reconfiguration but with L1 reconfiguration: - including switch to active SCell without L2/L3 reconfiguration |
| Group#3 | N | N | - intra-frequency cell switch without L1/L2/L3 reconfiguration, maybe intra-DU or inter-DU |
| Group#4 | Y | N | - intra-frequency cell switch with L2/L3 reconfiguration, maybe intra-DU or inter-DU |

Proposal 11: In cell switch delay requirements, Tprocessing,2=20ms for intra-FR cell switch and Tprocessing,2=40ms for inter-FR cell switch when software processing for L2/L3 reconfiguration and L1 reconfiguration is needed. FFS: the value for other groups.

- TΔ: In legacy L3 HO, TΔ is needed for two purposes. One purpose is for T/F fine tracking. Another is for decoding MIB to get SFN. If we want to omit TΔ, UE should have tracked the target cell recently and have obtained SFN before cell switch.

As analyzed in 2.2.3, even the TCI state of a neighbor cell is activated before cell switch command, considering that the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms, if there are too many cells and beams to monitor or SSB period is too long, it may not be guaranteed that UE has a chance to monitor the target cell every 160ms. Therefore, the base is that TΔ is needed. TΔ can omit only if UE has obtained SFN of the target cell and have tracked the target cell in the latest 160ms.

- Tmargin: When UE needs T/F fine tracking, Tmargin is needed and equals to 2ms.

Proposal 12: In cell switch delay requirements, the baseline is:

- TΔ=1 Tfirst-RS, Tmargin = 2ms

- TΔ and Tmargin can be 0 if UE has obtained SFN of the target cell and have tracked the target cell in the latest 160ms.

- Tsearch

Since the aim of LTM is to reduce HO latency, to achieve this goal, UE would be supposed to perform L1-RSRP measurement and even DL/UL synchronization before cell switch, we think the cell should at least be known in the typical procedure. So we prefer defining cell delay requirements only for known cell and active scell as target cell case. Thus Tsearch=0.

Proposal 13: Not define cell switch delay requirements for unknown cell case and Tsearch=0.

- Other components ---TCI state switching time:

In last meeting, some companies proposed that TCI state switching time is also needed during cell switch delay requirement.

In our understanding, this issue is related to RACH-less cell switch as NW does not need to indicate the TCI state for RACH-based cell switch.

Our view is that only consider known TCI state case. If TCI state is unknown, UE is supposed to perform L1-RSRP measurement during cell switch and report the results to network. This will lead to much longer cell switch delay and interruption, comparing allowing UE to use CBRA directly. Therefore, we see no benefit to define requirements for unknown TCI state case, as Rel-15 CBRA achieves a shorter delay.

For DL, the only component in TCI state switching is T/F fine tracking which is already included in cell switch delay requirements.

For UL, since now only SSB based L1-RSRP measurement is supported, we think it is reasonable to configure reference PL-RS as a known SSB but not CSI-RS. But when there are multiple cells to measure especially for inter-frequency, the measurement delay would be too long, we are open to discuss whether extra time is needed to measure PL-RS.

Proposal 14: Only define cell switch delay requirements for known TCI state case and not define requirements for unknown TCI state case.

Proposal 15: For PCell/PSCell switch delay, further discuss whether extra time is needed to measure PL-RS.

- Summary

In this paper, we provide some views on cell switch delay requirements. We have the following proposals and observations:

Proposal 1: For RACH-less cell switch, cell switch delay for PCell/PSCell ends at UE transmitting the first UL to the target cell.

Proposal 2: Use the following known cell conditions as a baseline in LTM:

The target cell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the cell switch command: - the UE has sent a valid L1 [or L3] measurement report for the target cell and - One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell or 9.3 for inter-frequency cell, - One of the SSBs measured from the target cell also remains detectable during the cell switch delay according to the cell identification conditions specified in clause 9.2 for intra-frequency cell or 9.3 for inter-frequency cell. Otherwise, it is unknown.

Proposal 3: Known TCI state conditions in LTM:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of cell switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the cell switch command - The TCI state remains detectable during the cell switching period - The SSB associated with the TCI state remain detectable during the cell switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

Proposal 4: TΔ will still be non-zero if 1) UE is not configured to perform pre-T/F tracking by network, or 2) UE is configured to perform pre-T/F tracking by network but the target SSB is not available at least once in every 160ms period.

Observation 1: TΔ may equals to Tfirst-RS. The interruption time of cell switch may be as long as L3 handover.

Proposal 5: Further discuss whether UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

Proposal 6: Texecution_time for ASN.1 decoding and validity/compliance check of target cell configuration should be added in the cell switch delay requirements.

Proposal 7: Further discuss whether longer Texecution_time is needed due to “reference configuration + delta configuration”.

Observation 2: If the target cell is an active SCell, L1 reconfiguration is necessary, and it is up to NW configuration whether L2/L3 reconfiguration is needed.

Observation 3: If the target cell is an inter-frequency cell of source SpCell, L1 reconfiguration is necessary.

Observation 4: For intra-frequency cell switch, L1 reconfiguration may be needed depending on NW configuration.

Proposal 8: To avoid defining too much Tprocessing,2 values for different scenarios, suggest focusing only on the typical scenarios and classifying the scenarios into limited groups.

Proposal 9: Detailed discussion on the impact of particular parameters on the processing time should be avoided.

Proposal 10: Categorize all the scenarios into at most four groups depending on if L2/L3 reconfiguration or L1 reconfiguration is needed and further discuss which group(s) to define requirements for.

|  | L2/L3 reconfiguration | L1 reconfiguration | Typical scenario |
| --- | --- | --- | --- |
| Group#1 | Y | Y | - intra-DU or Inter-DU, intra-frequency or inter-frequency cell switch with L1 and L2/L3 reconfiguration |
| Group#2 | N | Y | - intra-DU or Inter-DU, intra-frequency or inter-frequency cell switch without L2/L3 reconfiguration but with L1 reconfiguration: - including switch to active SCell without L2/L3 reconfiguration |
| Group#3 | N | N | - intra-frequency cell switch without L1/L2/L3 reconfiguration, maybe intra-DU or inter-DU |
| Group#4 | Y | N | - intra-frequency cell switch with L2/L3 reconfiguration, maybe intra-DU or inter-DU |

Proposal 11: In cell switch delay requirements, Tprocessing,2=20ms for intra-FR cell switch and Tprocessing,2=40ms for inter-FR cell switch when software processing for L2/L3 reconfiguration and L1 reconfiguration is needed. FFS: the value for other groups.

Proposal 12: In cell switch delay requirements, the baseline is:

- TΔ=1 Tfirst-RS, Tmargin = 2ms

- TΔ and Tmargin can be 0 if UE has obtained SFN of the target cell and have tracked the target cell in the latest 160ms.

Proposal 13: Not define cell switch delay requirements for unknown cell case and Tsearch=0.

Proposal 14: Only define cell switch delay requirements for known TCI state case and not define requirements for unknown TCI state case.

Proposal 15: For PCell/PSCell switch delay, further discuss whether extra time is needed to measure PL-RS.

4 Reference

R4-2306395, “WF on NR Mobility Enhancements RRM requirements (part 1)”, MediaTek Inc., RAN4#106bis e-meeting, April 17th – 26th, 2023.

R4-2307608, “Discussion on general aspects and scenarios of L1/L2 mobility”, MediaTek Inc., RAN4#107, May 22 – May 26, 2023.