3GPP TSG-RAN WG4 Meeting # 107	R4-2307613

Incheon, KR, May 22 – May 26, 2023

Agenda Item: 8.25.5

Source: MediaTek Inc.

Title: Discussion on Enhanced CHO configurations

Document for: Discussion

- Introduction

RAN4#106bis-e had some discussion on the RRM requirements for enhanced CHO configurations. There are still some open issues [1]. In this contribution, we would like to share our view on the remaining issues.

- Discussion

In last meeting, RAN4 agreed to define the requirements for both objective #3 and objective #4. In the next, we would like to discuss how to define the related requirements.

2.1 CHO including target MCG and target SCG in NR-DC (obj. 3)

In RAN4#106bis-e, RAN4 had the consensus on PCell HO delay in obj. 3 except the definition of Tprocessing.

Issue 3-2-1: PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) - Agreements: - PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as: - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution. - Definition of each component, except Tprocessing, is same as that defined in CHO requirement in TS38.133 clause 6.4.1.2. - Definition of Tprocessing: - Option 1: same as that defined in requirements of handover with PSCell. - Option 2: same as that defined in requirements of CHO.

To us, CHO including target MCG and target SCG in NR-DC is actually CHO with PSCell. So here we can reuse the Tprocessing defined in the requirements of HO with PSCell, i.e.:

Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

Proposal 1: For PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC, reuse the Tprocessing defined in the requirements of HO with PSCell, i.e.:

Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

Regarding the requirements for PSCell, there are still some open issues:

Issue 3-2-2: PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) - Candidate solutions: - Companies are encouraged to further discuss the following issues: - When does UE execute PSCell addition/change - Option 1: after PCell CHO is executed but before it is completed. - Option 2: after PCell CHO is completed. - Whether Tsearch_PCell_Conditional is neeed in DCHOwithPSCell_PSCell?

In our understanding, we can follow the similar rule as HO with PSCell. UE does not need to always execute PSCell addition/change after PCell CHO is completed. But there is some timing dependency on PCell if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. In this case, UE needs to know PCell’s SFN at first and then determines PSCell’s SMTC occasion. Even in CHO with target SCG, PCell is known but it doesn’t mean PCell’s SFN is known. PSCell addition/change is executed after acquiring PCell’s SFN (Tsearch_PCell_Conditional = TΔ + Tmargin). Thus, we propose the delay requirements for PSCell in CHO with target SCG as:

DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

- The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for PCell

- Tsearch_PCell_Conditional is the time for obtaining the timing reference of target PCell. If SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, Tsearch_PCell_Conditional = TΔ + Tmargin, where TΔ has the same definition in the delay requirements for PCell and Tmargin =2ms. Otherwise, Tsearch_PCell_Conditional = 0 ms.

- TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

Proposal 2: For CHO with target SCG from FR1+FR2 NR-DC to FR1+FR2 NR-DC, the delay requirements for PSCell DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

- The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for PCell

- Tsearch_PCell_Conditional is the time for obtaining the timing reference of target PCell. If SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, Tsearch_PCell_Conditional = TΔ + Tmargin, where TΔ has the same definition in the delay requirements for PCell and Tmargin =2ms. Otherwise, Tsearch_PCell_Conditional = 0 ms.

- TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

2.2 CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4)

According to RAN2 agreement, the baseline is that UE waits to perform CHO with PSCell until both CHO and CPC conditions are met. So the requirements to define for obj.3 can be used as a baseline.

- RAN2#121bis-e - For the CHO+CPC case: When both CHO and CPC conditions are met, both CHO and CPC cell change is executed. Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen). Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.

For CHO+CPC/CPA (objective #4), as there are separate events for PCell and PSCell, when UE starts to execute HO with PSCell, UE has already finished the measurement on both PCell and PSCell. In other words, PCell and PSCell are both known.

For the basis “The UE waits until both CHO and CPC conditions are met (always)”, UE behavior would be very similar to objective#3. The only difference is that PSCell is always known in objective #4 but may be unknown in objective #3. The requirements of CHO+CPC (objectives #4) are basic the same except PSCell is always known. Thus we propose the requirements as below:

For CHO + CPC from FR1+FR2 NR-DC to FR1+FR2 NR-DC, the delay requirements for PCell DCHOwithPSCell_PCell is similar as obj.3, i.e., DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that

- Tprocessing = 25 ms

- TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

For CHO + CPC from FR1+FR2 NR-DC to FR1+FR2 NR-DC, the delay requirements for PSCell DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

- The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for PCell

- TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

Proposal 3: For CHO + CPC from FR1+FR2 NR-DC to FR1+FR2 NR-DC, the delay requirements for PCell DCHOwithPSCell_PCell is similar as obj.3, i.e., DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that

- Tprocessing = 25 ms

- TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

Proposal 4: For CHO + CPC from FR1+FR2 NR-DC to FR1+FR2 NR-DC, the delay requirements for PSCell DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

- The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for PCell

- TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

- Summary

In this paper, we have some discussion on Enhanced CHO configurations. We have the following observation and proposals:

Proposal 1: For PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC, reuse the Tprocessing defined in the requirements of HO with PSCell, i.e.:

Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

Proposal 2: For CHO with target SCG from FR1+FR2 NR-DC to FR1+FR2 NR-DC, the delay requirements for PSCell DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

- The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for PCell

- Tsearch_PCell_Conditional is the time for obtaining the timing reference of target PCell. If SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, Tsearch_PCell_Conditional = TΔ + Tmargin, where TΔ has the same definition in the delay requirements for PCell and Tmargin =2ms. Otherwise, Tsearch_PCell_Conditional = 0 ms.

- TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

Proposal 3: For CHO + CPC from FR1+FR2 NR-DC to FR1+FR2 NR-DC, the delay requirements for PCell DCHOwithPSCell_PCell is similar as obj.3, i.e., DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that

- Tprocessing = 25 ms

- TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

Proposal 4: For CHO + CPC from FR1+FR2 NR-DC to FR1+FR2 NR-DC, the delay requirements for PSCell DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

- The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for PCell

- TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

4 Reference

R4-2306396, “WF on NR Mobility Enhancements RRM requirements (part 2)”, Apple, RAN4#106bis e-meeting, April 17th – 26th, 2023.