3GPP TSG-RAN WG4 Meeting #107	R4-2307630

Incheon, Korea, 22th– 26th May 2023

Source:	China Telecom

Title:	Discussion on the phase I study assumptions for the advanced receiver for MU-MIMO

Agenda Item:	8.19.1.2

Document for:	Discussion

- Introduction

In RAN4 #106bis-e meeting we discussed the advanced receiver to cancel inter-user interference for MU-MIMO within the NR_demod_enh3-Perf WI. The WF is approved in [1] as the discussion outcome.

In this paper, our views on the phase I study assumptions are given.

- Discussion

The number of co-scheduled UEs

Status in the WF in [1]:

Candidate options:

  - Option 1: 1 co-scheduled UE as higher priority

  - Option 2: Cover 2 co-scheduled UEs at least in phase I study

Way forward

  - Keep the previous meeting, and continue discussion in the next meeting.

In addition to 1 co-scheduled UE that has been covered, we have supported to also cover more than 1 co-scheduled UE at least for the phase I study. We thought we should cover the possible scenario that there are more than 1 co-scheduled UEs with different modulation order for MU-MIMO transmission.

At the same time, we noticed that in the last meeting, we have reached consensus on the R-ML assumption that ‘Within each PRB/PRG, UE applies R-ML to all interference layers with prior information that all interference layers have same modulation order’. As a consequence, it is impossible for UE to perform R-ML when there are more than 1 co-scheduled UEs with different modulation order within one PRB/PRG. Considering the above, as a compromise, we propose not to consider 2 co-scheduled UE for the scenario with full FDRA for all UEs, and we can further discuss the scenario with partial FDRA in the required information discussion part.

Proposal 1: Not to consider 2 co-scheduled UE for the scenario with full FDRA for all UEs, and we can further discuss the scenario with partial FDRA in the required information discussion part.

MCS for the target UE

Status in the WF in [1]:

- Cover MCS 13 for rank 1 and rank 2 for initial simulation

- Cover MCS 17 for rank 2 for initial simulation

- Further discuss whether to cover MCS 4 for rank 1 in the next meeting

- The assumption can be updated later based on available results.

In the last meeting, we have agreed to include MCS13 and MCS17 for phase I. To save the simulation workload, we prefer not to additionally consider MCS4.

Proposal 2: Not to additionally consider MCS4 for phase I study.

Modulation order for the co-scheduled UE

Status in the WF in [1]:

- For R-ML, E-IRC and IRC (baseline in Rel-17, for performance comparison purpose) for initial simulation

  - For rank 1+1: QPSK (high priority for the next meeting)

  - For rank 2+2: 64QAM (high priority for the next meeting)

  - For rank 2+2: QPSK (high priority for the next meeting)

  - For rank 1+1 (64QAM) +1 (QPSK) (lower priority)

  - For rank 1+1 (64QAM) +1 (16QAM) (lowest priority)

  - Other options on the modulation order for co-scheduled UE are not precluded.

  - These assumptions can be updated in the next meeting based on available simulation results.

In the Rel-17 evaluation, only 1 co-scheduled UE with random 16QAM modulated symbols is considered. R-ML receiver needs to detect the co-scheduled modulated symbols with the highest likelihood within the constellation. Only evaluate the detection within 16QAM constellation is not enough.

Furthermore, from the UE implementation complexity point of view, by increasing the modulation from 16QAM to 64QAM, to achieve similar gain, the R-ML receiver requires 3 times more complexity. We understand that the UE implementation complexity should be limited thus it is important to verify that the R-ML receiver can be workable under higher modulation order from the co-scheduled UE.

Considering the above it is proposed to cover QPSK, 16QAM and 64QAM for the co-scheduled layers.

Since we have proposed not to consider rank 1+1+1 with full FDRA scenario, and therefore 16QAM will not be covered in phase I study. So, we propose to also consider 16QAM for the co-scheduled UE for rank 1+1 cases.

Proposal 3: Support to cover QPSK, 16QAM and 64QAM for the co-scheduled layers:

- For rank 1+1: QPSK

- For rank 1+1: 16QAM

- For rank 2+2: 64QAM

- For rank 2+2: QPSK

PDSCH resource allocation for the target and co-scheduled UE

Status in the WF in [1]:

Candidate options:

  - Candidate scenarios:

    - Scenario 1: Maximum transmission bandwidth configuration for all UEs.

    - Scenario 2: Maximum transmission bandwidth configuration for the target UE and partial transmission bandwidth configuration for the co-scheduled UEs.

    - Full OFDM symbol allocation for both scenarios.

  - Option 1: Cover both scenario 1 and scenario 2 in the phase I study

    - Option 1A: Consider the following scenario for the study of R-ML and E-IRC with blind detection to the required information

- Target UE with frequency domain full PRB allocation (52PRBs)

- Co-scheduled UE1 with PRB0~19 allocation with 16QAM transmission.

- Co-scheduled UE2 with PRB40~51 allocation with QPSK transmission.

- Full OFDM symbol allocation.

  - Option 1B:

| Allocation | Layer 1 | Layer 2 | Layer 3 | Layer 4 |
| --- | --- | --- | --- | --- |
| PRB 0 ~ PRB19 | Target UE |  | UE3 64QAM | UE1 16QAM |
| PRB20 ~ PRB39 |  |  |  | None |
| PRB40 ~ PRB51 |  |  | UE2 QPSK |  |

  - Option 1C

- Target UE with frequency domain full PRB allocation (52PRBs)

- Co-scheduled UE1 with PRB0~26 allocation.

  - Option 2: Only consider scenario 1

  - Option 3: FFS based on the conclusion for Issue 1-2-2-8

Way forward

  - For initial simulation in phase I, cover scenario 1

  - FFS whether to cover scenario 2, based on the discussion outcome in the NWA part.

In the real network, BS could allocate different PRBs and different modulation specifically for each UE, considering the different traffic load and different channel condition. Such scenario will impact the R-ML and E-IRC performance since both receivers will need to estimate the channel for the co-scheduled UE. We think the scenario with partial bandwidth allocation for the co-scheduled UE, should be considered in the phase I study.

At the same time, we notice that scenario 2 has been covered in the DMRS port and FDRA blind detection study thus we think we only need to consider scenario 1 in the phase I study for the advanced receiver with genius information.

Proposal 4: Only need to consider scenario 1 in the phase I study for the advanced receiver with genius information and cover scenario 2 for the DMRS port and FDRA blind detection in the required information study part.

- Conclusion

Proposal 1: Not to consider 2 co-scheduled UE for the scenario with full FDRA for all UEs, and we can further discuss the scenario with partial FDRA in the required information discussion part.

Proposal 2: Not to additionally consider MCS4 for phase I study.

Proposal 3: Support to cover QPSK, 16QAM and 64QAM for the co-scheduled layers:

- For rank 1+1: QPSK

- For rank 1+1: 16QAM

- For rank 2+2: 64QAM

- For rank 2+2: QPSK

Proposal 4: Only need to consider scenario 1 in the phase I study for the advanced receiver with genius information and cover scenario 2 for the DMRS port and FDRA blind detection in the required information study part.

- References

- [1] R4-2305914, WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #106bis-e, April, 2023