3GPP TSG-RAN WG4 Meeting #107									              R4-2307639

Incheon, Korea, May 22 – 26, 2023

Agenda item:	8.10.3.2

Source:	Apple

Title:	Discussion on R18 inter-RAT measurement without gap

Document for:	Discussion

# Introduction

Inter-RAT measurement w/o gap RRM requirement was widely discussed during the previous RAN4 meetings. The last agreements can be found in [1]. In this contribution, we continue discussing the open issue.

# Discussion

For information, up to this meeting, all agreed using scenarios for inter-RAT NR/LTE measurements without gap can summarized as:

- the inter-RAT NR measurements without gap in Rel18 includes the two scenarios below.

  - Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements

  - Case a-2: NR reference signal to be measured are fully contained within UE’s LTE channel bandwidth

- the inter-RAT LTE measurements without gap in Rel18 includes the two scenarios below.

  - Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements

  - Case b-2: LTE CRS are fully contained within UE’s active BWP

Case a-2 has been agreed to deprioritized in this work item.

The first issue is about differentiation for case b-2

### Issue 2-1-1: Differentiate scenarios for Case b-2

< Way forward/Agreement >:

- FFS on whether to differentiate the following scenarios in case b-2.

  - Scenario 1: Inter-RAT LTE measurement for LTE CRS rate-matching feature

  - Scenario 2: Inter-RAT LTE measurement for CRS-IM receiver

From RRM measurement point of view, we don’t see the necessity to differentiate these two scenarios. The impact is on demodulation performance but not on RRM measurement.

Proposal 1: no need to differentiate the following two scenarios in RRM requirements:

- Scenario 1: Inter-RAT LTE measurement for LTE CRS rate-matching feature

- Scenario 2: Inter-RAT LTE measurement for CRS-IM receiver

Next issue is about UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2)

### Issue 2-2-2: UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2)

< Way forward >:

  - A new per-UE capability to support Case b-2 should be defined.

  - FFS on indication of “no gap with interruption” because of AGC,T/F offset issue

Support of case b-2 is unlike support of case b-1. Case b-2 does not require additional RF chain or dynamic BW adjustment. Therefore it can be decoupled with NeedForGaps. RAN4 is supposed to introduce some conditions under which UE can perform inter-RAT LTE measurement w/o gap, similarly to inter-frequency measurement w/o gap in R16. Regarding the FFS on indication of “no gap with interruption” because of AGC,T/F offset issue, we prefer to consider them as part of the conditions so that no interruption needs to be defined.

Proposal 2: A new per-UE capability to support Case b-2 should be defined. Indication such as “no gap with interruption” is not necessary unless well justified. Potential issues such as AGC can be reflected in applicability conditions for case b-2.

Next issue is about searcher limitation

### Issue 2-3-2: Searcher limitation

< Way forward/Agreement >:

  - Option 1:

    - Inter-RAT LTE measurement without gap(case b-2) can be performed in parallel with NR measurement without searcher limitation

  - Option 2:

    - Performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported

We continue supporting not to consider performing inter-RAT measurement and NR measurements in parallel without searcher limitation. The fundamental goal of this objective is to reduce measurement gap overhead by enabling inter-RAT measurement w/o gap, rather than facilitate the RRM measurement.

Proposal 3: Performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported. The fundamental goal of this objective is to reduce measurement gap overhead by enabling inter-RAT measurement w/o gap, rather than facilitate the RRM measurement.

Next issue is about effective measurement window:

### Issue 2-4-9: Effiective measurement window

< Way forward >:

  - Option 1:  vivo, MTK, Huawei, Ericsson

    - RAN4 to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap.

  - Option 1a:  Ericsson

    - The effective measurement window can be defined based on measurement duration, measurement periodicity and offset.

  - Option 1b: MTK

    - FFS on the overlap between NR SMTC and LTE SMTC-lte (i.e. the effective measurement window).

The effective measurement window is necessary for RRM measurement requirements. RAN4 can pick some typical Tinter1 from existing inter-RAT LTE measurement requirement (e.g. from some mandatory pattern such as Tinter1 = 30.)

Proposal 4: RAN4 to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap.

Next issue is about scheduling restriction for inter-RAT LTE measurements:

### Issue 2-5-2: Scheduling restriction for inter-RAT LTE measurements

< Way forward/Agreement >:

  - Scheduling restriction due to inter-RAT LTE measurement are applicable when

      - UE does not support simultaneous Tx and Rx on the serving cell and target band

      - FFS on Serving cell and target MO have mixed SCS and they are in the same band

Regarding the FFS on serving cell and target MO have mixed SCS and they are in the same band, we support to include it in scheduling restriction as baseline.

Proposal 5: Scheduling restriction due to inter-RAT LTE measurement is needed when serving cell and target MO have mixed SCS and they are in the same band.

# Conclusion

In this contribution, we provide further discussion on NeedForGaps requirements. After discussion, the following conclusions are provided:

Proposal 1: no need to differentiate the following two scenarios in RRM requirements:

- Scenario 1: Inter-RAT LTE measurement for LTE CRS rate-matching feature

- Scenario 2: Inter-RAT LTE measurement for CRS-IM receiver

Proposal 2: A new per-UE capability to support Case b-2 should be defined. Indication such as “no gap with interruption” is not necessary unless well justified. Potential issues such as AGC can be reflected in applicability conditions for case b-2.

Proposal 3: Performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported. The fundamental goal of this objective is to reduce measurement gap overhead by enabling inter-RAT measurement w/o gap, rather than facilitate the RRM measurement.

Proposal 4: RAN4 to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap.

Proposal 5: Scheduling restriction due to inter-RAT LTE measurement is needed when serving cell and target MO have mixed SCS and they are in the same band.

# References

- R4-2306331, WF on NR_MG_enh2 Part 2, Intel Corporation

- R4-2303305, WF on NR_MG_enh2 Part 2, Intel Corporation.

- RP-230755, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation