3GPP TSG-RAN WG4 Meeting #107									              R4-2307648

Incheon, Korea, May 22 – 26, 2023

Agenda item:	8.25.2.3

Source:	Apple

Title:	Discussion on L1/L2 based inter-cell mobility delay requirements

Document for:	Discussion

# Introduction

L1/L2 based inter-cell mobility delay requirements were widely discussed during the previous RAN4 meetings. The last agreements can be found in [1], in which there are still some open issues. In this contribution, we continue discussing the open issues.

# Discussion

The first issue we would like to discuss is about

Issue 3-1-6: LTM delay requirements

< Way forward >: FFS the following options

  - Option 1: Not define the LTM delay requirement which starts from UE receives RRC configuration on candidate cell(s).

  - Option 2: FFS

We support option 1. Because before UE receives MAC command for LTM, UE won’t trigger cell switch procedure. There is nothing to verify regarding RRC configuration here.

Proposal 1: Not define the LTM delay requirement which starts from UE receives RRC configuration on candidate cell(s).

Next issue is about procedure of cell switch.

Issue 3-2-2: Procedure of cell switch

< Way forward >: FFS the following options

  - Option 1: Further discuss whether UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

  - Option 2:  Under the condition that target cell is known, UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

  - Option 3: if T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2).

  - Option 4: UE cannot perform T/F tracking before necessary L1 reconfiguration, which get the UE's L1 ready to receive DL of the target cell. If needed, some other L1 configuration, L2/L3 reconfiguration and L2 reset can be performed during cell switch, i.e., in some cases it can be performed after the T/F tracking.

According to RAN1/2 design, network can trigger TCI state activation and PDCCH ordered PRACH for candidate cell before sending cell switch command. It is likely that UE needs to perform fine T/F tracking even before receiving cell switch command.

Observation 1: network can trigger TCI state activation and PDCCH ordered PRACH for candidate cell before sending cell switch command. It is likely that UE needs to perform fine T/F tracking even before receiving cell switch command.

For the case wherein UE still needs to perform T/F fine tracking after receiving cell switch command, i.e., no TCI activation or PDCCH ordered RACH before cell switch, we are not sure if it is a typical case and shall be captured in RAN4 spec. Even if so, we don’t think we can always guarantee that UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch. Some reconfiguration needs to be processed first so that UE can perform T/F fine tracking.

Proposal 2: if T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2) as baseline.

Regarding details of cell switch delay, we would like highlight Tprocessing,2.

Issue 3-3-3: Tprocessing,2

- What UE has done during Tprocessing, 1 before cell switch command? What UE needs to do during Tprocessing, 2 after cell switch command?

Tprocessing, 1 is similar to the first RRC processing time discussed in CHO design. RAN4 eventually agree

- What is the impact of reference +delta configuration on the processing time?

- What UE has prepared for target cell after performing pre DL synchronization?

- What UE has get prepared for target cell after performing pre UL synchronization?

- The factors to consider when target cell is a current scell ?

- How many different processing time values to define?

< Way forward >: FFS the following options

  - Option 1 (CTC, CMCC, Nokia, Huawei): The time for UE processing could be reduced if some procedures have been done before UE receive the cell switch command or for some scenarios.

    - Option 1a (CMCC): Tprocessing = 0 for the case that DL synchronization for candidate cell(s) is performed before cell switch command

    - Option 1b (Nokia):

      - Some of the RRC processing can be done prior to the cell switch to reduce the delay. FFS what parts are done when the configuration arrives.

      - At least partial RRC processing is done before the cell switch command. Exact conditions can be FFS.

      - LTM-processing Should consider a scenario where the user plane does not need reset, and there is no need for further RRC processing.

  - Option 2 (Apple): Tprocessing in LTM shall include execution time CHO and the legacy Tprocessing in Tinterrupt.

  - Option 3 (QC): RAN4 to not assume UE can always finish a processing of RRC configurations for LTM cells before LTM handover command reception, e.g., the processing and loading the configuration before the LTM handover command reception can be limited to measurement related configurations of the LTM cells. And RAN4 to not assume the processing and loading the measurement configuration of the LTM cell before LTM handover command reception means the entire downlink configuration of the LTM target cell is processed and loaded.

  - Option 4 (MTK):

    - To avoid defining too much Tprocessing,2 values for different scenarios, suggest focusing only on the typical scenarios and classifying the scenarios into limited groups.

    - Categorize all the scenarios into at most four groups depending on if L2/L3 reconfiguration or L1 reconfiguration is needed:

|  | L2/L3 reconfiguration | L1 reconfiguration | Typical scenario |
| --- | --- | --- | --- |
| Group#1 | Y | Y | - intra-DU or Inter-DU, intra-frequency or inter-frequency cell switch with L1 and L2/L3 reconfiguration |
| Group#2 | N | Y | - intra-DU or Inter-DU, intra-frequency or inter-frequency cell switch without L2/L3 reconfiguration but with L1 reconfiguration: - including switch to active SCell without L2/L3 reconfiguration |
| Group#3 | N | N | - intra-frequency cell switch without L1/L2/L3 reconfiguration, maybe intra-DU or inter-DU |
| Group#4 | Y | N | - intra-frequency cell switch with L2/L3 reconfiguration, maybe intra-DU or inter-DU |

    - Tprocessing,2=20ms for intra-FR cell switch and Tprocessing,2=40ms for inter-FR cell switch when software processing for L2/L3 reconfiguration and L1 reconfiguration is needed. FFS: the value for other groups.

  - Option 5 (vivo):

    - In the cell switch delay requirements, T_processing is specified in a case-by-case manner based on the current state of target cell.

    - For the baseline requirement, RAN4 assume UE forms the complete configuration and applies it during the execution of cell switch command. Before cell switch command, only necessary L1 configuration are formed and applied.

  - Option 6 (Ericsson):

    - Processing delay is 3ms when delta configuration is used and L2 reset is not performed.

    - Processing delay is 0ms for the role change case.

    - For all other cases, RAN4 shall try to optimise the UE processing delay so that LTM benefits can be fully realised

RRC parameters decoding and applying time in this procedure is similar to that in CHO. Tprocessing, 1 in LTM iss similar to the first RRC processing time discussed in CHO design. Regarding Tprocessing, 2 in LTM, it shall include execution time CHO and the legacy Tprocessing in Tinterrupt. It is possible that network pre-configures multiple candidate cells for L1 measurement, together with cell parameters of the candidate cells. But eventually UE only needs to switch to one target PCell. Therefore, it is unnecessary for UE to perform parameters validation and apply them for all the candidate cells, including L2/3 reconfiguration, RF retuning, baseband retuning, security update if needed, etc. Our expectation is that UE keeps measuring all the candidate cells. Once it receives switch command to certain target cell, it can make the corresponding cell parameters valid and apply them like CHO.

Proposal 3: UE is not expected to verify and apply all RRC parameters for all candidate cells before receiving cell switch command. Similar to CHO requirement, additional execution time shall be considered in Tprocessing (TLTM-processing or Tprocessing, 2 depending on conclusion on the equation in cell switch delay) besides the legacy Tprocessing in legacy Tinterrupt.

Regarding known cell conditions and known TCI conditions:

Issue 3-4-1: known cell conditions

< Way forward >: FFS the following option

  - Use the following known cell condition as a baseline for further study:

    - The target cell is known if it has been meeting the following conditions:

      - During the last 5 seconds before the reception of the cell switch command:

      - the UE has sent a valid L1 or [L3] measurement report for the target cell and

      - One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.3,

      - One of the SSBs measured from the target cell also remains detectable during the cell switch delay according to the cell identification conditions specified in clause 9.3.

    - otherwise it is unknown.

Issue 3-4-2: known TCI state conditions

< Way forward >: FFS the following option

  - Use the following known TCI state condition as a baseline for further study:

    - The TCI state is known if the following conditions are met:

      - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of cell switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

        - Cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

        - The UE has sent at least 1 L1-RSRP report for the target TCI state before the cell switch command

        - The TCI state remains detectable during the cell switching period

        - The SSB associated with the TCI state remain detectable during the cell witching period

          - SNR of the TCI state ≥ -3dB

    - Otherwise, the TCI state is unknown.

We support the wayforward in the previous meeting.

Proposal 4: known cell conditions in LTM:

  - Use the following known cell condition as a baseline for further study:

    - The target cell is known if it has been meeting the following conditions:

      - During the last 5 seconds before the reception of the cell switch command:

      - the UE has sent a valid L1 or [L3] measurement report for the target cell and

      - One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.3,

      - One of the SSBs measured from the target cell also remains detectable during the cell switch delay according to the cell identification conditions specified in clause 9.3.

    - otherwise it is unknown.

Proposal 5: known TCI conditions:

- The TCI state is known if the following conditions are met:

  - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of cell switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

    - Cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

    - The UE has sent at least 1 L1-RSRP report for the target TCI state before the cell switch command

    - The TCI state remains detectable during the cell switching period

    - The SSB associated with the TCI state remain detectable during the cell witching period

      - SNR of the TCI state ≥ -3dB

- Otherwise, the TCI state is unknown.

# Conclusion

In this contribution, we provide further discussion on LTM – inter-cell mobility delay requirements. After discussion, the following conclusions are provided:

Proposal 1: Not define the LTM delay requirement which starts from UE receives RRC configuration on candidate cell(s).

Observation 1: network can trigger TCI state activation and PDCCH ordered PRACH for candidate cell before sending cell switch command. It is likely that UE needs to perform fine T/F tracking even before receiving cell switch command.

Proposal 2: if T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2) as baseline.

Proposal 3: UE is not expected to verify and apply all RRC parameters for all candidate cells before receiving cell switch command. Similar to CHO requirement, additional execution time shall be considered in Tprocessing (TLTM-processing or Tprocessing, 2 depending on conclusion on the equation in cell switch delay) besides the legacy Tprocessing in legacy Tinterrupt.

Proposal 4: known cell conditions in LTM:

  - Use the following known cell condition as a baseline for further study:

    - The target cell is known if it has been meeting the following conditions:

      - During the last 5 seconds before the reception of the cell switch command:

      - the UE has sent a valid L1 or [L3] measurement report for the target cell and

      - One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.3,

      - One of the SSBs measured from the target cell also remains detectable during the cell switch delay according to the cell identification conditions specified in clause 9.3.

    - otherwise it is unknown.

Proposal 5: known TCI conditions:

- The TCI state is known if the following conditions are met:

  - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of cell switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

    - Cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

    - The UE has sent at least 1 L1-RSRP report for the target TCI state before the cell switch command

    - The TCI state remains detectable during the cell switching period

    - The SSB associated with the TCI state remain detectable during the cell witching period

      - SNR of the TCI state ≥ -3dB

- Otherwise, the TCI state is unknown.

# References

- R4-2306395, WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.

- R1-2304276, LS on beam indication of target cell(s) and time gap between a PDCCH order and the corresponding PRACH transmission for LTM, Fujitsu, MediaTek

- R4-2303175, WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.

- RP-230805, New WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, vivo